
[image: image1.png]



DALY CITY DEPARTMENT OF PUBLIC WORKS

PLAYGROUND SAFETY MAINTENANCE PLAN

This maintenance plan is to ensure that we keep the playground in a good working condition so the children can have it to play on for years to come and that the playground is kept in a good and safe condition to decrease and/or eliminate injuries.

The weekly inspection is to be completed by City staff. The focus of these inspections should include:

· Inspection of key areas for signs of broken equipment and vandalism.

· Check safety zones (slides) for any objects such as trash, bottles and anything that may be a hazard to the children and remove all hazards.
· Pick up litter.
· Check surfacing (wood fiber) in high-use areas. Rake over any areas that may have become worn away by use.

The monthly inspection is to be completed by City staff. The focus of these inspections should include:

· Inspection of key areas for signs of wear, loose fasteners, broken equipment, and vandalism.

· Check and tighten all bolts, nuts, and fasteners.

· Check safety zones (slides) for any objects such as trash, bottles and anything that may be a hazard to the children and remove all hazards.

· Pick up litter.

· Check surfacing (wood fiber) in high-use areas. Rake over any areas that may have become worn away by use.

· Check for equipment deterioration (chipped paint, rust).
An annual inspection will be performed by a Certified Playground Safety Inspector. The inspection will include, but not be limited to the following:

· Inspection of key areas for signs of wear, loose fasteners, broken equipment, and vandalism.

· Check and tighten all bolts, nuts, and fasteners.

· Lubricate any moving parts

· Measure, and make recommendations as to the condition and depth of the safety surface.

· Check for equipment deterioration (chipped paint, rust).

· Complete a written maintenance report

· Meet with staff to discuss report and to answer any questions or to identify any areas to watch

In addition to the scheduled maintenance inspections, the following checklist will be used to determine the condition of our playground. Person completing the inspection will sign off the checklist. All staff members are to be trained with the proper inspection guidelines. The weekly and annual checklist will be kept on file in the office of Daly City Building Maintenance Division, and will be available for review by KaBOOM!.
PLAYGROUND INSPECTION REPORT
	YES
	NO
	

	
	
	The equipment has adequate protective surfacing under and around it and the surfacing materials have not deteriorated.

	
	
	Loose-fill surfacing has no foreign objects or debris

	
	
	Loose-fill surfacing materials are not compacted and do not have reduced depth in heavy use areas such as swings or at slide exits

	
	
	There are no sharp points, corners or edges on the equipment

	
	
	There are no missing or damaged protective caps or plugs

	
	
	There are no hazardous protrusions and projections

	
	
	There are no potential clothing entanglement hazards, such as open S-hooks or protruding bolts

	
	
	There are no pinch, crush and shearing points or exposed moving parts

	
	
	There are no trip hazards, such as exposed footings on anchoring devices and rocks, roots or any other environmental obstacles in the play area.

	
	
	The equipment has no rust, rot, cracks or splinters, especially when it comes in contact with the ground.

	
	
	There are no broken or missing components on the equipment (e.g. handrails, guardrails, protective barriers, steps or rungs on ladders) and there are no damaged fences, benches, or signs on the playground

	
	
	All equipment is securely anchored

	
	
	There are no loose fastening devices or worn connections, such as S- hooks

	
	
	Moving components, such as swing hangers are not worn

	
	
	The paint used on the playground equipment has not deteriorated as noted by peeling, cracking, chipping or chalking

	
	
	There are no visible paint chips or accumulation of paint dust

	
	
	All equipment is securely anchored

	
	
	The entire playground is free from miscellaneous debris or litter

	
	
	There are no missing trash receptacles

	
	
	Trash receptacles are not full


Inspected By: ____________________________      Date: _________________

NOTES: __________________________________________________________________________________________________________________________________________________________________________________________________________

_984298940

