page

272

Extension

Sheet

Electrical power

Power,  P=potential difference,  Vcurrent,  IP=VI

(in watts)(in volts)(in amps)

Energy transferred,  E=power,  Ptime,  tE=Pt

(in joules)(in watts)(in s)

Example	Answer

A 10 V supply passes 2 A	a)	Power,  P = p.d.,  V			current,  I
through a resistor.			= 10 V			2 A

a)	Calculate the power.	= 20 watts

b)	How much energy is	b)	Energy transferred,  E = power,  P		time,  t
	transferred in 1 minute?			= 20 W		60 s
					= 1200 joules

Questions

For each question show all your working clearly.

1.A small laboratory immersion heater passes

2 A when connected to a 6 V supply.

What is the power of the heater?

2.A 2 kW kettle is switched on for 2 minutes.

How much energy is transferred to the water?

(1 kW = 1000 W)

3.	What is the current in each of the following
situations?

a)	A 460 W vacuum cleaner connected to the
		230 V mains.

b)	A personal cassette-player, rated at 4.5 W
		with a 9 V battery.

c)	A 48 W car headlamp bulb connected to a
		12 V car battery.

4.In the circuit shown,

a)What is the current flowing?

b)What is the power dissipated?

c)If the current flows for 0.5 minute, how

much energy is transferred to the resistor?

d)Where does this energy come from?

5.	An electric fire connected to the 230 V mains
supply carries a current of 4 A.

a)	Calculate the power of the fire.

b)	Calculate the energy transferred in 1 hour.