pages

266–267

Energy transfers in a circuit

Extension

Sheet

| Charge, Q = | current,I  time, t |  |  |  |  |  |  |  |  |  |  | Q | = | I  t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (coulombs) | (amps) |  |  |  | (seconds) |  |  |  |  |  |  |  |  |  |
| Energy transferred, E = p.d., V  charge, Q |  |  |  |  |  |  |  |  |  |  |  | E | = | V  Q |
| (joules) (volts) (coulombs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Energy transferred, E |  | = | p.d., V |  |  |  | current, I  time, t |  |  |  |  | E | = | V  I  t |

| Example |  | Answer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A current of 2 A flows for 10 s |  | a) | Charge, Q |  | = current, I  time, t |  |  |  |  |  |  |
| through a p.d. of 3 V. |  |  |  |  | = 2 A  10 s = 20 coulombs |  |  |  |  |  |  |
| a) What charge passes through the wire? |  |  |  |  |  |  |  |  |  |  |  |
|  |  | b) | Energy transferred |  |  |  | = p.d., V  charge, Q |  |  |  |  |
| b) How much energy is transferred? |  |  |  |  |  |  | = 3 V  20 C |  |  |  |  |
|  |  |  |  |  |  |  | = 60 joules |  |  |  |  |

Questions

For each question show all your working clearly.

1.a)  What charge passes if 4 A flows for 20 s?

b)  What current flows if 100 C passes in 10 s?

c)  What charge passes if 2 A flows for 2 minutes?

2.a)  How much energy is transferred by 5 C

moving through a p.d. of 10 V?

b)  If a 6 V battery transfers 30 J, how much

charge passes round the circuit?

3.a)  How much energy is transferred by a current of

2 A flowing for 10 seconds with a p.d. of 5 V.

b)  If 200 J are transferred by a current of 2 A

flowing for 100 s, what is the p.d.?

4.In the circuit shown,

a)  What is the current through the 3  resistor?

b)  If this current flows for 100 s, what charge

passes through the wires?

c)  How much energy is transferred in this case?

5.In the circuit shown,

a)  What is the p.d. across the resistor?

b)  aIf the current flows for 5 minutes, what charge

passes round the circuit?

c)  How much energy is transferred in this case?

d)  Where does this energy come from, and what

effect does it have on the resistor?