Competency Assessment – Skilled Clinicians

SUPPLEMENTAL – DOPPLER ABI PROCEDURES*

EMPLOYEE:____________________________                   Date:________________________

For the Evaluator: Check the appropriate box according to the SN’s ability to perform the defined skill: 	S= Satisfactory, U= Unsatisfactory. Place any comments if necessary and initial.

EVALUATOR:__________________________________

| # DOPPLER ABI PROCEDURES | OBS | S | U | COMMENTS | INITIAL |
| --- | --- | --- | --- | --- | --- |
| Prepare patient – rest on flat comfortable surface with a small pillow for 10 to 20 minutes |  |  |  |  |  |
| Apply Blood Pressure Cuff 2-3 cm above antecubital site of the arm |  |  |  |  |  |
| Palpate brachial pulse, apply gel over site and place Doppler probe tip at 45° angle pointed toward the patient’s head |  |  |  |  |  |
| Inflate pressure cuff 20-30 mm Hg above the point where the pulse is no longer audible. |  |  |  |  |  |
| Slowly deflate cuff 2-3 mm Hg per second noting reading at which the 1st pulse reading is heard and record reading.. |  |  |  |  |  |
| Apply cuff 2-3cm above the malleolus of the affected leg |  |  |  |  |  |
| Palpate tibial or dorsalis pedal pulse, apply gel and Doppler probe tip at 45°angle pointed toward the patient’s knee |  |  |  |  |  |
| Inflate pressure cuff 20-30 mm Hg above the point where the pulse is no longer audible |  |  |  |  |  |
| Slowly deflate cuff 2-3 mm Hg per second noting reading at which the 1st pulse reading is heard and record reading |  |  |  |  |  |
| ABI = Higher of either the dorsalis pedal or posterior tibial reading divided by the higher of the brachial pressure readings. |  |  |  |  |  |
| Documents result in Droid on Clinical Coordination Note |  |  |  |  |  |

(*abbreviated from WOCN ABI Quick Reference Guide  for  Clinicians)

Rev: 09/29/14