| Hitachi WJ200 Exercises |  |  |
| --- | --- | --- |
| Using the Demo Case |  | DEMO CASES |

Refer to the Inverter Instruction Manual:  NT325X or Quick Reference Guide: NT3251X for detail

Manual Number: DC001.2

June 2016

Hitachi Industrial Equipment Systems Co., Ltd.

Lab: Using the Demo Case

In the next few pages we will run through a step by step guide on how to use the Hitachi WJ200 when it is used in the Demo Case. The Demo Case shown has 7 switches, 3 LED’s, a potentiometer, the WJ200-002SF, a meter that measures Hz, and an induction motor. Exercise 1-6 will be used to program switches 1-7. Exercise 7-9 will show examples on how to program the LED’s. While Exercise 10 will show an example of how to use the ProDriveNext software

Note: Edit the Full Display mode and Software lock mode selection before attempting any exercise

Full Display Mode

1) Press on keypad until b001 is on the display and then press or until b037 is on the display

2) Press and then use the or keys until 00 is on the display, then press

Changing the parameter to 00 allows for a full display of parameters on the digital operator

Software Lock Mode Selection

1) Press on keypad until b001 is on the display and then press or until b031 is on the display

2) Press and then use the or keys until 10 is on the display, then press

Changing the parameter to 10 gives the operator a high level of access to all parameters in run or stop mode

Exercise 1

Forward Function

1) Press on keypad until C001 is shown on the display

2) Press , then press or until 00 is on the display, then press again to save the setting

From here there are many options on how one wants the motor to start.

A) Set motor to specific frequency

3) Pressuntil A001 is on display, then press  or to A002, press, and then press orto 01 “Controller”, then press

3) Press until A001 is on display then press then press or until 02 is on the display, changing the frequency source to “Function F001 setting” then press

4) Hit  until F001 is displayed on the screen, press

5) Shown on the display is the frequency at which the motor will run at. Press or hold the or key to 30 Hz then press

6) Turn the switch labeled “1” towards “ON” then the motor will turn on and accelerate to the desired frequency.

B) Vary motor speed with potentiometer

3) (If the motor is ON turn OFF) Press until A001 is on display, then press then press or until 01 is on the display changing the frequency source to “Control Terminal” then press

4) Turn the switch labeled “1” towards “ON”

5) Turn potentiometer labeled “Analog Input” clockwise to increase speed and counter clockwise to decrease speed

Exercise 2

Reverse Function

1) Press on keypad until C001 is shown on the display, then press or to C002, press and press  or to 01, then press

This function is now being stored onto the switch labeled “2”. From here the motor can follow steps A) or B) in Logic Input 1 section in the manual. Depending on desired function please follow those steps in the section above.

The motor is seen to be moving in the reverse direction once it is turned on and running.

Exercise 3

Multi-Speed Select ~Binary Operation

The inverter can store up to 16 different target frequencies (speeds) that the motor output uses for steady-state run condition. These can be programmed into 4 different terminals or fewer. In this example we will program it into the switches labeled “3” and “4”.

1) Press  until C001 is displayed, then press or to C003, press and press  or to 02, then press

This programs the switch labeled “3” to the function “Multi-Speed Select, Bit 0 (LSB)”

2) Press  until C001 is displayed, then press  or to C004, press and press  or to 03, then press

This programs the switch labeled “4” to the function “Multi-Speed Select, Bit 1”

From here we can select the speeds at which the motor will run when switch “3” is on, switch “4” is on or when both switches “3” and “4” are on this allows the motor to be able to run at 4 different speeds with only programming two switches.

How to program speeds for the Multi-Speed Select Setting

Program switch “3”

1) Turn the Run command OFF (Stop Mode)

2) Turn only switch “3” on

3) Pressuntil F001 is displayed, then press, then use or to 10 Hz then press  to save the value

Program Switch “4”

1) Turn the Run command OFF (Stop Mode)

2) Turn only switch “4” on

3) Pressuntil F001 is displayed, then press, then use or to 30 Hz then press  to save the value

Program Switches “3” and “4” together

1) Turn the Run command OFF (Stop Mode)

2) Turn only switch “4” and switch “3” on

3) Pressuntil F001 is displayed, then press, then use or to 60 Hz then press  to save the value

Testing these settings

1) Set display on WJ200 to view Hz, pressto d001, then press

2) Turn switch only switch “1” ON

3) Now turn switch “3” ON, the value programmed should be seen on the display

4) Turn switch “3” OFF then turn switch “4” ON, Notice that the motor goes to the programmed speed

5) Turn both switches “3” and “4” ON, Notice the motor also goes to that programmed speed.

This exercise showed that the motor can run at 4 different speeds depending on which switch is ON or OFF. Speed 1-both switches OFF, Speed 2-Switch “3” ON, Speed 3-Switch “4” ON, Speed 4-Switch “3” and “4” ON.

Exercise 4

Two Stage Acceleration and deceleration

When the switch that is programmed to this setting is turned ON, the inverter changes the rate of acceleration and deceleration from the initial settings (F002 and F003) to use the second set of acceleration/ deceleration values. When the terminal is turned OFF, the inverter is returned to the original acceleration and deceleration time (F002 acceleration time 1, and F003 deceleration time 1)

Programming switch “5” to two stage acceleration and deceleration

1) Press until C001 is displayed then press  to C005, then press, then use or to 09, then press

Now switch “5” is programmed to run the two stage acceleration and deceleration setting

2) Set acceleration time 1 to 5.00 by pressing  to F001, then press to F002, then press, then use or to 5.00 then press

3) Set deceleration time 1 to 5.00 by pressing  to F001, then press to F003, then press, then use or to 5.00, then press

4) Set acceleration time 2 to 50.00 by pressing to A001, then press to A092, then press, then use or to 50.00, then press

5) Set deceleration time 2 to 50.00 by pressing to A001, then press to A093, then press, then use or to 50.00, then press

Testing these settings

1) Turn switch “1” ON and turn the Analog Input Knob all the way clockwise.

2) While the motor is accelerating flip switch “5” ON and notice that the motor takes a lot longer time to accelerate when the switch is turned on.

3) Turn the Analog Input Knob counter-clockwise all the way and turn switch “5” ON and OFF and realize that when it is ON the deceleration time is a lot slower than when the switch is OFF.

Exercise 5

Reset Inverter

The [RS] terminal causes the inverter to execute the reset operation. If the inverter is in Trip Mode, the reset cancels the Trip state. When the signal [RS] is turned ON and OFF, the inverter executes the reset operation. Reference 4-29 in Instruction manual NT325X or page 35 in the Quick Reference Guide NT3251X

How to program Reset Inverter Function

1) Press  until C001 is displayed, then press or to C006, press and press  or to 18, then press

Switch “6” is now programmed to the Reset Inverter Function

Testing these settings

1) Turn Motor on by turning switch “1” ON, while motor is running turn switch “6” towards ON. Notice that the Inverter resets.

Exercise 6

Unattended Start Protection

If the Run command is already set when power is turned ON, the inverter starts running immediately after power up. The Unattended Start Protection (USP) function prevents that automatic startup, so that the inverter will not run without outside intervention. See page 34 in Quick Reference Manual NT3251X or 4-25 in Instruction manual NT325X

How to program Unattended Start Protection

1) Press  until C001 is displayed, then press or to C007, press and press  or to 13, then press

Testing these settings

1) Turn switch “7” ON then turn switch “1” ON

2) While the motor is running turn OFF the power switch

3) Turn the power switch back on and notice that the motor will not spin even though switch “1” is still on and the ALM light is ON, shown on the Inverters display.

4) To fix the problem turn switch “1” to OFF then turn switch “1” back ON

Exercise 7

Program Run Signal

Using these Logic outputs we will make the green LED light up when the motor is set to run mode. See page 36 Quick Reference Manual NT3251X or 4-54 in Instruction manual NT325X

1) Press  until C001 is displayed, then press or to C021, press and press  or to 00, then press

This programs sets the LED labeled 11 to turn ON when the run signal is ON

Testing these settings

1) Turn switch “1” ON and notice that the Green LED labeled 11 is now turned ON.

How to program

Exercise 8

Program Frequency Arrival Type 1-Constant Speed

The yellow LED in this application will light up when the motor is at its set frequency

1) Press  until C001 is displayed, then press or to C022, press and press  or to 01, then press

This programs the yellow LED labeled 12 to turn ON when the motor is at its set frequency

Testing these settings

1) Turn switch “1” ON

2) Turn switch “3” ON, this switch was programmed to a specific speed setting previously. Please refer to “How to program speeds for the Multi-Speed Select Setting Program switch “3””for programming switch “3”

Exercise 9

Program Alarm Signal

The inverter alarm signal (Red LED) is active when a fault has occurred and it is in the Trip Mode. When the fault is cleared the alarm signal becomes inactive. (Refer to the diagram below)

1) Press  until C001 is displayed, then press or to C026, press and press  or to 05, then press, The Alarm Signal is now set

Testing these settings

1) Turn switch “1” ON

2) Turn switch “7” ON (Switch “7” was programmed to unattended start protection, If switch 7 is not programmed to this please refer to Exercise 6)

3) While the Motor is spinning and switch “1” and “7” are on unplug the power supply. Notice the Red LED turns on because of a fault.

4) Plug power supply back in. Notice the motor is not spinning even though switch “1” is on, and the Red LED is on suggesting there was a fault.

Exercise 10

Setting parameters with the ProDriveNext software

1) Open up ProDriveNext software (if not downloaded please refer to page 9 of the reference manual) and connect a USB cable from the PC to the USB connector on the top right of the WJ200

2) Click on the “Configuration” tab on the top of the page and select “Add Device…”

3) Click on the  button and the software will find the connected device

4) The device is now connected

5) To change parameters via the PC click on the Parameter button shown below

6) Select, then all “C” group parameters will be shown

7) Right Click on any function and select, this will download all parameters that are set on the inverter to the PC

8) After the PC is uploaded with these parameters change C001 to Set Value 01 and C002 to set value 00. Notice that there can be no duplicate Set values for different parameters. For example C001 and C002 cannot have the set value of 00 at the same time!

9) When switching parameter C001 to 01 and C002 to 02, start by setting C001 to 255 (No Function) then set C002 to 00 then go back and set C001 to 01

Notice the parameters that you have changed are highlighted in pink

10) To upload the current parameters on the PC to the inverter right click on any parameter and then select. Wait for all parameters to upload.

Testing these settings

1) Turn switch “1” ON located on the Demo Case. Notice that the motor is now turning in reverse

2) Turn switch “1” OFF then turn switch “2” ON. Notice that the motor is now turning in the forward direction

Here we switched the Reverse and Forward functions from being on switches “1” and “2” to being on switches “2” and “1”