DOUBLE STRANDED STANCHION KIT ASSEMBLY INSTRUCTIONS

Item No. 959**-6 Kit contains the following:

6 each bases

6 each 24" poles

6 each 12" poles

6 each extensions

6 each ball tops

24 pairs O-rings

Chain is sold separately.

- Put O-rings into pairs, one male and one female.   Put chain on O-rings   and snap O-rings together.   Set aside.

b

base

pole

Put 24” pole onto base, press firmly down until it stops.

extension

piece

Take extension piece and slide part way into pole – approximately 1/4 of the length.

pole

Slot in top of

pole

O-ring and chain

Take O-rings with chain attached and slide O-ring into slot on pole             – -remember the extension has only been put in part way into the pole to allow the O-rings to be put into the slot on the pole.

- Push extension all the way down until it stops.

- Take 12” pole and slide onto top of extension, line up top pole slots with bottom pole slots.

- Take o-rings with chain attached and slide o-rings into slot on top pole. See pictures above.

- Place ball top onto top pole and press until it snaps into place.