Alexander Reed

Hist 1993

Methodology

For this project, I propose to use the methodology outlined in Bernard (2018), building off of the earlier work of Janet DeLaine (1997, 2001), to perform a quantitative cost analysis on the circuit walls of Falerii Novi. The utilization of this methodology is particularly appropriate given the comparisons that have frequently been drawn between the enceinte at Falerii and the Servian walls which are the subject of Bernard’s publication (Salmon 1982, Hammond 1972). At the most basic level, Bernard’s analysis simply involves “applying standard work rates to a volumetric reconstruction of the original monument…facilitated by the modular and repetitive opus quadratum technique employed by the walls’ builders” (Bernard p. 77). Of course, actually doing so is, in practice, a far more complex and involved process.

Fortunately, however, the excellent state of preservation of the walls over the better part of their course has done much to ease the act of calculating their total size. The detailed plan of di Stefano Manzella (1979), in combination with the magnetometry data collected by the Tiber Valley Project, as well as the raw GPR data published along with commentary by Millet et al. (2019) should permit a fairly reliable calculation of the total length of their circuit, which can also be checked against an estimate of 2.5 km provided by Giuseppe Lugli (1957, p. 271) in his handbook on Roman masonry and construction techniques.

Calculating the height of the walls constitutes a somewhat more difficult task. Thankfully, Opitz (2009, p. 9) offers a useful average of 4 m, however this number fails to account for variations which are, in some sections, substantial, particularly along the southern edge of the wall where a depression in the terrain around the so-called “Porta Puteana” causes the wall’s height to rise to a maximum of 27 courses, or approximately twelve meters. The extent of the walls below the surface, generally about 3 m in the case of Rome’s Republican walls (Bernard p. 82) as a means of stabilizing the structure, is also unknown at Falerii. While, given their smaller size, it is unlikely that the substructure of the fortifications was as extensive as at Rome, modern photographs do indicate that the walls sit atop at least one additional, partially buried, course of blocks. Though the Soprintendenza Archeologia di Viterbo did recently undertake some exploratory work on and around the walls, the results of their excavation unfortunately remain unpublished. Yet even if these and other such complications are ignored, the fact remains that Opitz’s estimate is necessarily imprecise and that there exist no published alternatives against which it can be compared. Despite this, the largely even topography of the plain on which the settlement was sited, as well as data provided by Lugli (1957) concerning the height of the blocchi composing the wall, when combined with information available from previously published photographs should nevertheless be sufficient to permit an independent estimate of the average height of the structure over the length of its course, which can then be taken in coordination with that provided by Opitz.

Gaining an idea of the average thickness of the walls represents a still more formidable challenge. There exist no known publications of this measurement either in English or Italian, and it has not been recorded in any of the recent geophysical surveys of the site due to the extent of the overgrowth along its margins. Lugli (1957) does include a photograph of a cross-section along with his discussion of its construction techniques, however he fails to offer any relevant quantitative data. Still, the consistent thickness of the walls along their course, a much more regular metric than their height, should allow a rough approximation of their average width. Thanks to photographs of the walls and, notably, their portals, available online as well as Lugli’s comment that “l’altezza dei filari è costante in m. 0,45,” (p. 271), it is possible to compute, using ratios, an average thickness of between 170 and 180 centimeters.

All of this still leaves the numerous towers dotting the walls unexamined. Their traditional number, much debated, has long been assessed at 45, though di Stefano Manzella’s estimate of 49 to 63, further refined by McCall (2007) to between 58 and 59, has seen increasing use in more recent investigations of the site. Unfortunately, both publications have little to say concerning the dimensions and metrology of the towers themselves. Here we must again rely on photographs available from the internet and the measurements of the blocchi offered by Lugli. There do exist interesting comparanda from Cosa, described by Benvenuti (2002), however the principles of their design seem to differ such that they have little to offer in the present context. What we can say from the photographs is that the towers, measured both directly using the scale of the images as well as by multiplying the number of rows and registers with the dimensions of the blocks provided by Lugli, seem to have been roughly square in plan (400 cm x 400 cm) including the curtain thickness, an assessment congruent with di Stefano Manzella’s map of the site.

Though the height of the towers looks to have been somewhat more variable than the size of their footprint, nowhere do they appear to have reached higher than the wall-walk. It is conceivable that, as at Cosa, the towers may once have been fitted with some sort of masonry superstructure designed to support artillery, but no direct evidence of such elements has yet been found. As with other urban fortifications of the period, the towers of Falerii do not—excluding the possible, albeit doubtful, existence of a now lost superstructure—seem to have been chambered. Rather, they are built of solid blocks of tufo and presumably would have been accessed via stairways or ladders built into the interior face of the wall, rather than from an internal staircase.

Finally, it also remains to address the changes to the landscape undertaken in the course of the walls’ construction. All along the southern edge of the plateau on which the city is sited, from the area directly south of the medieval church all the way to the eastern face of the so-called Southeast Bastion—a stretch of nearly 700 meters—the bedrock has been quarried back to be flush with the external face of the wall. While much has been made of the additional labor given to constructing the artificial agger and fossa (embankment and ditch) at Rome as part of the city’s defensive network at roughly the same period, the nature of the landscaping at Falerii Novi is of a fundamentally different nature. As McCall (p. 101) notes, this quarrying served two functions, neither of which was primarily poliorcetic. First, to those coming into the city from the direction of Rome, it made the made the walls appear taller, and more impressive. Second, and more importantly for us, it provided an excellent and readily accessible source of construction materials for the builders. Though it remains difficult, in light of later modifications to the terrain, to calculate just how much stone was removed from the quarry, the uniformity of the stone blocks used to construct the walls and their identical composition to the exposed bedrock beneath them, provide strong evidence that nearly all of the material for the city’s walls was harvested locally: if not from the quarry immediately south of the city, then probably just across the Rio Purgatorio, from the cliffs that rise along the well-trod Via Amerina. While this key fact does not thus impact our calculation of the total quantity of material processed in the course of the walls’ construction, it will be a critical consideration in our understanding the total amount of labor expended upon them.

Altogether, then, we have a wall running some 2.5 km in length, standing on average 4 m high and 1.8 m wide, and containing some 18,000 cubic meters of stone, with the towers adding an additional 3,000 cubic meters of material. Though, once again, the only firm measurement that we have for the dimensions of the individual blocks is Lugli’s note that the height of each row is generally around 45 cm, photographs from the site indicate that each block measures approximately 90 x 45 cm in plan—congruent with the dimensions of the tufo giallo blocks cut on an ideal 2 x 2 x 4 module that make up the Servian walls—and therefore occupies a space of 0.182 m3. This then gives us a total of about 115,500 individual blocks quarried from the surrounding terrain and used in the construction of the wall.

The final factor left to consider, the efficiency of labor in each component of the construction process, is a subject about which much could be said. For the purposes of the present project, however, it will suffice to note that, so as to facilitate comparison with energetic studies of other, similar structures at sites like Rome and Syracuse, we will use the estimates of work rates provided in Adams (1994) and Bernard (2018). Though necessarily imperfect, based on a combination of experimental archaeology and analogues from a wide variety of situations and societies, the relatively wide currency of these rates renders them preferable to any other alternatives for the purpose of gaining a better understanding of Rome’s willingness to invest in its colonies and the extent to which it may have been necessary to collaborate with indigenous communities. It is hoped that it will then be possible to expand on this work through additional survey and data collection during subsequent field seasons in order to develop a more robust body of knowledge concerning the walls of Falerii Novi and, accordingly, a more robust picture of the encounter that took place there more than two millennia ago.

Bibliography

Abrams, Elliot. 1994. How the Maya Built Their World: Energetics and Ancient Architecture. Austin: University of Texas Press.

Benvenuti, Valérie. 2002. “The Introduction of Artillery in the Roman World: Hypothesis for a Chronological Definition Based on the Cosa Town Wall.” Memoirs of the American Academy in Rome 47:199–207.

Bernard, Seth. 2018. Building Mid-Republican Rome: Labor, Architecture, and the Urban Economy. Oxford: Oxford University Press.

DeLaine, Janet. 1997. The Baths of Caracalla: A Study in the Design, Construction, and Economics of Large-Scale Building Projects in Imperial Rome. Portsmouth: Journal of Roman Archaeology.

2001. “Bricks and Mortar: Exploring the Economics of Building Techniques at Rome and Ostia.” In Economies Beyond Agriculture in the Classical World, edited by John Salmon and David Mattingly, 242–80. New York: Routledge.

Lugli, Giuseppe. 1957. La tecnica edilizia romana, con particolare riguardo a Roma e Lazio. Roma: G. Bardi.

McCall, Walter Frank. 2007. Falerii Novi and the Romanisation of Italy During the Mid-Republic.

Millett, Martin, Lieven Verdonck, Ninetta Leone, and Alessandro Launaro. 2019. Beneath the Surface of Roman Republican Cities. York: Archaeology Data Service. https://doi.org/10.5284/1052663.

Opitz, Rachel. 2009. “Integrating Lidar and Geophysical Surveys at ‘Falerii Novi’ and ‘Falerii Veteres’ (Viterbo).” Papers of the British School at Rome 77: 1–343.

Salmon, Edward Togo. 1982. The Making of Roman Italy. Ithaca: Cornell University Press.

di Stefano Manzella, Ivan. 1979. Falerii novi negli scavi degl'anni 1821-1830: con un catalogo degli oggetti scoperti, un'appendice di documenti inediti e una pianta topografica. Roma: "L'Erma" di Bretschneider.