
INSTALLATION AND SAFETY INSTRUCTIONS
[image: image1.wmf]FOR YOUR SAFETY WARNING
BE SURE THE ELECTRICITY TO THE WIRES YOU ARE WORKING ON IS SHUT OFF; EITHER THE FUSE IS REMOVED OR THE CIRCUIT BREAKER IS SHUT OFF. GENERAL You don’t need special tools to install this fixture. Be sure to follow the steps in the order given. Under no circumstances should a fixture be hung on house electrical wires, nor should a swag type fixture be installed on a ceiling which contains a radiant type heating system. Read instructions carefully. If you are unclear as to how to proceed, consult a qualified electrician. NOTE: Proper wiring is essential for the safe operation of this fixture.
GENERAL
You don’t need special tools to install this fixture. Be sure to follow the steps in the order given. Under no circumstances should a fixture be hung on house electrical wires, nor should a swag type fixture be installed on a ceiling, which contains a radiant type heating system. Read instructions carefully. If you are unclear as to how to proceed, consult a qualified electrician.

NOTE: Proper wiring is essential for the safe operation of this fixture.
FIXTURE ASSEMBLY
Carefully removed the fixture from the carton and check that all 
parts are included as shown in figure. Be careful not to misplace 
any of the screws or parts, which are needed to install this fixture.
INSTALLATION
IMPORTANT: Do not attach fixture directly to outlet box.
STEP 1: Secure back plate (A) to outlet box with outlet box screws (B), then allowing for the wood screws (D) to protrude through the back plate （A) holes onto the plastic wall anchors(C), and secure wood screws (D).
GROUNDING INSTRUCTIONS
STEP 1: Insert the green grounding screw into the hole with two raised dimples on the circular strap. Wrap the ground wire from the fixture (if supplied) around the green grounding screw, then connect it to ground wire from the outlet box (if not fixed on the outlet box) using a wire connector (not supplied). If ground wire from the fixture is not supplied, wrap the ground wire (if not fixed on the outlet box) from the outlet box around the green grounding screw and tighten it.

NEVER CONNECT GROUND WIRE TO BLACK OR WHITE POWER SUPPLY WIRES.
STEP 2: Take note of the color of the wire(s) on your fixture.

Identify which group your fixture wire(s) falls into and connect the wires according to the directions below:
	GROUP  A:  Connect  to  Black

House Wire
	GROUP  B:  Connect  to  White

House Wire

	BLACK
	WHITE

	*PARALLEL  WIRE  (round  &

smooth)
	*PARALLEL  WIRE  (square  &

ridged)

	WHITE    OR    GREY    WITH TRACER
	WHITE  OR  GREY  WITHOUT TRACER

	BROWN  GOLD  OR  BLACK WITHOUT TRACER
	BROWN,   GOLD   OR   BLACK WITH TRACER


*NOTE: When parallel wire (SPT-I & SPT-2) is used, the tracer

wire is square shaped or ridged and the less tracer wire is round
in shape or smooth (seen best when viewed from wire end.)
STEP  3:Take  your  fixture  wire(s)  from  group  A  and  place
evenly against the black wire from the outlet box. Do NOT twist wires together before using wire connectors.
STEP 4: Fit a wire connector (not supplied) over the wires and
screw the connector clockwise until you feel a firmness.
STEP 5: Try gently to pull the connector off the wires. If you can pull the connector off, carefully re-do steps 3 and 4, as above, and check again for a firm connection .

STEP 6: Connect the fixture wire from group B to the white wire from the outlet box in the same manner. Make sure no bare wires can be seen outside wire connectors.
FINAL ASSEMBLY
STEP 1: After wires are connected, install the front plate (E) onto  back plate (A), then secure front plate (E) to back plate (A) with screws (F)  

STEP 2: Install bulb (I) to socket (H). (Bulb not included).
STEP 3: Secure fixture frame (J) to front plate (A) with screw (G) 
STEP 4: Attach crystal panel (K) and (L) to the fixture frame (J) according to the drawing below. 
CLEANING
To clean, wipe fixture with a soft cloth. Clean glass with a mild soap. Do not use abrasive materials such as scouring pads or powders, steel wool or abrasive paper.

ORDERING PARTS
Keep this sheet for future reference, and in case you need to order replacement parts. All parts for this fixture can be ordered from place of purchase. Be sure to use exact wording from illustration when ordering parts.

Line art shown may not exactly match the fixture enclosed. However, the installations do apply to this fixture.
ITEM NO: 822-2S 

� EMBED AutoCAD.Drawing.18  ���








_1727248735.dwg

