3 LUG ENDCAP DRIVER TOOL
FOR STRATUS Suppressor

For use on .22LR TAC suppressor.

Material:

ABS Plastic

100% densification

Dimensions:

1" hex

Over all Hight = 0.70”

Lug Hight = 0.09”x 3pls

Lug O/S Rad. = 0.45”x 3pls

Lug I/S Rad. = 0.30”x 3pls

Lug end Rad. = 0.0675”x 6pls

Gap between each Lug = 0.235”x 3pls.

Lug cord length = (0.51” x 3pls)

OD below lugs = 0.91”

OD Hight = 0.16”

Hex Hight = (0.50”)

Not Shown:

0.25” Hex thru center replaces the ID shown

.225”3pl

Lug 3 plc

0.25” Hex instead of a hole.

All Features are intended T/B on Center