UEFI Shell Update BIOS & EC & ME SOP

Z14AK

11/22/2018

- If you don’t have any USB shell, please create a UEFI Shell at USB dongle.

  - Put these files into USB dongle as blow and follow the path: ( “H” is my USB )

    - H:\efi\boot\bootx64.efi (this file can download from internet)

    - H:\efi\boot\startup.nsh

    - H:\BIOS\AfuEfix64.efi

    - H:\BIOS\FALL.nsh

    - H:\BIOS\FBIOS.nsh

    - H:\BIOS\FlashEC.nsh

    - H:\BIOS\Fpt.efi

    - H:\BIOS\fparts.txt

    - H:\BIOS\Z14AXXXX.IMG (BIOS file)

    - H:\BIOS\Z14AKXXX.bin (EC ROM file)

  - When enter the shell, the script is “FALL.nsh” and “FBIOS.nsh”

    - FALL.nsh : ME , BIOS and EC Update

    - FBIOS.nsh : BIOS update

    - ECflashEC.nsh: EC update

Update All Me, BIOS and EC

Fall.nsh steps:

Step 1: Switch ME to unlock mode and system will turn off,

Step 2, Press power button to turn on PC

Step 3: Re-run Fall.nsh to Update ME and BIOS

Step 4: Switch ME to Lock

Step 4: Update EC and system will power off

Update BIOS: FBIOS.nsh

Update EC: ECflash.nsh and system will power off