# ST

Chapter summaries

CHAPTER 13

Mechanical engineering

Chapter summaries

1  	what is mechanical engineering?

•	Mechanical engineering is a branch of engineering that focuses on the design, production, analysis, working and improvement of technical objects with moving parts (p. 426).

2 	LINKING IN TECHNICAL OBJECTS

•	A link holds two or more parts of the same technical object together (p. 427).

•	In mechanics, a component is a part or fluid that performs a mechanical function (p. 427).

•	Linking is the mechanical function performed by any component that connects different parts 
of a technical object (p. 427).

•	Every link displays four basic characteristics: direct or indirect, rigid or flexible, removable or non-removable, and complete or partial (pp. 427–428).

3 	GUIDING CONTROLS

•	Guiding is the mechanical function performed by any component that controls the motion of one or more moving parts (p. 431).

•	A guiding component or control is a component whose mechanical function is to guide the motion of moving parts (p. 431).

•	There are three main forms of guiding: translational, rotational and helical (p. 431).

•	Translational guiding ensures the straight translational motion of a moving part (p. 431).

•	Rotational guiding ensures the rotational motion of a moving part (p. 432).

•	Helical guiding ensures the translational motion of a moving part while it rotates about the same axis (p. 432).

4 	MOTION TRANSMISSION SYSTEMS

•	Motion transmission is the mechanical function of relaying a motion from one part to another without altering the nature of the motion (p. 435).

•	A motion transmission system is a set of components that perform the function of transmitting motion (p. 436).

•	Any mechanical system contains a driver (component) and at least one driven component. Some systems also contain an intermediate component (p. 436).

•	The most common motion transmission systems are:

- gear trains

- chain and sprocket systems

- worm and worm gear systems

- friction gear systems

- belt and pulley systems (p. 436)

•	In mechanical engineering, a system is described as reversible when a driven component can become a driver, and vice versa. Of the common motion transmission systems, only the worm and worm gear system is irreversible (p. 437).

•	In a gear train:

- The teeth of all the gears in the system must be identical. They must have the same shape and direction and be equally spaced.

- When rotational axes are parallel, straight gears are commonly used. When the axes are perpendicular, bevel gears are preferred.

- The higher the number of teeth, the lower the speed of rotation (p. 438).

•	In a chain and sprocket system:

- The teeth on the system sprockets must be identical.

- The chain links must mesh easily with the sprocket teeth.

- The system requires frequent lubrication to avoid the rapid wear of the teeth and chain.

- The smaller a sprocket, the faster it turns (p. 439).

•	In a worm and worm gear system:

- The groove of the worm must fit the gear teeth so that they can mesh.

- The driver (to which the force is applied) must be the worm (p. 440).

•	In a friction gear system:

- Straight, bevel and spherical friction gears may be positioned along parallel, perpendicular 
or other rotational axes.

- The larger the diameter of a gear, the slower its rotation.

- Friction between gears must be high. For this reason, the gear rims must be made of materials that adhere well to one another (p. 441).

•	In a belt and pulley system:

- Pulleys must contain a groove where the belt can fit. The groove must be smooth to prevent damage to the belt.

- The belt must adhere to the pulleys to avoid slipping as much as possible.

- The smaller a pulley, the faster it turns (p. 441).

•	A speed change occurs in a motion transmission system when the driver does not turn at the same speed as the driven component or components (p. 442).

•	In a worm and worm gear system, the greater the number of teeth on the gear, the greater the decrease in speed (p. 442).

•	In friction gear systems and belt and pulley systems:

- There is an increase in speed when motion is transmitted from one gear or pulley to a gear or pulley of smaller diameter.

- There is a decrease in speed when motion is transmitted from one gear or pulley to a gear or pulley of larger diameter.

- There is no change in speed when motion is transmitted between two gears or pulleys of the same diameter (p. 443).

•	In gear trains and chain and sprocket systems:

- There is an increase in speed when motion is transmitted from one gear or sprocket to another with fewer teeth.

- There is a decrease in speed when motion is transmitted from one gear or sprocket to another with more teeth.

- There is no change in speed when motion is transmitted between two gears or sprockets with the same number of teeth (p. 443).

•	To find out the ratio of a speed increase or decrease in a motion transmission system, the ratio between the gear diameters or between the numbers of teeth on the gears must be calculated (p. 443).

5 	MOTION TRANSFORMATION SYSTEMS

•	Motion transformation is the mechanical function of relaying a motion from one part to another while altering the nature of the motion (p. 445).

•	The most common motion transformation systems are:

- rack and pinion systems

- screw gear systems

- cam and follower systems

- slider-crank mechanisms (p. 446)

•	In a rack and pinion system:

- The teeth on the rack and on the pinion must be identical.

- The system requires frequent lubrication to prevent wear.

- The greater the number of teeth on the pinion, the slower its rotation (p. 447).

•	There are two types of screw gear systems that can transform motion:

- In the first type, the screw is the driver, and its rotational motion is transformed into the translational motion of the nut.

- In the second type, the nut is the driver, and its rotational motion is transformed into the translational motion of the screw (p. 448).

•	In the first type of screw gear system:

- The nut must be connected to the screw in such a way that the nut cannot rotate.

- The threads of the screws and nuts must match (p. 448).

•	In the second type of screw gear system:

- The nut must be fixed in such a way that its only possible motion is rotational.

- The threads of the screws and nuts must match (p. 448).

•	In a cam and follower system:

- The follower must be guided in its translational motion.

- The shape of the cam determines how the follower will move.

- A device such as a return spring is usually necessary to keep the follower in continual contact with the cam (p. 448).

•	In a slider-crank mechanism:

- The connecting rod contains two bushings to attach it to the crank and piston.

- The cylinder must guide the part moving in translation.

- The system requires frequent lubrication (p. 449).