Components with other functions

Support activities – Second Year of Secondary Cycle Two

ACTIVITY 63

AST

student BOOK: Chapter 14, pages 473–478

RELATED HANDOUT: Concept review 63

1.	Complete the following sentences, using the words or groups of words in the box below. You may use some words more than once.

| • amplify • bigger • block • capacitance • capacitor • collector • computer • digital circuits | • diode(s) • direct current • direction • electrical charge • electrical contact • electromagnet • electromechanical relay | • emitter • farads • high-voltage • insulating material • light-emitting diodes • light-sensitive cells • low-voltage | • powerful • protect • relay • signal • solid-state relay • stabilize • transistor(s) |
| --- | --- | --- | --- |

a)	A ___________________________ is a device composed of two electrical surfaces separated by an ___________________________. The device can store an ___________________________. This ability to store energy is called “___________________________” and is expressed in ___________________________.  A capacitor can release its stored charges almost instantly; it can also serve to ___________________________ a fluctuating power supply.

b)	A ___________________________ is a device that allows electric current to flow in only one ___________________________. ___________________________ can guide and block an electric current, ___________________________ fragile electronic parts and convert alternating current into ___________________________. ___________________________ (LEDs) give off light when a current flows through them.

c)	A ___________________________ is an electronic device used to ___________________________ or ___________________________ an electric current. It is made up of a ___________________________, a base and an ___________________________. ___________________________ play an important role in the field of electronics.

d)	A ___________________________ is a component used to control a ___________________________ circuit from a distance with a ___________________________ circuit. The ___________________________ 
circuit emits a___________________________ that can come from a switch, a ___________________________ or a photoelectric (light-sensitive) cell. There 
is no ___________________________ between the two circuits. In an ___________________________, the control depends on an ___________________________, which causes metallic parts to move. In a ___________________________, control is made possible by an LED and ___________________________. An ___________________________ has a shorter life span and is ___________________________, but it can control more ___________________________ circuits. A ___________________________ is usually airtight and more compatible with ___________________________.

2. 	What am I?

a)	I can be found in remote controls, where I protect electronic parts when the battery is installed upside down.

b)	There are millions of me in computers.

c)	I am made up of two metal plates separated by a dielectric.

d)	I am used in theatre lighting systems, the heating systems of public buildings and industrial motors.

3.	Explain how a camera flash works.

4. 	True or false? If the statement is false, correct the error.

a)	LED bulbs consume less energy than ordinary light bulbs.

b)	In a transistor, the collector acts like a door that can block or facilitate current flow.

c)	Binary code works with the help of capacitors.

d)	A diode is made of silicon, which is a semiconductor.

5.	How does a diode transform alternating current into direct current?