Components with other functions

Support activities – Second Year of Secondary Cycle Two

ACTIVITY 63

EST

STUDENT BOOK: Chapter 14, pages 473–475

RELATED HANDOUT: Concept review 63

1.	Complete the following sentences, using the words or groups of words in the box below. You may use some words more than once.

| • capacitance • capacitor • diode(s) | • direct current • direction • electrical charge | • farads • insulating material • light-emitting diodes | • protect • stabilize |
| --- | --- | --- | --- |

a)	A ___________________________ is a device composed of two electrical surfaces separated by an ___________________________. The device can store an ___________________________. This ability to store energy is called “___________________________” and is expressed in ___________________________.  A capacitor can release its stored charges almost instantly; it can also serve to ___________________________ a fluctuating power supply.

b)	A ___________________________ is a device that allows electric current to flow in only one ___________________________. ___________________________ can guide and block an electric current, ___________________________ fragile electronic parts and convert alternating current into ___________________________. ___________________________ (LEDs) give off light when a current flows through them.

2. 	What am I?

a)	I can be found in remote controls, where I protect electronic parts when the battery is installed upside down.

b)	I am made up of two metal plates separated by a dielectric.

3.	Explain how a camera flash works.

4. 	True or false? If the statement is false, correct the error.

a)	LED bulbs consume less energy than ordinary light bulbs.

b)	A diode is made of silicon, which is a semiconductor.

5.	How does a diode transform alternating current into direct current?