SECTION 08 53 13   VINYL WINDOWS

PART 1     GENERAL

1.01	SUMMARY

- Section Includes:

  - Fixed Vinyl Window Units

  - Glass and Glazing for Vinyl Windows.

  - Wood Blocking, Shims, Anchors, Clips, and all accessories necessary for a complete installation furnished and installed.

  - All vinyl trim and closure pieces

  - Installation labor, tools, equipment, and services necessary for installation of Vinyl Windows.

- Related Sections:

      - Section 07 62 00  Sheet Metal Flashing and Trim

      - Section 07 92 00  Joint Sealants

      - Section 08 32 16  Sliding Vinyl Doors and Frames

      - Section 08 41 13  Aluminum Entrances and Storefront

      - Section 08 80 00  Glazing

- 1.02	REFERENCES

- American Architectural Manufacturers Association (AAMA):

  - 101 – “Voluntary Performance Specification for Windows, Skylights and Glass Doors”:  AAMA/WDMA/CSA 101/I.S.2/A440 “Standard/Specification for Windows, Doors and Unit Skylights”

  - 502 – “Voluntary Specification for Field Testing of Newly Installed Fenestration Products”

  - 615 – “Voluntary Specification, Performance Requirements And Test Procedures For Superior Performing Organic Coatings On Plastic Profiles”

  - 664 – “Voluntary Specifications for In-Process Quality Control- Requirements for Applicators of Synthetic Interior and Exterior Laminates to Substrates of:  Cellulosic Composite, Cellular Polyvinyl Chloride (PVC), Fiber Reinforced Thermoset, Finished Aluminum, Rigid and Reinforced Thermoplastic, and Wood.

  - 1503 – “Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections”

  - 2400 – “Voluntary Specification for Installation of Windows with a Mounting Flange in Stud Frame Construction”

- American National Standards Institute (ANSI) Publications

  - Z97.1 – “Performance Specifications and Methods of Test for Safety Glazing Materials Used in Buildings”

- ASTM International (ASTM) Publications:

  - C518 – “Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus”

  - C1036 – “Standard Specifications for Flat Glass”

  - C1048 – “Standard Specifications for Heat-Treated Flat Glass – Kind HS, Kind FT Coated and Uncoated Glass”

  - D3985 – “Standard Test Method for Oxygen Gas Transmission Rate Through Plastic Film and Sheeting Using a Coulometric Sensor”

  - E90 – “Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements”

  - E283 – “Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen”

  - E330 – “Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors by Uniform Static Air Pressure Difference”

  - E331 – “Standard Test Method for Water Penetration of Exterior Windows, Curtain Walls, and Doors by Uniform Static Air Pressure Difference”

  - E413 – “Classification for Rating Sound Insulation”

  - E547 – “Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential”

  - E774 – “Standard Specification for Sealed Insulating Glass Units”

  - F1249 – “Standard Test Method for Water Vapor Transmission Rate Through Plastic Film and Sheeting Using a Modulated Infrared Sensor”

- National Glass Association (NGA): “GANA Glazing Manual”

- Insulating Glass Certification Council (IGCC)

- National Fenestration Ratings Council (NFRC)

- U.S. Consumer Product Safety Commission (CPSC) Publications:

  - 16 CFR Part 1201 “Safety Standard for Architectural Glazing Materials”

1.03	SUBMITTALS

- Submit “Letter of Conformance” in accordance with Section 01 33 00 (01330) – with the following supporting data:

  - Product data for each type of vinyl window specified, including standard construction details, dimensions of individual components, profiles, finishes, and accessories.

  - Shop drawings for each type of window specified, including ¼-inch scale wall elevations, typical unit elevations at ¾-inch scale details, full size details of typical composite members and the following:

    - Panning Details

    - Flashing and drainage details.

    - Mullion details, including reinforcement and stiffeners.

    - Joinery details.

  - Samples:  Provide full-size or partial-size sample of window illustrating glazing system, quality of construction and finish.

  - Product certificates signed by the window manufacturer certifying that window units comply with specified performance requirements.

  - Submit certified independent laboratory test reports verifying compliance with all test requirements of 1.05 PERFORMANCE REQUIREMENTS as requested by Architect.

1.04	DEFINITIONS

- Performance grade number, included as part of the AAMA/WDMA/CSA     101/I.S.2/A440 product designation code, is actual design pressure in pounds force per square foot used to determine structural test pressure and water test pressure.

1.05    PERFORMANCE REQUIREMENTS

- Certify that windows have been tested in accordance with American	       Architectural Manufacturers Association (AAMA/WDMA) Specification 	        for Performance Class specified complying with the following performance 	       standards:

### AAMA/WDMA/CSA 101/I.S.2/A440 Performance Requirements:  Provide vinyl windows of the performance class and grade indicated that comply with AAMA/WDMA/CSA 101/I.S.2/A440.

    - Performance Class:	F- R

    - Performance Grade:  40

  - Structural Test Performance Requirements (ASTM E330):

    - Uniform Load Structural Test:  Unit to be tested at 60 PSF, both positive and negative, with no glass breakage; damage to make windows inoperable; or permanent deformation of any main frame or ventilating member in excess of 0.2% of its clear span.

  - Water Resistance (ASTM E331 and ASTM E547):	No uncontrolled water penetration at test pressure indicated.

    - Class F-R-40:  6.0 PSF

  - Air Infiltration (ASTM E283):

    - Fixed Windows:  Maximum 0.30 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure differential of 1.6 PSF minimum.

- Project Wind Loads:

Please specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by boundary layer wind tunnel testing.  Minimum for positive and negative pressure = 10.0 p.s.f.

  - The system shall be designed to withstand the following loads with respect to the plane of the wall:

    - Positive pressure of ___ p.s.f. at non corner zones.

    - Negative pressure of ___ p.s.f. at non corner zones.

    - Negative pressure of ___ p.s.f. at corner zones.

1.06	QUALITY ASSURANCE

- All window units shall be manufactured by a single source.

  - All windows in any one project must be by the same manufacturer and with comparable frame depth, profile, glazing bite, and installation requirements.  Manufacturer must provide a window system that can incorporate all window configurations used on the project.

  - Standards:  Requirements for vinyl windows, terminology and standard of performance, and fabrication workmanship are those specified and recommended in AAMA/WDMA/CSA 101/I.S.2/A440.

    - All window units shall be labeled as conforming to AAMA/WDMA/CSA 101/I.S.2/A440.  The label shall state the name of the manufacturer, the approved labeling agency, and the product designation as specified in Section 1.05 “PERFORMANCE REQUIREMENTS” above.

    - All testing shall be conducted using (AAMA/WDMA/CSA 101/I.S.2/A440) Gateway Performance minimum specified test sizes.

1.07	DELIVERY, STORAGE AND HANDLING

- Transportation and Handling:  Transport products by methods to avoid product damage, deliver in undamaged condition in manufacturer’s unopened containers or packaging.  Provide equipment and personnel to handle products by method to prevent soiling or damage.  Promptly inspect shipments to assure products comply with requirements, quantities are correct, and products are undamaged.

- Storage and Protection:  Store products in accordance with manufacturer’s instructions, with seals and labels intact and legible.  Store sensitive products in weather-tight enclosures; maintain with temperature and humidity ranges required by manufacturer’s instruction.

1.08	 WARRANTIES

- Vinyl Window Warranty

  - Products:  Submit a written warranty, executed by the window manufacturer, for the following:

    - Framing components:  A period of (5) years from the date of manufacture, against defective materials and workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which results in premature failure of the windows or parts, outside of normal wear.

    - Insulating glass units:  A period of (10) years from the date of manufacture, against insulating glass seal failure unrelated to glass breakage.

    - In the event windows or components are found defective, manufacturer will repair or provide replacements without charge at manufacturer’s option.

    - Where applicable, materials which are applied to the face of insulating glass for the purpose of simulating division in glass openings (SDL’s) are warranted against detaching from the glass surface for a period of (5) years.

    - Finish:  Refer to Part 2, Section 2.06 “FINISHES” for warranty requirements.

    - Warranty for all components must be direct from the manufacturer (non- pass through) and non- prorated for the entire term.  Warranty must be assignable to the non-residential owner, and transferable to subsequent owners through its length.

  - Installation:  Submit a written warranty, executed by the window installer, for a period of (1) year from the date of substantial completion, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements, which result in premature failure.

    - In the event installation of windows or components is found to be defective, installer will repair or provide replacements without charge at the installer’s option.

PART 2     PRODUCTS

2.01   MANUFACTURERS

- Approved Manufacturers:

  - Quaker Window Products Company, Inc. (800) 347-0438

    - Fixed Window: “V100 DS Series”

  - Substitutions: Only pre-approved products specified by the Architect will be acceptable.  Submit the following information with proper documentation as required for pre-bid substitution requests, and at least (10) working days prior to bid date.

    - Independent test reports certifying that proposed product is in accordance with, and meets all criteria specified in Section 1.05 “PERFORMANCE REQUIREMENTS”.

    - Drawing details of elevations and sections, and samples in accordance with, and as specified in Section 1.03 “SUBMITTALS”.

    - Copy of manufacturer’s warranty specified in accordance with, and as specified in Section 1.08 “WARRANTIES”.

    - Any additional information requested by the Architect.

2.02 	MATERIALS

- Vinyl Members:

  - All extruded vinyl sections shall be of Polyvinyl (vinyl chloride) (uPVC).

2.03	MANUFACTURED UNITS

- Principal window frame members and insert members shall be manufactured in accordance with AAMA/WDMA/CSA 101/I.S.2/A440.

- Window frame depth shall be 2 3/4” minimum.

- Glazing:  Refer to Section 2.05 “GLASS MATERIALS”.

- Window frames shall be designed to receive snap-on accessory components to exterior

2.04	COMPONENTS

- All fasteners, tools, equipment, and other materials necessary for a complete 	     installation shall be furnished by the Contractor.

- Thermoplastic or thermo-set plastic caps, housings, and other components to	      be injection-molded nylon, extruded PVC, or other suitable compounds.

- Accessories:

  - Nailing fin: Shall be integral to the frame and consist of extruded vinyl as a manufacturer’s standard.  							Optional Item

  - Snap-in J-channel around perimeter of windows shall be provided, and field applied.

2.05	GLASS MATERIALS:

Please select one of the [ item “A.”] glass types below which is in accordance with design.  The recommended Low-Emissivity (Low-E) coating options are listed below; however, if the design requires tinted glass, or another specific type of (Low-E), please choose the blank option and add pertinent information

- Clear Float Glass:  ASTM C1036, Type 1 (transparent glass, flat), 			     Class 1 (clear), Quality q3 (glazing select) as manufactured by one of the 	     following:

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat), Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

  - Low E Coating:  Surface #2 on insulating units

  - Approved Manufacturers:

    - “LoE2- 272”; Cardinal Industries

    - Approved Substitution by Architect

  - Whole Window U-Value shall be a maximum of 0.28.

  - Solar Heat Gain Coefficient shall be a maximum of 0.35.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat), Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

  - “LoE3-366”; Cardinal Industries

  - Approved Substitution by Architect.

  - Whole Window U-Value shall be a maximum of 0.27.

  - Solar Heat Gain Coefficient shall be a maximum of 0.23.

- Tempered Glass:  Condition A (uncoated surfaces), Type 1	 		   	      (transparent glass, flat), Class 1 (clear), Quality q3, clear, fully tempered 		      safety glass (meet requirements of ANSI Z97.1).

  - All tempered glass shall conform to ASTM C1048, ANSI Z97.1, and CPSC 16 CFR Part 1201.  Tempered glass shall bear permanent monogram indicating tempered quality.  Fabrication marks on tempered glass shall be located to be concealed in completed installation.

- Windows shall be glazed as follows:

  - Sound Transmission Class (STC) (ASTM E413):  Provide glazing required for conforming to overall STC ratings as specified for vinyl windows.

- Insulating Glass:  Manufacturer’s standard units that comply with specified quality standards and coatings.

  - Provide preassembled units consisting of organically sealed panes of glass enclosing a hermetically sealed dehydrated air space and complying with ASTM E774 for performance classification indicated as well as with other requirements specified for glass characteristics, air space, sealing system, sealant, space material, and desiccants.

    - Total thickness:  3/4”

    - Thickness of Each Pane:  as necessary to meet structural performance criteria.

    - Air Space:  argon gas filled

  - Exterior Pane of Glass:

    - Provide tempered glass where shown on Drawings and as required by local codes and ordinances.

  - Insulating Unit Sealing System: Insulating glass unit spacer system must include a secondary dual seal. This also applies to solid foam warm edge seal glass spacer systems.

2.06	FINISHES

- Finish of Vinyl Components:

  - Finish of Vinyl Windows and Frames:  White

Optional exterior coating:

  - Acrylic Based Exterior Film Application:

    - Multi-Layered Laminate Film shall consist of the following heavy-metal -free composition:

      - Base film composed of colored acrylic (PMMA).

      - Two layers of transparent protective surface consisting of Polymethyl Methacrylate (PMMA) middle layer, and a Polyvinylidene Fluoride (PVDF) top layer

    - Laminate Film shall conform to (AAMA 664).

    - Non acrylic based film products, or any film products containing PVC shall not be allowed.

    - Film Warranty Period:  20 years from date of manufacture consisting of a 10-year full replacement warranty with labor and materials included, and an additional 10-year prorated finish material only warranty.

    - Color Selection:  Please select a color

    - Exterior film shall cover all exposed exterior window frame and sash components.

2.07	FABRICATION

- Fabricate windows allowing for minimum clearances and shim spacing around perimeter of assembly yet enabling installation.

- Develop drainage holes with moisture pattern to exterior.

- Prepare components to receive anchor devices.  Fabricate anchorage items.

- All joints of main frames and sashes shall be mitered and joined neatly and secured by the means of fusion welding.

PART 3      EXECUTION

3.01	 EXAMINATION

A.  Site Verification of Conditions:

1.  	Verify that building substrates permit installation of windows according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

2.  	Do not install windows until unsatisfactory conditions are corrected.

3.02	 INSTALLATION

A. Erection of Vinyl Windows

- Install windows with skilled tradesmen in exact accordance with approved Shop Drawings, Installation Instructions, Specifications, and in accordance with (AAMA 101/I.S.2/A440).

- Windows must be installed plumb, square, and level for proper weathering and operation.  Jambs must not be “sprung”, bowed, or warped during installation.

Optional Item

B.   Field Tests

- Field testing procedure of installed windows shall be in accordance with AAMA 502.

- The test pressure used during the field test procedure shall be 2/3 of the rated test pressure of the test specimen in accordance with AAMA 502.  There shall be no optional variances over 2/3 used during testing.

- Initial field testing must be performed prior to no more than 5% of windows have been installed.  or specify

- All field testing expenses shall be at the burden of specify

3.02   ADJUSTING AND CLEANING

A.     After completion of window installation, windows shall be inspected, 	adjusted, put into working order and left clean, free of labels, dirt, or other 	debris.  Protection from this point shall be the responsibility of the General 	Contractor.

END OF SECTION