Script for Advanced Motion Manager

Basics of Camera

- Open F-5_tiger.sldprt

- Add a camera

- Show numeric controls for position of target point

- Show numeric controls for position of camera

- Toggle floating and target camera

- Move camera and target in second window

- Change FOV

- Change FOV aspect ratio

- Attach target point to curve (slide)

- Attach camera to point

- Set roll

- Press ok

- Unlock camera

- Move camera (use all the on screen controls)

Multiple Cameras

- Open jeep_through_cones.sldasm

- Show sketches

- Play animation of jeep traveling through cones

- Add camera:

  - Camera on point in assembly

  - Target on jeep

  - Roll control to top plane

  - Rename camera: camera_pan_with_jeep

- Set time 0 view orientation to be camera_pan_with_jeep; replace key

- Play animation

- Add camera

  - Camera on point in jeep

  - Target on point out of jeep

  - Roll control to top plane

  - Rename camera: camera_sled_jeep

- Set time 0 view orientation to be camera_slep_jeep; replace key

- Turn off sketches

- Play animation

- Add camera

  - Camera on curve @ 7%

  - Target on curve @ 6%

  - Roll control to top plane

  - Adjust FOV

  - Rename camera: front_of_jeep

- Set time 0 view orientation to be front_of_jeep; replace key

- @ time 10 edit camera:

  - Camera on curve @ 100%

  - Target on curve @ 99%

- Turn off sketches

- Play animation

- Add camera

  - Set to floating

  - Position right next to the car

  - Rename to be: floating camera

- Set time 0 view orientation to be front_of_jeep; replace key

- @ time 5 move camera

- @ time 10 move camera

- Play animation

- Change cameras in animation

Mates/Suppress

- Open conveyor_system.sldasm

- Disable play back of view keys

- Show the distance mates

- Show the animation from top view

- Set speed to 200%

- Show from side view

- Zoom in the animation so you get seconds or half seconds

- At 4.5 seconds:

  - Suppress:

    - Parallel to horizontal

    - Coincident to top

  - Unsuppress

    - Parallel to slope belt

    - Coincident to slope belt

- At 7.5 seconds:

  - Suppress:

    - Parallel to slope

  - Unsuppress:

    - Coincident to lower flat belt

- At 7.75 seconds:

  - Suppress:

    - Coincident to slope belt

  - Unsuppress:

    - Parallel to horizontal

- Talk about how the mates could have been added in the motion study itself (local mates).

Moving Cut/Motor

- Open sharpener_and_pencil.SLDASM

- Show how the mates work:

  - pencil_cut moves an empty component to which an assembly cut is attached (range 5-30)

  - pencil_position moves the pencil (range 155 – 250)

- Explain the script: pencil goes into the sharpener and gets cut

- Pencil Motion; edit pencil_position mate as follows:

  - Time 0: 250mm

  - Time 8: 155mm

  - Copy time 0 to time 12

  - Copy time 8 to time 9.5

- Play animation

- Pencil Cut; cut the pencil

  - Eyeball to 6.8 sec and copy time 0 pencil cut key

  - Time 8: 30mm

- Play animation

- Add a motor 200 rpm around pencil lead

- Copy on key point to 6.8sec

- Turn off at 9.6 sec

- Play animation

Part Relations and Equations

- Play animation

- Show that the animation only drives a distance mate

- Edit the driving sketch in context – show the relations to the plane and the slingshot (highlight relations – turn sketch relations display on)

- Show the equations that drive the size of the center of the elastic

Rendered Animation

- Note load file beforehand (make sure the scene works)

- Play animation unrendered

- Test render

  - Render Window

  - Box render

  - Preview Window

- Save animation to file

  - Show options