1.1  
STAND ALONE LOCKSETS

A.  
Battery Powered Cylindrical Lockset with Digital Keypad:

1.  
Product: Model E75K - Entrycheck Standalone Lockset with Digital Keypad as manufactured by SDC Security Door Controls.

a.  
Battery powered electronic lockset for 3000 users. Standard features include an ANSI Grade 1 cylindrical lockset with a lever clutch mechanism, 4-6 digit pin code, all weather keypad and housing, keypad or computer programming, 32 user groups, multi-year scheduling, temporary users, multiple zones, real time clock, auto daylight savings adjustment and more. 9/16 inch (9.5 mm) latch, 4-7/8 inch (124 mm) strike standard.

b.  
UL Listed, USA and Canada for 3 hour "A" label fire rated doors. ANSI A156.25, Electrified Lockset Compliant. ANSI A156.2, Grade 2 Compliant.

c.  
ANSI A117.1 Accessibility Code Compliant. ADA Compliant. California State Reference Code (Formerly Title 19, California State Fire Marshal):  Compliant. Levers Galaxy and Eclipse return to within 1/2 inch (13mm) of the door face.

** NOTE TO SPECIFIER **  Lever Handles with/without cylinders. Delete lever not required.

2.  
Lever and Cylinder: G1 - Galaxy lever, cylinder with keyway as required. 

3.  
Lever and Cylinder: E1- Eclipse lever, cylinder with keyway as required. 
4.  
G5 - Galaxy lever IC core prep, Tailpiece required.

5.  
E5 Eclipse lever IC core prep, Tailpiece required.

** NOTE TO SPECIFIER **  Black Dull Chrome is standard. Delete finish for keypad and lock that are not required.

6.  
Keypad Finish: 
1.  
Q - Dull chrome 

2.  
B – Black

3.  
C – Polished Brass

4.   
P – Bright Chrome

7. 
Lock Finish: 

a.  
Q- Dull chrome 

b.  
C – Polished Brass
c.  
P – Bright Chrome

