INTERNACHI® HEALTHY HOMES VISUAL INSPECTION CHECKLIST
Certified Healthy Homes Inspector Program at www.nachi.org/healthy-homes-certification. 

Date of Assessment_____
Client Name_____
House Address_____

WEATHER CONDITIONS:

__ Dry

__ Rain today or recently
__ Snow today or recently 
Temperature __°F

FRONT DOOR OF UNIT FACES:

__ North
__South
__East

__ West

TYPE OF UNIT BEING INSPECTED:

__Single-family
__Duplex
__Triplex
__Fourplex 
   __Townhome


UNIT STATUS:

__Occupied
__Vacant

MAJOR VISUAL ASSESSMENT AREAS

1. Site (Items 1–29)


2. Building Exterior (Items 30–62)


3. Building Systems (Items 63–81)


4. Common Areas (Items 82–102)


5. Housing Unit (Items 103–196)


6. Other Items (Items 197–229)


SITE (ITEMS 1–29)

Items to inspect in this category are:
· Fencing and Gates

· Grounds or Pavement

· Children’s Play Areas

· Other

A comment area for the site category is also included in this section.

FENCING AND GATES
Site 1. Damaged/Falling/Leaning/Deteriorated Paint
__Damaged but not functional or deteriorated paint in an area larger than 20 square feet: An exterior fence or gate is so damaged that it does not function as it should. An exterior fence, security fence, or gate is damaged and does not function as it should or could threaten safety or security.

__No fencing or gates present

__Damaged but functional; no deteriorated paint areas larger than 20 square feet: An exterior fence, security fence, or gate shows signs of deterioration, but still functions as it should, and it presents no risk to security or safety

__No damage

Site 2. Holes or Openings in Soil Below Fence

__≥6 square inches

__<6 square inches

__No holes or not applicable

GROUNDS OR PAVEMENT

Site 3. Areas of Erosion

__Large erosion (depression, rut or groove more than 8 inches wide by 5 inches deep):

Runoff has extensively displaced soil, which has caused visible damage to structures

__Advanced erosion threatens the safety of pedestrians or makes an area of the grounds unusable

__There is a rut larger than 8 inches wide by 5 inches deep

__There is extensive ponding

__Water or ice has collected in a depression or on ground where ponding was not intended

__Pooling of water (small erosion; depression, rut or groove less than 8 inches wide by 5 inches deep): Erosion has caused surface material to collect, leading to a degraded surface that would likely cause    water to pool in a confined area, especially next to structures, paved areas, or walkways, or a small groove is 6-8 inches wide and 3-5 inches deep
__No erosion

Site 4. Overgrown Vegetation

__Vegetation has damaged building. Plants have visibly damaged a component, area, or system of the property or have made them unusable/impassable. Vegetation is extensive and dense; it is difficult to see broken glass, holes, and other hazards.

__Vegetation penetrates an unintended surface—building; gutters; fence; wall; roof; HVAC

__Vegetation is producing excessive moisture that may lead to mold or mildew on nearby exterior building surface

__Tree is in danger of falling

__Vegetation is present or contacts building, but no damage: Extensive, dense vegetation obstructs the intended path of walkways or roads, but the path is still passable

__Vegetation is present but causes no problem

__No vegetation present

Site 5. Graffiti

__6 or more places: Graffiti in 6 or more places
__2–5 places: Graffiti in 2-5 places

 __One place: Graffiti in one place

__No graffiti

Site 6. Litter

__Excessive: More than 10 large trash or litter items 
__Slight or Moderate: 2–10 large trash or litter items 
__None: Fewer than 2 large trash or litter items

Site 7. Cracks in Pavement

__≥¾ inch, hinging/tilting, or missing section(s) that affect traffic ability over more than 5% of the property’s parking lots/driveways/roads or that cause trip hazards: major trip hazard

__<¾ inch displacement (vertical height): minor trip hazard

__No cracks: level

Site 8. Fire Ants/Mounds or Harmful Insects

__Yes: Presence of fire ants/mounds or harmful insects

__No: Fire ants/mounds or harmful insects not seen

Site 9. Containers That Hold Water

__Yes: Presence of water-holding containers conducive to mosquito breeding

__No: Water-holding containers not present

CHILDREN’S PLAY AREAS

Site 10. Equipment

__Equipment that poses an immediate threat: REPORT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__≥50% of surface area broken/damaged: Most of the equipment (50% or more) does not operate as it should, but poses no safety risk

__<50% of surface area broken/damaged: Less than half of the equipment does not operate as it should, but poses no safety risk

__No play equipment observed (skip to Site 21)

Site 11. Paint Condition

__Deteriorated paint on ≥50% of surface area of play area equipment

__Deteriorated paint on <50% of surface area of play area equipment

__No deteriorated paint on play area equipment

Site 12. Bare Soil

__≥9 square feet of bare soil present in play area

__<9 square feet of bare soil present in play area

Site 13. Injury-absorbent Surface Depth

__No: No injury-absorbent surface under playground equipment or injury-absorbent surface depth less than 12 inches

__Yes: Injury-absorbent surface under playground equipment at proper depth (at least 12 inches)

13b. Surface type: ___________________

Site 14. Deteriorated Injury-absorbent Surface

__Yes: Injury-absorbent surface ≥50% deteriorated 
__Yes: Injury-absorbent surface <50% deteriorated 
__No deterioration

Site 15. Fencing and Gates

__Damaged, not functional: An interior fence or gate is so damaged that it does not function as it should. An exterior fence, security fence, or gate is damaged and does not function as it should or could threaten safety or security.

__Damaged, but functional: An exterior fence, security fence, or gate shows signs of deterioration, but still functions as it should, and it presents no risk to security or safety

__No damage

__No play area fencing/gates

Site 16. Condition

__Refuse, animal feces, broken glass/sharp edges, or holes/trip hazards observed

__No refuse, animal feces, broken glass/sharp edges, or holes/trip hazards observed

Site 17. Exposed Bolts

__Yes: Exposed bolts are found on the playground equipment

__No: Playground equipment does not have exposed bolts

Site 18. Hanging or Choking Hazards

__Yes: Hanging and/or choking hazards on playground equipment

__No: No hanging and/or choking hazards present on playground equipment

Site 19. Open “S” Hooks

__Yes: Open “S” hooks on swings or other playground equipment

__No: No open “S” hooks on swings or other playground equipment

Site 20. Pinch Hazards

__Yes: Pinch hazards on playground equipment

__No: No pinch hazards on playground equipment

OTHER

Site 21. Refuse Disposal

__Wall or roof for outdoor enclosed area is leaning or collapsed

__Concrete slab deteriorated

__Collection area overflowing: Area is too small to store refuse until pickup

__Garbage cans are overflowing

__No exterior refuse disposal

__Refuse properly contained

Site 22. Retaining Walls

__Severe deterioration/safety risk: A retaining wall is damaged and does not function as it should or is a safety risk

__Some deterioration: A retaining wall shows some signs of deterioration, but it still functions as it should, and it is not a safety risk

__No deterioration

__No retaining walls present

Site 23. Standing Water

__Yes: Storm drainage areas (e.g., ditches) have standing water

__No: No standing water in storm drainage areas

NOTE: This does not include storm water detention basins, which are addressed in Site 24 and 25. 
Site 24. Storm Drainage

__Completely blocked: The system is completely blocked or a large segment of the system has failed because a large quantity of debris has caused: backups into adjacent area(s)

__Runoffs into areas where runoffs are not intended

__Partially blocked: The system is partially blocked by a large quantity of debris, causing backup into adjacent area(s)

__No designed storm drainage

__No obstructions

Site 25. Outdoor Water

__Yes: Pond/lake/stream __Yes: Drainage reservoir __No: No other water on site

Site 26. Outdoor Water: Drainage Reservoir Fencing

__No: Reservoir not fenced

__Yes: Reservoir fenced but fence needs repair 
__Yes: Reservoir fully fenced and fence intact 
__Not applicable: No drainage reservoir

Site 27. Septic Tank

__Moist ground in septic tank area:

IMMINENT HEALTH HAZARD: REPORT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__No evidence of excessive ground moisture

__No septic tank present

Site 28. Walkways/Steps/Hand Railing

__Missing or damaged or loose: A hand rail for four or more stairs is missing, damaged, loose, or other- wise unusable; hand rail only present on one side, visible faults in the pavement: longitudinal, lateral, alligator, etc.

___Pavement that sinks or rises because of the failure of sub-base materials. Five percent or more of the walkways must be impacted—50 out of 1,000 square feet, for example. Relief joints are there by design; do not consider them cracks. When observing traffic ability, consider the capacity to support pedestrians, wheelchairs, and people using walkers. Cracks greater than ¾ inch, hinging/tilting, or missing section(s) that affect traffic ability over more than 5% of the property’s walkways/steps.

__No damage

__No walkway/steps

Site 29. Large Trees

__Hanging over unit and touching unit 
__Well maintained: Trimmed back from unit 
__No large trees present

COMMENTS, SITE SECTION

BUILDING EXTERIOR (ITEMS 30–62)

Items to inspect in this category are

· Doors

· Fire Escapes

· Foundations

· Lighting

· Roofs

· Walls

· Windows

Exterior 30. Building Access for the Disabled

__Building is not accessible for the disabled

__Building is accessible for the disabled
__Not applicable (single-family unit with no disabled residents)

DOORS

Exterior 31. Damaged Frames/Threshold/Lintels/Trim

__At least one fire/emergency door not working or cannot be locked because of damage to the frame, threshold, lintel, or trim. This also includes the main front door.

__At least one door not working or functioning or cannot be locked because of damage to the frame, threshold, lintel, or trim

__No damage

NOTE: This does not include damage to door hardware, which is addressed in Exterior 32.
Exterior 32. Damaged Hardware/Locks

__One or more door’s panic hardware does not function as it should

__One entry door or fire/emergency door does not function as it should or cannot be locked because of damage to the door’s hardware

__One or more doors cannot be locked and does not function as it should or cannot be locked because of damage to the door’s hardware

__No damage

Exterior 33. Damaged Surface

__≥1 inch diameter: One door has a hole or holes larger than 1 inch in diameter, significant peeling/ cracking/no paint, rust that affects the integrity of the door surface, or broken/missing glass

__One entry door or fire/emergency door has a hole or holes with a diameter ranging from

¼ inch to 1 inch

__¼ inch–1 inch diameter: One door has a hole or holes with a diameter ranging from ¼ inch to 1 inch

__No damage

Exterior 34. Screen/Storm Doors Damaged/Missing

__Security door inoperable

__Missing screen or glass: At least one screen door or storm door is damaged or is missing screens or
__Missing door: There must be evidence that a screen/storm/security door existed

__No damage

Exterior 35. Deteriorated/Missing Caulking/Seals/Flashing

__Missing or damaged caulk, seals, or flashing: The seals/caulking is missing on one entry door, or they are so damaged that they do not function as they should

__No missing or damaged caulk, seals, or flashing

__Not designed to have seals, caulk, or flashing

Exterior 36. Missing Doors (In Exterior Doorway)

__Yes: One or more doors is missing

__No: All doors are present

FIRE ESCAPES

Exterior 37. Egress

__Fire escape blocked or otherwise not functioning

__No fire escape

__Fire escape functioning and not blocked

Exterior 38. Components

__Ladder, railing, stair missing (or not operational): Any of the functional components that affect the function of the fire escape (for example, one section of a ladder or a railing) are missing

__No fire escape

__No missing components

Exterior 39. Fire Escape/Emergency Egress

__No: No fire escape for basements with bedrooms and finished living spaces

__Yes: At least one fire escape for basements with bedrooms and finished living spaces

__Not applicable

FOUNDATIONS

Exterior 40. Foundation Type

__Slab   
__Crawl space 
__Basement
Exterior 41. Cracks/Gaps

__≥1/8 inches wide × 1/8 inches deep × 6 inches long: Cracks more than 1/8  inch wide by 1/8  inch deep by 6 inches long

__Large pieces—many brick for example—are separated or missing from the wall or floor

__Large cracks or gaps (a possible sign of a serious structural problem)

__Cracks run the full depth of the wall, providing opportunity for water penetration

__Sections of the wall or floor are broken apart

__<1/8 inches wide × 1/8 inches deep × 6 inches long: Cracks smaller than these dimensions

__No cracks/gaps: No signs of deterioration

Exterior 42. Spalling/Exposed Rebar

__≥50%: Obvious, significant spalled area(s) are affecting 50% or more of any foundation wall

__Spalling is exposing any reinforcing material (rebar or other material)

__10 to <50%: Obvious, large spalled area(s) are affecting 10%–50% of any foundation wall

__<10%

__Not applicable (no foundation)

LIGHTING

Exterior 43. Fixtures/Bulbs

__≥20% broken/inoperable

__The condition constitutes an obvious safety hazard: REPORT TO BUILDING MANAGEMENT/ OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__<20% broken/inoperable, but this does not constitute an obvious safety hazard

__No exterior lighting

__No broken/inoperable fixtures/bulbs

ROOFS

Exterior 44. Damaged/Clogged Drains (Roofs)

__Fully clogged: Drain is so damaged or clogged with debris so the drain no longer functions (shown by ponding)

__Partially clogged: Debris around or in a drain, but no evidence of ponding

__Drain is damaged or partially clogged with debris, but the drain system still functions and there is no evidence of ponding

__No clog

__No drain

NOTE: This does not include gutters and downspouts, which are addressed in Exterior 48. 
Exterior 45. Chimney Clearance

__All chimneys do not have good clearance above roof line: Chimney height is less than 3 feet above the highest point where the chimney penetrates the roofline
__All chimneys have good clearance above the roof line: Chimeny height is more than 3 feet above the point where the chimney penetrates the roofline

__No chimneys

Exterior 46. Damaged Soffits/Fascia/Flashing

__Missing or damaged with water intrusion: Soffits or fascia that should be there are missing or so damaged that water penetration is visibly possible

__Some cracks but no water intrusion: Damage to soffits or fascia, but no obvious opportunities for water penetration

__No damage

Exterior 47. Vents

__Missing or major damage: Vents are missing or so visibly damaged that further roof damage is possible.

__Some damage: The vents are visibly damaged, but do not present an obvious risk to promote further roof damage

__No damage

Exterior 48. Gutters/Downspouts

__Some components missing: Splashblocks or other components are missing. Drainage system components are missing, causing visible damage to the roof, structure, exterior wall surface, or interior.

__Some components damaged: Splashblocks or other components are damaged. Drainage system components are damaged, causing visible damage to the roof, structure, exterior wall surface, or interior.

__Both: Some components are missing and some are damaged

__No damaged or missing

Exterior 49. Gutters/Downspout Discharge

__Less than 2 feet from building foundation or grading causes water to pool near foundation

__Discharges directly to storm water system

__More than 2 feet from building and grading is sloped away from foundation

Exterior 50. Shingles or Tiles or Other Roofing Material

__≥100 square feet of shingle or tile damage

__One shingle to less than 100 square feet of shingle, tile, or roofing material damage

__One shingle or tile missing/damaged (<1 square foot)

__No missing or damaged shingles, tiles, or roofing material

WALLS

Exterior 51. Primary Exterior Wall Surfaces

__Brick
__Stucco 

__Wood 

__Stone

__Cement/concrete block

__Asbestos 

__Vinyl

__Other:_____
Exterior 52. Secondary Exterior Wall Surfaces

__Brick
__Stucco 
__Wood 
__Stone

__Cement/concrete block

__Asbestos 
__Vinyl

__Other:_____
__Not applicable

Exterior 53. Wall Cracks and Gaps

__≥1/8 inches wide × 1/8 inches deep × 6 inches long: Crack(s) more than 1/8 inch wide by 1/8  inch deep by 6 inches long

__evidence of moisture intrusion

__Pieces—many bricks, for example—are separated from the wall
__Crack(s) run the full depth of the wall, providing opportunity for water penetration

__Sections of the wall are broken apart

__<1/8 inches wide × 1/8 inches deep × 6 inches long: Crack(s) less than 1/8  inch wide by 1/8  inch deep by 6 inches long and no evidence of moisture intrusion

__No cracks/gaps

Exterior 54. Damaged Chimney

__Chimney separated from wall

__Holes >4 inches × 4 inches: The surface of the chimney shows surface damage on more than one piece-a few bricks or a section of siding, for example

__The surface of the chimney has holes that affect an area larger than 4 inches by 4 inches

__Both holes and separation

__Holes observed, total area of opening <4 inches × 4 inches

__No damage or no chimney required

Exterior 55. Wall Surface Deterioration

__≥8½ × 11 inches: A missing piece or a hole larger than ½ inch in diameter

__Deterioration affects an area larger than 8½ inches by 11 inches

__Deterioration exposes any reinforcing material (rebar)

__There is a hole of any size that completely penetrates the exterior wall

__Wall surfaces out of plumb (≥1 inch in 20 feet)

__Wall surface out of horizontal alignment (≥1 inch in 20 feet)

__Up to 8½ inches × 11 inches: A missing piece or a hole smaller than ½ inch in diameter

__Deterioration affects an area up to 8½ inches by 11 inches

__No missing pieces/holes/spalling

Exterior 56. Masonry Caulking and/or Mortar

__≥12 inches missing/damaged: Mortar is missing around more than one contiguous masonry unit

__Deteriorated caulking in an area longer than 12 inches

__<12 inches missing/damaged: Mortar is missing around a single masonry unit

__Deteriorated caulk is confined to less than 12 inches

__No damage or no caulking/mortar required

Exterior 57. Wall/Soffit/Fascia Paint/Water Damage

__≥20 square feet: More than 20 square feet of building exterior walls affected

__<20 square feet, but some staining: Less than 20 square feet of building exterior walls affected

__No water stains/peeling or no paint required (e.g., brick walls)

WINDOWS

Exterior 58. Window Panes

__One or more missing or broken: A glass pane is missing

-OR-A glass pane is cracked or broken AND sharp edges are seen

__Both broken and missing: More than one window has broken and missing glass panes

__One or more cracked: A glass pane is cracked but no sharp edges are seen

__None broken, cracked, or missing

Exterior 59. Screens

__1 or more screens damaged: One or more screens in one building are punctured, torn, or otherwise damaged

__1 or more screens missing: Do not cite this if the window is not designed to have a screen

__Both damaged and missing: One or more screen damaged or missing

__No screens damaged or missing or no screen required

Exterior 60. Sills/Frames/Lintels/Trim

__Major damage, missing or exposed interior wall, not weathertight: Sills, frames, lintels, or trim are missing or damaged, exposing the inside of the surrounding walls and compromising its weather tightness

__Some damage, but no wall exposed, still weathertight: Damage to sills, frames, lintels, or trim, but nothing is missing. The inside of the surrounding wall is not exposed. No impact seen on either the functioning of the window or weather tightness.

__No damage

Exterior 61. Caulking/Seals/Glazing Compound

__Missing or deteriorated, leaks or damage present: There are missing or deteriorated caulk or seals--with evidence of leaks or damage to the window or surrounding structure

__Missing/deteriorated, but no leaks or damage: Most of the window shows missing or deteriorated caulk or glazing compound, but there is no evidence of damage to the window or surrounding structure or leaks

__Not missing or deteriorated

Exterior 62. Window Assembly or Trim Paint

__≥50%: Peeling paint or a window that needs paint on more than 50% of the painted surface

__10% to <50%: Peeling paint or a window that needs paint on between 10%–50% of the painted surface

__<10%, but some non-intact: Peeling paint or a window that needs paint on less than 10% of the painted surface

__All intact: All paint on exterior windows is intact or no paint is required (e.g., aluminum or vinyl windows)

COMMENTS, EXTERIOR SECTION

BUILDING SYSTEMS (ITEMS 63–81)

NOTE: This section is for multi-home sites only. If this is a single-family home, skip to Item 103.
Items to inspect in this category include

· Electrical systems
· Fire protection

· HVAC

Building Systems 63. Central Water Supply or Sewage System

__Water leaks observed: If leaking water is a safety concern (i.e., is leaking on or near electrical equipment), REPORT IT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__No water leaks observed

Building Systems 64. Outside Water Spigots

__Outside water spigots protected by hose bibb vacuum breakers

__Outside water spigots not protected by hose bibb vacuum breakers

Building Systems 65. Chimney and Exhaust Ventilation for Fuel-fired Equipment

__Improper exhaust venting: Any misalignment, blockage, rust, corrosion, or other deficiency that may cause improper or dangerous venting of exhaust gases

__There is no pressure relief valve

__Proper exhaust venting: flue ok

__No chimney exhaust ventilation system required (e.g., electric water heater)

Building Systems 66. Makeup Air

__Makeup air not provided to the fireplace, gas water heater, or other fuel-burning fixtures

__Makeup air provided to the fireplace, gas water heater, or other fuel-burning fixtures

ELECTRICAL SYSTEMS

Building Systems 67. Breakers/Fuses

__Damaged breakers or fuses, frayed wiring, arcing scars: Carbon residue, melted breakers, or arcing scars

__Missing breakers/open panels/missing covers: Missing breakers or open panels (breaker port or receptacle or panel cover)

__Improper fusing: Fuse receptacles with improper fuses or bypassed

__Access blocked; could not inspect: The electrical system could not be visually accessed because of blockage or inaccessibility

__No deficiency observed

Building Systems 68. Water Leaks or Corrosion On or Near Electrical Systems

__Evidence of water leaks/corrosion: Any corrosion that affects the condition of the components that carry current

__Any stains or rust on the interior of electrical enclosures

__Any evidence of water leaks in the enclosure or hardware

__No evidence of water leaks/corrosion

Building Systems 69. Wiring

__Deteriorated insulation exposing conducting wire: Nicks, abrasions, or fraying of the insulation exposing any conducting wire. Do not check this for a bare grounding wire.

__No deteriorated insulation

Building Systems 70. Extension Cord Use

__Extension cords not used properly: Extension cords under carpets or across doorways

__Too many appliances plugged into one extension cord

__Extension cords used properly: Extension cords not draped across doorways or under carpets and not overloaded with too many appliances

__No extension cord use

Building Systems 71. Extension Cord Condition

__Not good: Extension cords cracked or frayed

__Good: Extension cords are not cracked or frayed

__No extension cord use

Building Systems 72. Electrical Covers

__One or more missing covers: A cover is missing, which results in exposed visible electrical connections

__Covers not missing

FIRE PROTECTION

Building Systems 73. Fire Sprinklers

__Sprinkler disabled, missing, blocked, or painted over: Any sprinkler head is missing, visibly disabled, painted over, blocked, capped or otherwise disabled. REPORT TO BUILDING MANAGEMENT/ OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION.

__No sprinkler system

__Sprinkler not disabled/missing/blocked

Building Systems 74. Missing, Damaged, Expired, or Wrong Kind of Fire Extinguishers/Fire Hoses

__≥10%, or none in building: More than 10% of the fire extinguishers are missing, damaged, or expired.

IF THERE ARE NO FIRE EXTINGUISHERS, REPORT TO BUILDING MANAGEMENT/ OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION.

__There is not an operable/non-expired fire extinguisher on each floor

__The building does not have a fire extinguisher

__≥5% to <10%:  5%–10% of the fire extinguishers are missing, damaged, expired, or wrong kind

__<1% to <5%:  <1% to <5% of extinguishers missing/damaged/expired or wrong kind

__None missing/damaged/expired

Building Systems 75. Emergency Exit/Egress Routes

__All exits not clear of furniture, toys, and clutter

__All exits clear of furniture, toys, and clutter

HVAC

Building Systems 76. Boiler/Pump

__Water or steam leaks: Water or steam leaking in piping or pump packing or boiler

__No leaks

__Does not apply

Building Systems 77. Fuel Supply

__Leaks observed or odor of natural gas, propane, or oil detected: Any amount of fuel is leaking from the supply tank or piping. REPORT LEAKS TO BUILDING MANAGEMENT/OWNER IMMEDIATE AND RECORD SPECIFICS IN THE COMMENTS SECTION. THE ODOR OF NATURAL GAS OR PROPANE IS AN IMMINENT HEALTH HAZARD; THE STRUCTURE SHOULD BE EVACUATED.

__No leaks observed or odor detected

__Does not apply

Building Systems 78. Chimney Exhaust

__Reversed air flow in chimney observed. REPORT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION.

__Misaligned, damaged, blocked, or disconnected: Misalignment of an exhaust system on a gas-fired or oil-fired unit that causes improper or dangerous venting of gases

__Evidence of blockage or disconnection

__Evidence of rust and corrosion that could cause improper flue pipe and chimney function

__Not misaligned, damaged, blocked, or disconnected

__No chimney exhaust ventilation required

Building Systems 79. Chimney Spark Arrestor and Rain Cap

__No chimney spark arrestor or rain cap 
__Chimney spark arrestor and rain cap installed 
__No chimney

Building Systems 80. HVAC Condensate and Sewage Corrosion

__Rust or corrosion prevents functioning: Significant formations of metal oxides, significant flaking, discoloration, or the development of a noticeable pit or crevice

__Equipment or piping does not function because of this condition

__A drain is clogged or components of the sanitary system are leaking.

__Evidence of standing water, puddles, or ponding (a sign of leaks or clogged drains)

__Some rust or corrosion or other damage, but system functioning

__No rust/corrosion

__Not applicable: No ducts or pipes

Building Systems 81. HVAC Air Supply

__From basement only

__Supply (return) air entirely from living area
__Supply (return) air includes fresh (outdoor) air 
__No forced air system present

COMMENTS, BUILDING SYSTEMS SECTION

COMMON AREAS (ITEMS 82–102)

NOTE: This section is for multi-home sites only. If this is a single-family home, skip to Item 103.
Items to inspect in this category include

· Walkways/steps
· Ceilings

· Floors

ELEVATORS

Common Areas 82. Elevators

__Elevators and elevator equipment do not work properly

__Elevators and elevator equipment work properly

__No elevators

SIGNAGE

Common Areas 83. Exit Signage

__Exit signs missing or broken or not visible

__Exit signs present and functioning

SMOKING AREA
Common Areas 84. Designated Smoking Area

__Area littered with butts/food debris 
__No butts/food debris  observed 
__No designated smoking area

INTERIOR TRASH

Common Areas 85. Trash Collection Areas

__Trash on floor: Extensive trash and/or garbage on the floor

__Trash containers/chutes missing covers: Missing or damaged covers to trash chutes or trash or garbage containers

__Both: Both trash on floor and missing or damaged covers
__No trash on floor or missing covers

OUTLETS, SWITCHES, COVER PLATES

Common Areas 86. Electrical Outlets

__Exposed wiring

__Missing cover plates

__Both: #PUI FYQPTFE XJSJOH BOE NJTTJOH DPWFS QMBUFT

__No exposed wiring or missing cover plates

SMOKE AND CARBON MONOXIDE DETECTORS
Common Areas 87. Smoke Detectors

__Not operational: One smoke detector tested per inspected common area; detector does not work as designed

__No smoke detector: No smoke detectors in common area

__Operational: One smoke detector tested per inspected common area; detector works as designed

Common Areas 88. Carbon Monoxide Detectors

__Not operational: One CO detector tested per inspected common area; detector does not work as designed

__No carbon monoxide detector: No CO detectors in common area

__Operational: One CO detector tested per inspected common area; detector works as designed

WALKWAYS/STEPS
Common Areas 89. Walkways/Steps

__Missing/damaged/loose: Walkways and steps have missing surfaces or are otherwise damaged

__A missing or loose handrailing

__No damage

__No walkway/steps

CEILING

Common Areas 90. Ceiling Buckling

__Bulging or buckling: Bulging, buckling, sagging, or a lack of horizontal alignment
__No bulging/buckling

Common Areas 91. Ceiling Holes

__Large holes: Total area larger than 8½ inches × 11 inches

__A hole penetrates the area above

__More than three tiles or panels are missing

__Small holes: Total area not larger than 8½ inches × 11 inches

__No hole penetrates the area above

__No more than three tiles or panels are missing

__No holes observed

Common Areas 92. Peeling/Needs Paint

__≥2 square feet: More than 2 square feet of peeling or deteriorated paint in one or more common areas 
__<2 square feet: Less than 2 square feet of peeling or deteriorated paint in one or more common areas 
__All intact: All paint intact

Common Areas 93. Water Stains/Water Damage

__≥2 square feet: One or more ceilings(s) has evidence of a leak, water damage, or water staining (such as a darkened area) over a large area (more than 4 square feet)
__<2 square feet: One or more ceiling(s) has evidence of a leak, water damage, or water staining (such as a darkened area) over a small area (less than 4 square feet)
__No water stains/water damage

Common Areas 94. Mold

__≥4 square feet mold observed or musty odor detected: On one or more ceilings(s), mold is seen in a large area (more than 4 square feet) or there is a musty odor

__<4 square feet visible mold: On one or more ceiling(s), mold is seen in a small area (less than 4 square feet)

__No mold or musty odor

Common Areas 95. Mold Source

__Leaking roof

__Leaking appliance

__Leaking water pipe in wall or ceiling

__Poor ventilation

__Do not know

FLOORS

Common Areas 96. Floor Buckling

__Yes: Bulging, buckling, sagging, or alignment problem
__No: No bulging, buckling, sagging, or alignment problem
Common Areas 97. Floor Covering

__≥50% damaged: For one or more floor(s), more than 50% of the floor covering is damaged

__Damage to the floor covering exposes the underlying material

__10%–<50% damaged: An estimated 10%–50% of the floor covering has stains, surface burns, shallow cuts, small holes, tears, loose areas, exposed seams, or other defect. The covering is fully functional, and there is no safety hazard.

__<10% damaged: Less than 10% of the floor covering has stains, surface burns, shallow cuts, small holes, tears, loose areas, exposed seams, or other defect. The covering is fully functional, and there is no safety hazard.

__No damage observed on any of the floors

Common Areas 98. Flooring/Tiles

__≥50% missing or damaged: More than 50% of the flooring is affected by small holes and damage.

-OR-The condition causes a safety problem

__10%–<50% missing or damaged: An estimated 10%–50% of the flooring has small holes in areas of the floor surface, but there are no safety problems

__<10% missing or damaged: For a single floor, there are small holes in areas of the floor surface. Less than 10% of the floor is affected and there are no safety problems.

__No damaged or missing flooring

Common Areas 99. Peeling or Deteriorated Paint

__≥2 square feet: Peeling or deteriorated paint in an area larger than 2 square feet in any one room or common area

__<2 square feet: Peeling or deteriorated paint in an area smaller than 2 square feet in any one room or common area

__No peeling or deteriorated paint

Common Areas 100. Subfloor

__≥4 square feet rotting or deteriorated: -BSHF BSFBT PG SPU (NPSF UIBO 4 TRVBSF GFFU) TFFO

__Applying weight to the floor causes noticeable deflection

__<4 square feet rotting or deteriorated

__Subfloor cannot be observed

Common Areas 101. Waters Stains/Water Damage

__≥4 square feet: A large portion of one or more floors (more than 4 square feet) has been substantially saturated or damaged by water, mold, or mildew. Cracks, mold, and flaking are see; the floor surface may have failed.

__<4 square feet: Evidence of a water stain (such as a darkened area) over a small area of floor (less than 4 square feet). Water may or may not been seen. Less than 10% of the floors are affected. 
__No water stains/water damage

Common Areas 102. Mold

__≥4 square feet mold observed or musty odor detected: On one or more floor(s) there is evidence of mold over a large area (more than 4 square feet)

__A musty odor is detected

__<4 square feet visible mold: On one or more floor(s) there is evidence of mold over a small area (less than 4 square feet)
__No visible mold present

COMMENTS, COMMON AREAS SECTION

HOUSING UNIT (ITEMS 103–196)

Items to inspect in this area are

· Bathroom
· Ceiling, floors, and walls

· Doors

· Electrical

· Water heater

· HVAC system

· Kitchen 

· Laundry area

· Lighting

· Patio/porch/deck/balcony

· Smoke and carbon monoxide detectors

· Stairs

· Windows
BATHROOM

Housing Unit 103. Bathroom Cabinets

__Damaged: Shelves, vanity tops, or drawers damaged or doors not functioning as they should

__Missing: Shelves, vanity tops, drawers, or doors missing

__Both: Both damaged and missing elements seen
__No damage/missing cabinets

Housing Unit 104. Lavatory Sink

__≥50% discoloration or cracks: The sink cannot be used because of extensive discoloration or cracks

__The sink or associated hardware is missing or has failed

__<50% discoloration or cracks: The sink can be used, but there are either cracks or extensive discoloration affecting less than 50% of the basin

__A stopper is missing

__No cracks/discoloration

Housing Unit 105. Plumbing Drain

__Drain completely clogged: Fixtures are not usable because the drain is completely clogged or shows extensive deterioration

__Slow drain: Water does not drain freely, but the fixtures can be used

__Drain working properly

Housing Unit 106. Plumbing Faucets/Fixtures

__Large water leak: There is a steady leak adversely affecting the area around it

__The faucet or pipe cannot be used

__Small water leak: There is a leak or drip contained by the basin

__No leaks observed

Housing Unit 107. Water Temperature

__Only hot water present, but not hotter than 120°F

__Only cold water present

__Hot and cold water present

Housing Unit 108. Water Pressure

__Inadequate at any bathroom plumbing fixtures

__Adequate at all bathroom plumbing fixtures

Housing Unit 109. Shower/Tub Surface

__≥50% or surface area damaged, inoperable or missing: The shower or tub can be used but there are cracks or extensive discoloration in more than 50% of the basin surface area

__The shower or tub cannot be used for any reason

__The shower, tub, faucets, drains, or associated hardware are missing or have failed

__<50% of surface area damaged: The shower or tub can be used but there are cracks or extensive discoloration in less than 50% of the surface area of the basin or stall

__No damage

Housing Unit 110. Shower/Tub Grab Bars

__Grab bars not installed

__Grab bars improperly installed

__Grab bars are properly installed inside and outside of tub
Housing Unit 111. Bathroom Exhaust

__Exhaust fan not working

__No exhaust fan or window present

__Exhaust fan working

Housing Unit 112. Toilet

__Toilet seat and/or bowl cracked or broken: Fixture elements (seat, flush handle, cover, etc.) are missing or damaged

__There is a hazardous condition: REPORT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__The bowl is fractured or broken and cannot retain water

__The water closet/toilet is missing

__The water closet/toilet cannot be flushed because of obstruction or another defect

__Toilet seat cracked or broken

__Not cracked or broken: A water closet/toilet is not damaged and functions properly

Housing Unit 113. Toilet Grab Bars

__Grab bars not installed

__Grab bars improperly installed

__Grab bars properly installed next to toilet

NOTE: Applies to households with elderly residents or a resident with a physical handicap.

Housing Unit 114. Shower/Bath/Toilet Caulking and/or Seals

__Deteriorated caulk/seals

__No deterioration observed

Housing Unit 115. Bathroom Call-for-Aid

__Damaged or not working

__Missing

__No call-for-aid unit

__No damage/working/not missing

NOTE: Applies to households with elderly residents or a resident with a physical handicap.

Housing Unit 116. Permanent Carpet on Bathroom Floor

__Permanent carpet: Does not include removable bath mats

__No permanent carpet: Bathroom floor is a hard, cleanable surface

CEILING, FLOORS, AND WALLS

Housing Unit 117. Bulging/Buckling

__Bulging, buckling, or alignment problem: Bulging, buckling, sagging, or alignment problem.
__No bulging, buckling, or alignment problem

Housing Unit 118. Holes

__Large holes ≥8½ inches × 11 inches: A hole is larger 8½ inches by 11 inches but it does not penetrate the area above or adjacent

__More than three tiles or panels are missing

__There is a crack more than 1/8-inch wide and 11 inches long

__A hole penetrates the area above or adjacent

__Medium-sized holes present: Holes less than 8½ inches × 11 inches in area

__No hole penetrates the area above or adjacent

__No more than three tiles or panels are missing

__Small holes present: Holes smaller than 8½ inches × ½ inches (do not count pinholes) in total hole area.

__No holes observed

Housing Unit 119. Peeling/Needs Paint

__≥2 square feet damage: Peeling or deteriorated paint in an area larger than 2 square feet in any one room

__<2 square feet damage: Peeling or deteriorated paint in an area smaller than 2 square feet in any one room

__No damage/peeling paint

Housing Unit 120. Water Stains/Water Damage

__≥4 square feet water stains/water damage: Any one ceiling, floor, or wall has evidence of water stains/water damage, a leak over a large area. Water may or may not be visible.

__<4 square feet water stains/water damage: Any one ceiling, floor, or wall has evidence of water stains/water damage, a leak over a small area. Water may or may not be visible.

__No water stains/water damage

Housing Unit 121. Condensation on Windows

__Condensation on windows, doors, walls

__No condensation on windows, doors, walls

Housing Unit 122. Mold

__≥4 square feet visible mold present or musty odor detected: Any one ceiling, floor, or wall has visible mold over a large area (4 square feet or more)

__A musty odor is detected

__<4 square feet visible mold present: Any one ceiling, floor, or wall has visible mold over a small area (less than 4 square feet)

__No mold observed or musty odor detected

Housing Unit 123. Mold Source

__Leaking roof

__Leaking appliance

__Leaking water pipe in wall or ceiling

__Poor ventilation

__Do not know

DOORS

Housing Unit 124. Door Surface

__≥1 inch: One door has a hole or holes equal to or larger than 1 inch in diameter in total surface area, significant peeling/cracking/no paint, rust that affects the integrity of the door surface, or broken/missing glass

__¼ inch to 1 inch diameter: One interior door—not a bathroom or entry door—has a hole or holes or peeling cracking no paint, or rust with a diameter ranging from ¼ inch to 1 inch in total surface area

__No damaged surface observed

Housing Unit 125. Frame/Threshold/Lintel/Trim

__Bathroom or entry door not working (closing, opening and/or latching): At least one bathroom door or entry door is not functioning or cannot be locked because of damage to the frame, threshold, lintel, or trim or door hardware

__At least one interior door not working (closing, opening and/or latching): At least one door is not functioning or cannot be locked because of damage to the frame, threshold, lintel, or trim or hardware

__Both: Both bathroom or entry door and other interior door not working
__No damage observed: All doors functioning

Housing Unit 126. Seals (Entry Only)

__Entry door seals deteriorated/missing: The seals are missing on one or more entry door(s), or they are so damaged that they do not function as they should

__No damage observed

Housing Unit 127. Door Missing

__Bathroom door missing

__One or more missing (not bathroom or entry): A door is missing, but it is not a bathroom door or entry door

__Entry door missing

__None missing

Housing Unit 128. Deadbolt Locks

__Deadbolt locks cannot be unlocked from the inside without a key

__No deadbolt locks

__Deadbolt locks can be unlocked from the inside without a key

Housing Unit 129. Door Lock Operation

__Door locks cannot be operated by a child in an emergency

__No door locks

__Door locks can be operated by a child in an emergency

ELECTRICAL

Housing Unit 130. Electrical Panel Access

__Yes: One or more fixed items or items of sufficient size and weight can impede access to the unit’s electrical panel during an emergency

__No: Access is not impeded

Housing Unit 131. Breakers/Fuses

__Damaged breakers or fuses, frayed wiring, arcing scars: Carbon residue, melted breakers, or arcing scars

__Missing breakers/open panels/missing covers: Missing breakers or open panels (breaker port or receptacle or panel cover)

__Improper fusing: Fuse receptacles with improper or bypassed fuses

__Access blocked; could not inspect: Electrical system could not be visually accessed due to blockage or inaccessibility

__No deficiency observed

Housing Unit 132. Water Leaks or Corrosion Near Electrical Systems

__Yes: Any leaks or corrosion

__Any stains or rust on the interior of electrical enclosures

__Any evidence of water leaks in the enclosure or any hardware deficiency (such as nicks, abrasions, or fraying of the insulation that expose wires that conduct current). NOTE: Do not consider this

a deficiency for wires that are not intended to be insulated, such as grounding wires.

__No: Leaks or corrosion not observed

Housing Unit 133. Wiring

__Deteriorated electrical insulation: Nicks, abrasions, or fraying of the insulation that exposes any conducting wire

__No deterioration

Housing Unit 134. Ground Fault Circuit Interrupters (GFCI)

__Inoperable or missing

__Operable

Housing Unit 135. Arc Fault Circuit Interrupters (AFCI)

__Inoperable or missing

__Operable

Housing Unit 136. Missing or Broken Electrical Covers

__Exposed wiring: An open breaker port or exposed wiring

__A cover is missing and electrical connections are exposed

__None missing/broken/exposed

Housing Unit 137. Child Tamper-resistant Outlet Covers

__No tamper-resistant outlet covers in units with young children
__Installed tamper-resistant outlet covers in units with young children 
__Not applicable (no young children in unit)

Housing Unit 138. Extension Cord Use

__Extension cords not used properly: Extension cords under carpets or across doorways

__Too many appliances plugged into one extension cord

__Extension cords used properly: Extension cords not draped across doorways or under carpets and not overloaded with too many appliances

__No extension cord use

Housing Unit 139. Extension Cord Condition
__Not good: Extension cords cracked or frayed

__Good: Extension cords not cracked or frayed
__No extension cord use

WATER HEATER

Housing Unit 140. Water Heater Exhaust

__Misaligned: Any misalignment that may cause improper or dangerous venting of gases.

__Not misaligned

__Does not apply: Electrical hot water or heater used instead of gas-fired or oil-fired unit

__No water heater inside unit.

Housing Unit 141. Water Temperature

__Temperature set at or above 120°F

__No hot water

__Temperature set below 120°F

Housing Unit 142. Leaks

__Water leak observed

__No water leak observed

Housing Unit 143. Water Heater Temperature/Pressure Relief Valve

__Absent 
__Present

Housing Unit 144. Water Heater Secured

__Not strapped down

__Strapped down

HVAC SYSTEM

Housing Unit 145. General Rust/Corrosion (HVAC)

__Significant rust/corrosion: Significant deterioration from rust and corrosion on HVAC units in the dwelling unit (includes ducts, radiators, baseboard heaters, etc.). The system does not provide sufficient heating or cooling.

__Surface rust/corrosion: Deterioration from rust and corrosion on HVAC units in the dwelling unit (includes ducts, radiators, baseboard heaters, etc.). The system still provides sufficient heating or cooling.

__No rust/corrosion in HVAC units in the dwelling unit (includes ducts, radiators, baseboard heaters, etc.)

Housing Unit 146. HVAC Operation

__Not working: HVAC system does not function; it does not provide the heating or cooling it should. The system does not respond when the controls are engaged.

__Working

Housing Unit 147. Supply Air for HVAC 

__Supply (return) air entirely from living area

__No forced air system present

__Supply (return) air includes fresh (outdoor) air

Housing Unit 148. HVAC Filters

__Need replacement

__Clean

__Not applicable

Housing Unit 149. HVAC Exhaust Ventilation System

__Reversed air flow in chimney observed: REPORT TO BUILDING MANAGEMENT/OWNER IMMEDIATELY AND RECORD SPECIFICS IN THE COMMENTS SECTION

__Misaligned, damaged, blocked, rusted, corroded, or disconnected

__Not misaligned, damaged, blocked, or disconnected

__No exhaust ventilation required (e.g., electric or no HVAC systems in unit)

Housing Unit 150. HVAC Noise

__Noisy/vibrating/leaking: HVAC system shows signs of abnormal vibrations, other noise, or leaks when engaged

__Not noisy

__Does not apply: No HVAC inside unit

Housing Unit 151. Space Heaters

__Space heaters used in unit are not at least 3 feet from anything that can burn

__Space heaters used in unit are at least 3 feet from anything that can burn
__Not applicable: No space heaters used in unit

Housing Unit 152. Fireplace Screen

__Fireplace does not have a sturdy screen to catch sparks

__Fireplace has a sturdy screen to catch sparks

__Not applicable: No fireplace in unit

Housing Unit 153. Fireplace Dampers

__Fireplace dampers not operational 
__Fireplace dampers operational 
__Not applicable: No fireplace in unit

Housing Unit 154. Wood Stove Barrier

__No barrier to keep children from getting too close to wood stove surfaces

__Barrier in place to keep children away from wood stove surfaces
__Not applicable: No wood stove in unit

KITCHEN
Housing Unit 155. Cabinets

__≥50% cabinets or cabinet doors missing: More than 50% of the cabinets or doors are missing   __<50% cabinets or cabinet doors missing: Less than 50% of the cabinets, doors, or shelves are missing __No doors missing

Housing Unit 156. Cabinet Damage

__≥20% damaged or laminate separation 
__<20% damaged or laminate separation 
__No damage or laminate separation

Housing Unit 157. Countertops

__≥20% missing/damaged: More than 20% of the countertop working surface is missing, deteriorated, or damaged below the laminate. Countertop is not a sanitary surface on which to prepare food.

__<20% missing/damaged: 20% or less of the countertop working surface is missing, deteriorated, or damaged below the laminate. Countertop is not a sanitary surface on which to prepare food.

__No missing/damaged countertops

Housing Unit 158. Dishwasher

__Not working: The dishwasher does not function as it should

__Working

__No dishwasher

Housing Unit 159. Garbage Disposal

__Not working: The garbage disposal does not function as it should

__Working

__No garbage disposal

Housing Unit 160. Kitchen Drain

__Kitchen drain completely clogged: Drain completely clogged or extensively deteriorated

__Slow kitchen drain: Basin does not drain freely
__Kitchen drain working properly

Housing Unit 161. Kitchen Plumbing

__Steady leak/adverse effect: A steady leak is having an adverse affect on the surrounding area

__The kitchen faucet or pipe is not usable

__Leak contained by kitchen sink: A leak or drip is contained by the basin or pipes and the faucet is functioning properly

__No leak

Housing Unit 162. Electrical

__No GFCI near kitchen sink

__GFCI does not work properly

__GFCI is present and is functions when tested

Housing Unit 163. Water Temperature

__Only hot water present at kitchen plumbing fixtures 
__Only cold water present at kitchen plumbing fixtures 
__Hot and cold water present at kitchen plumbing fixtures

Housing Unit 164. Water Pressure

__Inadequate water pressure at kitchen plumbing fixtures

__Adequate water pressure at all kitchen plumbing fixtures

Housing Unit 165. Range Hood

__Not working: Range hood does not turn on

__Partial blockage: An accumulation of dirt threatens the free passage of air

__Flue completely blocked

__No range hood/exhaust fan

__No blockage/functional: Range hood works properly

Housing Unit 166. Range or Stove

__Stove and/or oven missing

__Two or more burners not working
__One burner not working

__Stove and oven working

Housing Unit 167. Refrigerator

__Refrigerator missing or inoperable

__Seals deteriorated: Refrigerator has an excessive accumulation of ice

__Seals around refrigerator doors are deteriorated

__Refrigerator does not cool adequately for the safe food storage (temperature above 40℉)

__Refrigerator functioning properly (temperature 40℉ or below)
Housing Unit 168. Kitchen Sink

__≥50% discoloration, chips, or cracks or inoperable: Sink cannot be used because of extensive discoloration, chips, or cracks

-OR-Sink cannot be used because the sink or associated hardware is missing or has failed

__<50% discoloration, chips, or cracks: Sink can be used but cracks, chips, or extensive discoloration are seen in less than 50% of the basin

__A stopper is missing

__No cracks/discoloration/chips; sink operable

Housing Unit 169. Permanent Carpet on Kitchen Floor

__Permanent carpet on kitchen floor (does not include removable mats)

__Kitchen floor is a hard, cleanable surface
Housing Unit 170. Cleaning Products

__Cleaning products not stored out of the reach of children 
__Cleaning products stored out of the reach of children 
__No cleaning products stored in kitchen area

LAUNDRY AREA

Housing Unit 171. Clothes Dryer

__Vent missing: Dryer vent to outside is missing

__Vent damaged: Dryer exhaust is not effectively vented to the outside because of blockage or inadequate design or is vented into the interior

__Vent not missing or damaged: Exhaust vent is functioning properly

__No dryer

Housing Unit 172. Exhaust Duct from Dryer

__Flexible plastic: Dryer exhaust duct is made of flexible plastic 
__Flexible metal: Dryer exhaust duct is made of flexible metal 
__Other: Wood or other combustible material

__Rigid metal: Dryer exhaust duct is made of rigid metal

Housing Unit 173. Dryer Venting

__Dryer vents to basement 
__Dryer vents to attic 
__Dryer vents to crawl space

__Other:  


__Dryer vents to outside

LIGHTING

Housing Unit 174. Interior Housing Unit Lighting

__One or more lights missing: In one or more rooms in a unit, a permanent lighting fixture is missing, and no other switched light source is functioning in the room

__One or more lights not working: In one or more rooms in a unit, a permanent lighting fixture is not working, and no other switched light source is functioning in the room

__All lights working/none missing

Housing Unit 175. Outlets/Switches

__Broken, wires exposed: Broken cover plates with wires exposed
__Outlets or switches missing
__Broken, but no exposed wires 
__No broken cover plates

PATIO/PORCH/DECK/BALCONY
Housing Unit 176. Railings

__Missing: The baluster or side rails are missing

__Loose or damaged: The baluster or side rails enclosing this area are loose or damaged

__No damage

Housing Unit 177. Electrical Outlets

__No GFCIs present

__GFCIs not functional

__GFCIs present and functional
__No exterior outlets

Housing Unit 178. Spindles and Railings

__Missing: Spindles or railings missing on porch, deck, or balcony 
__Present: Spindles and railings present on porch, deck, or balcony 
__Not applicable: No porch, deck, or balcony

Housing Unit 179. Spindles and Railings: Condition

__Damaged 
__Loose 
__Too low

__Too far apart

__Missing

__Good condition and properly spaced

__Not applicable: No porch, deck, or balcony

Housing Unit 180. Spindles

__Spindles more than 4 inches apart

__Spindles not more than 4 inches apart
__Not applicable: No porch, deck, or balcony

Housing Unit 181. Railing Height

__Railing is not between 30 and 42 inches in height

__Railing is between 30 and 42 inches in height
__Not applicable: No porch, deck, or balcony

Housing Unit 182. Patio Surface

__≥¾ inch displacement 
__≤¾ inch displacement 
__Uneven steps

__No cracks/level

SMOKE AND CARBON MONOXIDE DETECTORS
Housing Unit 183. Smoke Detectors

__Not operational: At least one smoke detector tested in each unit; detector does not work as designed

__No smoke detector present: No smoke detector in unit

__Operational: One smoke detector tested in each unit (if feasible); detector works as designed

Housing Unit 184. Smoke Detector Location

__No smoke detectors in unit

__Smoke detectors in home, but not on every level, outside each bedroom, and in a common living area

__Smoke detectors on every level of the home, outside each bedroom, and in a common living area

Housing Unit 185. Smoke Detector Power

__No smoke detectors in unit

__Smoke detector powered by main electrical supply without battery backup

__Smoke detector powered by battery

__Smoke detector powered by main electrical supply with battery backup

Housing Unit 186. CO Detectors

__Not operational: At least one CO detector tested in each unit; detector does not work as designed

__No CO detector present: No CO detector in unit

__Operational: All CO detectors tested in each unit (if feasible); all detector(s) work as designed

Housing Unit 187. CO Detector Location

__No CO detectors in unit

__CO detector in dwelling unit but not near bedroom area

__CO detector near bedroom area

Housing Unit 188. Fire Extinguisher

__No fire extinguisher present

__Fire extinguisher present in home

__Fire extinguisher present in home and charged

STAIRS

Housing Unit 189. Stair Railings

__Missing: Handrail missing

__Broken, insecure, or missing: Handrail damaged, loose, or otherwise unusable or insecure

__Handrail present on both sides and not broken, missing, or insecure

__Does not apply: No stairs or three or fewer stairs

Housing Unit 190. Steps: Condition

__One or more broken or missing: One or more steps are broken or missing

__Not broken or missing: No broken or missing steps

__Does not apply: No steps

Housing Unit 191. Steps: Covering

__No covering on stairs

__Covering on stairs is not firmly attached or is poor condition

__Covering on stairs (e.g., nonslip tread covers) is firmly attached and in good condition

WINDOWS

Housing Unit 192. Windows

__One or more windows missing

__One or more windows cracked or broken 
__One or more windows cannot be opened 
__All windows intact and can be opened

Housing Unit 193. Window Sills

__Missing or damaged: A sill is missing or damaged, but the inside of the surrounding wall is not exposed and is still weathertight

__Not weathertight: A sill is missing or damaged enough to expose the inside of the surrounding wall and compromise its weather tightness

__Not missing or damaged

Housing Unit 194. Window Locks

__Not functioning and cannot be secured/locked __Not functioning but can be secured/locked __Functioning and lockable

Housing Unit 195. Window Caulking/Seals

__Missing/deteriorated (leaks present): There is missing or deteriorated caulk or seals and evidence of leaks or damage to the window or surrounding structure

__Missing/deteriorated (no leaks): There is missing or deteriorated caulk on widows, but there is no evidence of damage to the window or surrounding structure

__Not missing/deteriorated

Housing Unit 196. Window Paint

__Deteriorating paint: Deteriorating paint or a window that needs paint on 10% or more of its surface

__No deteriorating paint: All paint intact or deteriorating paint on less than 10% of the surface

COMMENTS, HOUSING UNIT SECTION

OTHER ITEMS (ITEMS 197–229)

Issues to assess in this area include

· Garbage and debris
· Injury hazards

· Childproofing measures

· Poisoning hazards

· Pest hazards

· Moisture hazards

· Swimming pool, spa, or whirlpool

· Other hazards

GARBAGE AND DEBRIS

Other 197. Indoors

__Garbage and debris not properly stored: Missing, uncovered, or leaking container

__Garbage and debris properly stored

Other 198. Outdoors

__Garbage and debris not properly stored: Missing, uncovered, or leaking container

__Garbage and debris properly stored
INJURY HAZARDS

Other 199. Sharp Edges

__Yes: Physical hazard present that could produce a skin cut or injury

__No: Sharp edges not present
Other 200. Trip Hazards

__Yes: Tripping hazards present

__No: Tripping hazards not present

Other 201. Garage Door Opener

__Garage door does not reverse properly

__Garage door opener reverses properly
__No garage door or no garage

CHILDPROOFING MEASURES
Other 202. Window Cords (Strangulation Hazard)

__Yes: Window cords looped or tied together

__No: Window cords not looped or tied together


Other 203. Window Guards

__Missing or not operational

__Present and operational

Other 204. Cabinet Locks

__Childproof cabinet locks missing or not operational

__Childproof cabinet locks in place and operational

Other 205. Water Safety

__Toilets not covered (toilet lids open)

__Toilets covered (toilet lids closed)

Other 206. Chemicals, Pesticides, Cleaning Supplies, or Medications Stored Within Easy Reach of Children

__Yes

__No

Other 207. Hobbies

__Evidence of household hobbies that could pose a risk to young children

__No evidence of household hobbies that could pose a risk to young children

POISONING HAZARDS

Other 208. Unvented Combustion Appliances

__Yes: Unvented combustion appliances (e.g., fuel-fired space heaters, gas clothes dryers, gas logs, charcoal, stoves etc.) present

__No: Unvented combustion appliances (e.g., fuel-fired space heaters, gas clothes dryers, gas logs, charcoal, stoves etc.) not present

Other 209. Attached Garage

__Attached garage not sealed from living area

__Do not know: Not sure whether attached garage sealed from living area

__Attached garage sealed from living area

__Not applicable: No attached garage

PEST HAZARDS

Other 210. Infestation: Roaches

__Frass or shells

__One or more live roaches

__No roaches or roach evidence

Other 211. Infestation: Rats or Mice

__Droppings or chewed holes

__One or more rats/mice

__No rats/mice/droppings/holes

Other 212. Other Insects or Vermin

__Yes: Other insects or vermin seen

__No: Other insects or vermin not seen

Other 213. Termite Tunnels

__Yes: Termite tunnels

__No: No termite tunnels

MOISTURE HAZARDS

Other 214. Sources of Excessive Humidity Present

__Yes: Sources of humidity (e.g., humidifier, dryer vented inside, uncovered fish tank) present

__No: Sources of humidity (e.g., humidifier, dryer vented inside, uncovered fish tank) not present

Other 215. Moldy or Musty Odor Present

__Yes

__No

Other 216. Dehumidifier Present

__No 
__Yes

SWIMMING POOL, SPA, OR WHIRLPOOL
Other 217. Fencing and Gates

__Missing or broken fencing or gate

__Damaged fencing or gate open

__Pool surrounded by undamaged three-sided fencing (house acts as fourth side)

__Pool surrounded by undamaged four-sided fencing

Other 218. Doors and Gates

__All doors and gates to swimming pool and spa areas do not close and latch automatically

__All doors and gates to swimming pool and spa areas close and latch automatically

Other 219. Latches

__Latching devices are not at least 48 inches from the ground

__Latching devices are not self-closing, self-latching

__Self-closing, self-latching devices are at least 48 inches from the ground
Other 220. Safety Equipment (Swimming Pool)

__None 
__Life ring

__Shepherd’s hook

__Both life ring and shepard’s hook
Other 221. GFCI

__No GFCI in area

__GFCI present but not working

__GFCI present and working in area
Other 222. Drain Cover

__Drain cover missing or broken: SHUT DOWN POOL, SPA, OR WHIRLPOOL IMMEDIATELY AND REPORT TO BUILDING MANAGEMENT/OWNER. RECORD SPECIFICS IN THE COMMENTS SECTION.

__Drain cover in place and not broken

Other 223. Safety Cover

__No safety cover on spa 
__Unlocked safety cover on spa 
__Locked safety cover on spa

OTHER HAZARDS

Other 224. Visible Dust on Surfaces

__Heavy 
__Slight

__No visible dust on surfaces

Other 225. Air Cleaning Device Present

__Yes 
__No

Other 226. Ozone Generator Present

__Yes 
__No

Other 227. Pets Present

__Yes

__No

Other 228. Tobacco Smoke or Odor Present

__Yes 
__No

Other 229. Other Hazards

__Yes

__No

COMMENTS, OTHER ITEMS SECTION

Page 1 of 87

