DV424 Operating Manual

DV424

Thank you for using our Mobile DVR. Please read this User’s Manual carefully to ensure that you can use the device correctly and safely.

The contents of this manual are subject to change without notice.

Specifications

| DV424 |  |  |
| --- | --- | --- |
| System | Operating system | Linux |
|  | Operating interface | Graphical menu operation interface(OSD) |
|  | Video permission | Administrator & user setting |
| Video | Video input | Max 8 x 1080P analog high definition |
|  | CVBS output | 2 CVBS output(1 * 6 PIN OUT, 1 * 4 PIN OUT) PAL/NSTC |
|  | HDMI output | 1CH type-A connector output, 1080P |
|  | Video display | 1 or 4 or 8CH |
|  | Video standard | PAL: 25FPS, NTSC: 30FPS |
|  | Compression | H.264 main profile |
| Audio | Audio input | 8 channels |
|  | Audio output | 1 channel |
|  | Record format | Synchronized video & audio recording |
|  | Audio compression | ADPCM |
| Digital processing & storage | Image resolution | Max 8 x 1080P(1920*1080) |
|  | Image quality | 1~8 level adjustable |
|  | Video bit rate | 512kbps~4Mbps/channel |
|  | Storage | 56~2700MB/(channel*hour) |
|  | Playback resolution | 8 x 720P or 4 x 1080P(encoding & decoding at the same time) |
|  | Audio bit rate | 32kbps |
|  | Storage | 2.5 inch SATA SSD/HDD x 1, max 2TB; SD card x 1, max 128GB |
| Alarm | Alarm input | 8 channels |
|  | Alarm output | 2 channels, 1 buzzer |
|  | Motion detection | High/low sensitivity adjustable |
| Interface for communication | IR | 1 channel |
|  | RS232 | 2 channel |
|  | RS485 | 2 channel |
|  | CAN | 1 channel |
|  | RJ45 | 1 channel |
|  | USB2.0/USB3.0 | USB3.0 x 1, USB2.0 x 1 |
| Wireless | 3G/4G | HSPDA/EVDO/FDD-LTE/TDD-LTE module optional |
|  | WIFI | optional |
|  | WIFI hotspot/AP | optional |
| GPS |  | Internal or external GPS/GLONASS module, coordinate/speed can be encoded in video stream and upload to server by wireless communication |
| G-Sensor /Gyroscope | Available | G-Sensor&Gyroscope |
| Software | Windows client | Available |
|  | iOS client | Available |
|  | Android client | Available |
|  | Web portal | Available |
| Electrical | Power supply & consumption | DC 12~32V, less than 15w(without camera) |
|  | Operating temp. & humidity | -20～70 degree/ <80% |
|  | Super Capacitor | Available |
|  | Clock | Built-in clock, Calendar |

Accessories

| Accessories | Description | Quantity | Accessories | Description | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 9 pin aviation power input wire |  | 1 | DB44 wire |
|  | 1 | DV424 key |  | 1 | Remote controller |
|  | 1, optional | GPS Antenna |  | 1, optional | HD293 |
|  | 1, optional | WIFI Antenna |  | 1, optional | HDMI wire |
|  | 1 | DB26 alarm input/output,RS232,RS485,Speed wire |  | 2, optional | 3G/4G Antenna |
|  | 1 | 10 pin alarm input/output, external GPS input wire |  | 1 | 5 pin can input/output, external temp.sensor input wire |
|  | 1 | 硬盘盒 |  | 1 | Panic Button |

| Rec. | Record | DB44 | 44PIN connector, A/V Input, Output.IO/ALARM Output |
| --- | --- | --- | --- |
| G-sensor | Accelerometer sensor | DB26 | 26PIN connector, Alarm Input/RS232/RS485 |
| GPS | Global Positioning System | ALM | Alarm |
| WIFI | WIreless-FIdelity | VLOSS | Video Loss |
| Cam | Camera | COMM | Communication |
| AVI | Audio Video Interleaved | ERR | Error |
| OSD | On-Screen Display | MEM | Memory |
| APN | Access Point Name | HD293 | High Definition Monitor MO293 |
| DHCP | Dynamic Host Configuration Protocol | MO113 | Monitor MO113 |
| SSID | Service Set Identifier | MMSHOW | Media Player |
| IP | Internet Protocol | MSV | Stonkam dedicated video format |
| MAC | Media Address Control | FTP | File Transfer Protocol |
| RSSI | Received Signal Strength Indication | DVR | Digital Video Recorder |
| SSD | Solid State Drive | IR | Infrared Radiation |
| HDD | Hard Disk Drive | SYS | system |
| SD | Secure Digital Memory Card | LED | Light Emitting Diode |
| USB | Universal Serial Bus |  |  |

Abbreviation & Description

- Main Features

Controlled by touch screen

- All settings and operations could be done through the monitor if connected with the suggested touch screen. The main function is specially needed in awkward occasions for using mouse, like in vehicles.

Video and Audio

- 8 channels * 1080p, 8 video inputs with audio

- 2 CVBS output(1 * 6 PIN OUT, 1 * 4 PIN OUT), 1 HDMI(1080P), total 3 video output with audio

Recording

- 8-CH Video & Audio Recorder (with image resolution up to 1920*1080), and with G-force data and GPS data

- Multiple recording modes: power on recording, normal recording, schedule recording, event recording (i.e., G-force recording, speed recording, motion detection recording, Alarm recording 1~8, Panic button recording). Support cyclic recording and 15 seconds pre-recording.

- Recording files saved by HDD, SSD or SD cards. Cyclic recording is optional.

- Real-time recording of license plate numbers, bus line numbers, driving speed, G-force/Gyroscope 6D accelerated speed, longitude and latitude, and GPS tracks.

Preview and Play Back

- Support single channel, or 8 channels audio and video play simultaneously

- Support searching recorded files by recording date, recording type

- Able to drag the progress bar when playing back

- Indicate recording status, alarm status.

Storage Types

- Support HDD, SSD, SD cards (SDHC, SDXC)

- The SD cards can be removed conveniently when they are not in recording or playing status.

Backup

- Support USB disk or USB hard disk for backup

Network

- Transfer through LAN / Cellular / WIFI. Picture is preferred rather then video to transmit to save cellular traffic.

- LAN / WIFI / Cellular connection (default priority: LAN > WIFI > Cellular); auto switch to LAN / WIFI connection when available to save Cellular traffic.

- File uploading function via FTP function, enables users to search or download in file list on the client.

Alarm

- 8 channels alarm inputs one channel buzzer output and 2 channels alarm outputs

- Over-speed alarm and accelerated-speed alarm

- Motion detection alarm

- G-force alarm

- Video loss alarm

- Panic button alarm

Charger

- 5V, 1.5A output from the USB interface to mobile devices, such as mobile phone.

Security

- User password protection. The DVR can only be accessed with correct password.

- Support account management.

- Wiring Diagram

- Connection - Front Panel

| ① Front Cover Buckle | ⑤ Hard Driver Box |
| --- | --- |
| ② Electronic lock | ⑥ SD Card Slot |
| ③ LED Indicators | ⑦ USB3.0 Interface |
| ④ IR Receiver | ⑧ SIM Card Slot |

  - Electronic Lock

Close the front cover and turn the groove by the key to the icon “off”, so as to prevent hard disk drive from moving out. Or turn to the icon “on” to open the front cover.

- Electronic Lock Function: DVR will stop recording and buzzer beeps when lock is open.

  - LED

PWR: Power indicator. It always lights up when the DVR is powered on.

RUN: Working status indicator. It will flash slowly when DVR is running.

ALM: Alarm indicator. It keeps on with the record when there is Alarm-in, G-force activation or motion detection or speed or panic button event alarm.

GPS: GPS indicator. It lights up when GPS is running.

CAM: Camera indicator. It keeps on when camera is working and off when not working.

SD: SD card indicator. It blinks when recording, and keeps on when no recording, and is off when SD card in abnormal status.

SSD: SSD/HDD indicator. It blinks when recording, and keeps on when no recording, and is off when SSD/HDD in abnormal status.

CELL: Cellular indicator. It always lights up when uploading data and off when no cellular module or cellular module in abnormal status.

WIFI: WIFI indicator. It always lights up when uploading data and off when no cellular module or WIFI module in abnormal status.

  - Remote Receiver

The IR Receiver is for the DVR to receive command from the remote control.

Remote control instructions:

  - SSD/HDD

- Hard disk Type: SSD/HDD,Max. Capacity 2T

- Size: 2.5 inches

- Installation of the disk

Step 1: Make sure the lock is OPEN, open the two ears of the front panel, and pull down the front plate.

Step 2: Pull the hard disk case from the DVR by the ring.

Step 3:  Take the 6 screws on the bottom side out of the disk case by screw driver.

Step 4:  Take down the bottom panel of the case and check the wiring of the hard disk.

Step 5: Take out the 2 screws on the movable supporting shelf.

Step 6: Revolve the shelf into the vertical direction.

Step 7: Insert the 2.5’’ HDD or SSD disk.

Step 8: Revolve the shelf back to the horizon direction and lock the disk with 4 screws in the accessory packet.

Step 9: Lock the shelf with the 2 screws.

Step 10: Check if the waterproof plastic on the bottom is missing or not, if not, place the bottom plate on the case and  lock it with the 6 screws.

Step 11: Insert the disk case back into the DVR.

Step 12: Pull back the front plate back and lock the 2 ears. And then revolve the key back to CLOSE position.

  - SD card slot

- SD card type: Each card Max. Capacity 128G

- Insert、remove SD card

Step 1: Use the key to unlock and open front plate

Step 2: Insert SD card to SD card slot

Step 3: Close the front plate and use the key to lock

  - USB Slot

USB 2.0/3.0, Max. Capacity 2T

  - SSD,HDD,CARD speed LIST(128G SD)

| Brand Name | Model Name | SIZE (Inch) | Cache (M) | Capacity | Interface | Hot Plug | Restart negotiation | Test Read Speed (MB/s) | Test Write Speed (MB/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Toshiba | MQ01ABF050 | 2.5 | 32MB | 500G | SATA3 | OK | OK | 83.3 | 74.9 |
| HGST | HTB-TT5SAE500 | 2.5 | 32MB | 500G | SATA3 | OK | OK | 71.6 | 67.2 |
| Seagate | ST2000LM015 | 2.5 | 128MB | 2T | SATA3 | OK | OK | 87.4 | 69.8 |
| Seagate | ST1000LM035 | 2.5 | 128MB | 1T | SATA3 | OK | OK | 92.0 | 57.9 |
| Seagate | ST500LT025 | 2.5 | 16MB | 500G | SATA3 | OK | OK | 88.7 | 52.3 |
| Seagate | ST500LM030 | 2.5 | 128MB | 500G | SATA3 | OK | OK | 85.6 | 64.2 |
| Seagate | ST500LM032 | 2.5 | 128MB | 500G | SATA3 | OK | OK | 87.6 | 60.5 |
| WD | WD10JUCT | 2.5 | 16MB | 1T | SATA3 | OK | OK | 90.3 | 46.3 |
| WD | WD5000LPVX | 2.5 | 32MB | 500G | SATA3 | OK | OK | 91.0 | 50.6 |
| WD | WD3200LPVX | 2.5 | 8MB | 320G | SATA3 | OK | OK | 87.6 | 51.0 |
| WD | WD5000LPLX | 2.5 | 32MB | 500G | SATA3 | OK | OK | 93.8 | 54.0 |
| WD | WD3200LPLX | 2.5 | 32MB | 320G | SATA3 | OK | OK | 90.8 | 56.3 |
| WD | WD2500LPLX | 2.5 | 32MB | 250G | SATA3 | OK | OK | 91.9 | 50.6 |

- Back Plate

| ① Power input (12-32V) | ⑨ Panic button, external GPS module connector |
| --- | --- |
| ② Camera cable input | ⑩ Ethernet(RJ45) |
| ③ HDMI output | ⑪ 6pin CVBS video monitor output |
| ④ Alarm input/output,RS232,RS485,Speed wire extend | ⑫ USB 2.0 input |
| ⑤ GPS Connector | ⑬ debug |
| ⑥ Cellular Connector, TX/RX | ⑭ HDMI cable fixing hole |
| ⑦ Wifi Connector | ⑮ Cellular Diversity Connector, Rx |
| ⑧ Can input/output, External temp.sensor input |  |

  - Power

- power input

9 PIN Avation female  connect to 9 PIN Avation male on the DVR424

- Connection

Connect ignition wire to yellow ACC wire of DVR, battery Positive to V+(Red wire), Negative to GND(black wire)

  - Camera

- See below 8 cameras diagram.

- How to connect cameras

Connect 8 cameras on below cable which connects to back plate of DVR

DB44P female on the DV424 connect to DB44P male

Connect No1 camera to AVIN1   4PIN Aviation male

Connect No2 camera to AVIN2  4PIN Aviation male

Connect No3 camera to AVIN3  4PIN Aviation male

Connect No4 camera to AVIN4  4PIN Aviation male

Connect No5 camera to AVIN5  4PIN Aviation male

Connect No6 camera to AVIN6  4PIN Aviation male

Connect No7 camera to AVIN7   4PIN Aviation male

Connect No8 camera to AVIN8  4PIN Aviation male

Connect monitor to AVOUT1  4PIN Aviation female

  - LCD monitor

- HD293 introduction

① HD293 power button, LED red for stand by mode, and green for working mode

② HDMI connector

③ HDMI cable

④ HD293 6pin aviation cable connector, with Power and Video/Audio pin.

| HD293 |  |
| --- | --- |
| Description | HD 10.1"Color monitor |
| Features | used for HD DVR |
| Resolution | 1024 x 600 (RGB) |
| Maximum Number of Cameras | 1 |
| Audio input | 1 |
| Audio output (loudspeaker) | 1W |
| HDMI input | 1 |
| VGA input | / |
| DVD/CVBS input | 1 |
| Trigger | No |
| Max. Brightness | 500 cd/m² |
| Contrast | 600：1 |
| Minimum Operating Temperature | -20ºC, RH 90% |
| Maximum Operating Temperature | +70ºC, RH 90% |
| Viewing Angle Monitor | U: 70/ D: 50, R/L: 70/70 |
| Mirror Function | No |
| Monitor Diameter (mm) | 267mm (W) × 159.5mm (H) × 30mm (T). |
| Split Screen | No |
| Volts | 10-32V |
| Consumption | less than 8W(without cameras) |

- Connect to high-definition LCD monitor (HD293)

* By HDMI cable

* By 6pin aviation cable

Each of the method can display and touch control the DVR.

- Connect to standard LCD monitor (CVBS)

* By 4pin AVOUT on the DB44 camera cable.

There is no touch control in this way. You can use remote control to operate the menu.

  - NO LCD monitor

若是DVR不接任何显示器，DVR会通过蜂鸣器的声音来提醒客户异常情况。

If the DVR is not connected to any monitors,  it will remind the customer of the exception by  Buzzer alarm .

Buzzer warning functions are as follows：

- 所有的报警事件发生，均是持续长鸣一段时间；

No matter what types of alarm event occur(including Alarm recording 1~8, Motion detection recording, G-force recording,Speed recording and Panic button recording), the Buzzer alarm will last for a while

- 只要出现蜂鸣器间断式鸣笛，即代表DV424当前是无法录像的；

When the Buzzer alarms intermittently, it means that the DVR is unable to record currently.

  - Alarm,Speed Interface， DB26

- alarm, speed interface cable

See the picture below

DB26p female head on the DV424 connect to DB26p male head

- Pin Definition:

| PIN 14 | PIN 23 | PIN 6 | PIN 15 | PIN 7 | PIN 16 | PIN 25 | PIN 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SENSOR-IN1 | SENSOR-IN2 | SENSOR-IN3 | SENSOR-IN4 | SENSOR-IN5 | SENSOR-IN6 | SENSOR-IN7 | SENSOR-IN8 |
| ALARM INPUT 1~4 |  |  |  | Reversal input | Brake input | Turn left input | Turn right input |
| red | red | red | red | red | red | red | red |

| PIN 5 | PIN 24 | PIN 26 | PIN 9 | PIN 17 | PIN 18 |
| --- | --- | --- | --- | --- | --- |
| 12V OUT | SENSOR-OUT1 | SENSOR-OUT1 | GND | SPEED+ | SPEED- |
| 12V DC power output | Alarm output 1 | Alarm output 2 | GND line | SPEED+ | SPEED- |
| red | black | black | black | black | black |

- There are 8 alarm inputs including alarm inputs 1 ~ 4, reversal input, brake input, turn left input, turn right input, which can trigger the alarm recording. The first 4 alarm inputs are normal alarm input defined by user.  The last 4 are specific alarm input and display cursor for each channel.

- Alarm output 1 and Alarm output 2 are 12V output by default, which can be used as a trigger and need to be set up to combine with alarm input . You can also setup BUZZER for the output.

- If Alarm input 1 is active and combined with Alarm output 1, the Alarm output 1 will output a high-level voltage to trigger other device.

- SPEED+&SPEED-

These 2 cable is for speed detection and needs to connect to speed line on vehicle.

Check the real-time vehicle speed with the vehicle speedometer:

Wiring diagram:

The vehicle speedometer is driven by the Speed sensor, see the right figure.

If the speedometer has a speed signal output, it can be connected directly to the PIN17of  the DB26 cable and PIN18 for reference level.

If the speedometer does not have the speed signal output, then connect SIG Output of the speedometer which is output by Speed sensor to the PIN17.

  - Panic button (Optional)

- Overview

The LEDs are used to show the device’s working status. But when the device is installed in the vehicle,it is not easy to check the LED on the front panel.  Each of the LED indicates the corresponding status. Furthermore, the panic button on the panel make it easier to trigger alarm for emergency or bookmarking a manual event.

- Pin Definition

DB26p male head connect to DB26p female head on the DV424

2*5 PIN/3.0 interface connect to the interface on panic button

- LED

| LED | Color | ON | OFF |
| --- | --- | --- | --- |
| VLoss | Amber | Any of the cameras have no signal alarm | Normal Operation |
| Rec | Soft green | Recording Normal driving | Not recording |
| GPS | Amber | GPS cannot latch | Normal Operation |
| Mem | Red | Storage Alarm or no Storage device | Normal Operation |
| Comm | Amber | Device is not connected to server | Normal operation or device is not connected to server if this feature is disabled |
| Power | Pale Blue | Device has power | Device does not have power |
| Error | Red | Error with device | Normal Operation |
| Event | Red | Event-based Recording (remains lit during Event) | Normal Operation |

- Button

PANIC button, printed as “Bookmark”

a. When pressed, will trigger a manual event

b. When pressed, will temporarily illuminate the Event LED

  - Built-in GPS antenna

Built-in GPS antenna socket and GPS antenna, as shown in the picture.

- The Menu

The default setting can meet the requirement of most of users.

Menu Introduction

Press [MENU] on the remote or Touch the bottom area, the LOGIN page will be displayed on the LCD screen. The Shortcut Menu will be displayed after login. If you press [MENU] on the remote or Touch the bottom area again, the Main Menu will be displayed.

① System Time Display

② License plate number Display

③ Recording Sign

The Recording Sign will turn red when recording

④ Playback Sign

The Playback Sign will turn red during playback.

⑤ Menu Lock Sign

Lock图标变红 :  电子锁锁住状态。

Lock indicator turns red when electronic lock is locked.

电子锁和菜单锁不一样，电子锁是DV424前面盖的锁，菜单锁是上图所示[Area 10的菜单的锁

Electronic lock is different from menu lock.

⑥ GPS Sign

The GPS Sign will be flashing when connecting, it will be always ON when successfully connected.

⑦ Cell Sign

⑧ WiFi Sign

⑨ Video Loss Sign

⑩ Menu

Press [Area 10] to display MENU Sign

⑪ Channel Name Sign

  - Manually Record

Click this icon to start or stop recording. Video files are saved in NORMAL list of Player menu.

  - Playback

Video Playback button: Click this button to enter the calendar menu.

If a date is marked green, that means it has recording files saved on this day. Click the icon to enter the video file list and choose the one you want to play and press Play button. You can play single or multiple videos. Multiple videos can be played in sequence and can shift into the next or the previous one.  Specific operation sees below.

:Search month

:Search year

| Type | Recording Time Control Mode | View Position |
| --- | --- | --- |
| Normal recording | Manual control | Normal list |
| Power on recording | Manual control | Normal list |
| Schedule recording | Pre-setup time | Normal list |
| Alarm recording 1~8 | Event recording setup time | Event list |
| Motion detection recording | Event recording setup time | Event list |
| G-force recording | Event recording setup time | Event list |
| Speed recording | Event recording setup time | Event list |
| Panic button recording | Event recording setup time | Event list |

  - Log

System memo checking, memo output

  - Display mode switching

Mode switch button: click the button, preview interface will switch from the eight-split display to quad display, CH1 ~ CH4

  - Setup

Setup button: Click the button system will enter the setup menu.A prompt dialog will display "Unable to record in set-up mode! Continue?” Click OK to enter.

  - Disk

Disk management button: click the disk management button, you can view the HDD/SSD, SD card, USB status.

①磁盘类型

①Disk type

②ALL：磁盘总容量，free：磁盘剩余容量

②ALL：The total capacity of disk，free：The residual capacity of disk

如果ALL显示0.00，则表示DV424没有接入这种类型的磁盘

If ALL shows 0.00MB, it means that DV424 does not have access to this type of disk

③绿色区域表示磁盘的已使用容量

③The green area indicates the used capacity of the disk

④格式化按钮，格式化磁盘会有弹框提示，下图以格式化USB2为例

④Click the button to format the disk .

A dialog box displaying “Disk data will be deleted! Continue?”will pop up. Click OK to start formatting the disk

The following picture is an example of formatting USB2

如果不能格式化磁盘，请检查以下情况

If the disk cannot be formatted, please check whether:

- 卡槽是否有磁盘

- There is a disk in the slot

- 有没有把所有录像关掉

- All recordings are turned off

- 有没有关掉FTP按钮

- The FTP button is turned off

⑤图标表示磁盘需要格式化后才能使用，一般新卡都需要格式化之后才能使用

⑤  It shows that the disk needs to be formatted before it can be used.

Generally all new disks need to be formatted before they can be used

  - Volume

Modulation：0 is the minimum volume, and 10 is the maximum volume.And the default value is 5.

音量调节：0是最小音量（也是关闭音量），10是最大音量，默认的设置值是5

- Record Setup

  - Power On Rec

Recording will start when power is on if set the 'Power On Rec’ ON. Default is ON.

  - Cyclic Rec

New video files will overwrite the previous ones when disk is full if setting the Continuous Rec ON. Recording will stop when disk is full if setting the Continuous Rec OFF. Overwriting will not cover event recording files.

  - Event Rec

Event recording type includes motion detection triggered alarm, G-force triggered alarm, alarm 1 ~ 8 triggered alarm and over speed alarm. If the event switch is on and corresponding alarm parameters are set, event recording will be activated when above events are triggered. If the event switch is off, event recording will not be activated even if the event is triggered.

  - Video Quality

① Resolution

There are 4 kinds of optional resolution, 1080P, 720P, D1 (PAL), D1 (NTSC). The higher the resolution & the better the video quality, the larger the video file. Therefore, the configuration should be taken into account together with the file size

② Bit rate

There are 4 levels of bit rates for selection, 4Mbps, 2Mbps, 1Mbps, 512Kbps. The higher the bit rate, the clearer the image, the larger the video file. Therefore, all factors should be considered comprehensively.

③ Frame rate

There are 4 levels of frame rates for option : 25fps，20fps，15fps，10fps. The higher the frame rates is, more smooth the picture is and the larger the video file is.

④ Audio

Device has audio output when audio switch is on.

| SSD/HDD/SD capacity | Video Quality | File length |
| --- | --- | --- |
| 2T | 8 * 1080P、4Mpbs | ≈150h |
|  | 8 * 720P、2Mpbs | ≈300h |
|  | 8 * D1、1Mpbs | ≈600h |
|  | 1 * 1080P、4Mpbs | ≈1200h |
|  | 1 * 720P、2Mpbs | ≈2400h |
|  | 1 * D1、1Mpbs | ≈4800h |
| 4*128G | 8 * 1080P、4Mpbs | ≈38h |
|  | 8 * 720P、2Mpbs | ≈75h |
|  | 8 * D1、1Mpbs | ≈150h |
|  | 1 * 1080P、4Mpbs | ≈304h |
|  | 1 * 720P、2Mpbs | ≈608h |
|  | 1 * D1、1Mpbs | ≈1216h |

  - Record Channel

After turn on recording (including all types) and select the recording channel, the corresponding channel will be recorded. If turning off the video channel, the corresponding channel will not be recorded even if the recording function is on.

Note: The config is for normal record and NOT for event record. event record will record all channels by default and can’t be changed.

  - Event duration

When alarm recording is on, the video file length can be set to 5s, 10s, 15s after the event happens.

  - File Length

When normal recording is on, the video file length for each file can be set to 5mins, 10mins, 15mins.

  - Motion Sensitivity

Motion detection recording and sensitivity level setting: When there is an object moving and its movement amplitude exceeds the preset motion detection sensitivity level, event recording will be triggered. For this kind of event record or alarm record, the pre-record time will be set as 15s and the post-event time is configured by Event Duration above.

Total video file length = pre-recording file length(fixed 15s) + file length (configured by Event Duration).

If motion detection is off, event recording will not be triggered. Motion detection sensitivity can be set to two levels, middle / high. Motion detection recording is on when middle / high is selected.Motion detection recording is off when OFF is selected.

  - G-force Sensitivity

G-force triggered recording and sensitivity level setting: When the acceleration  or gyroscope of the G-force reaches the preset sensitivity value, G-force recording will be triggered. For this kind of event record or alarm record, the pre-record time will be set as 15s and the post-event time is configured by Event Duration above.

Total video file length = pre-recording file length(fixed 15s) + file length (configured by Event Duration).

If G-force triggered recording is off, event recording will not be triggered. G-force sensitivity can be set to two levels, low / high. G-force triggered recording is on when low / high is selected.G-force triggered recording is off when OFF is selected.

  - File Type

File format setting.

- Display

  - Camera display setting

Parameter setting for each corresponding camera channel: including brightness, contrast, saturation, hue. By default, all setting values are 50.

Corresponding value can be changed by pressing and moving the bar to left or right.

  - Camera name setting

Camera name： Set channel name, the channel name then will be displayed at the bottom of that channel.

Press the channel name on menu, there will be an keyboard menu pop up to input a new channel name.

Each channel contains max. 8 characters.And camera name must not be blank.

  - System Language setting

Language Multiple Languages available: English, French, German, Russian, Japanese, Italian, Austrian

  - Audio Out

Audio out: choose the audio output channel in split mode

  - OSD display setting

OSD configuration, select whether to display time, channel name, license plate in video (if yes, all these information will be written in video and can be seen in playback)

  - Menu on

Set duration of menu display

- 更改Menu lock的状态需要输入密码，如下图将Menu lock的状态改为Off

Password is needed to change the status of Menu lock. A为The following picture shows how to change the Menu Lock status from ON to OFF.

当Menu lock的状态为on，则进入playback、log、setup和disk四个菜单时需要输入密码

When the status of Menu Lock is On, password is needed to enter when access menu of Record, Playback, Log, Setup, Disk.

Menu lock——Password设置如下，初始密码是123

Password setting for Menu Lock are as follows. And the initial password is 123.

  - Speed

speed setting

There are two speed source, one is from GPS and the other from vehicle speedometer, as described above about DB26 cable.

  - GPS

When GPS is set as on, latitude, longitude and speed will be recorded into video files.The menu provides GPS information of latitude / longitude, detectable satellites, accessible satellite etc.

Mode:GPS status,it will be shown as below:

connected

disconnected

Connecting

  - Mirror

ON:Turn on Mirror function

OFF:Turn off Mirror function

- Network

  - LAN port and server setting

DHCP: Dynamic Host Configuration Protocol, when it is on, it is dynamic IP, when it is off, it is static IP, you need to input IP address, sub-net mask and gateway manually. MAC address can be auto assigned or revised

OK：Press OK to save setting and exit

Cancel：Press cancel to exit menu without saving settings

  - WIFI network setup and server setup

WIFI： WIFI on/off

DHCP： DHCP means Dynamic IP switch. When DHCP is on, that is dynamic IP. If DHCP is off, that is static IP. Static IP requests to manual input IP address, mask and gateway. Mac address can be automatically assigned or revised.

SSID： WIFI hot spot list

- Enable WIFI

Step 1: WIFI hot spot available

Step 2:Connect the WIFI antenna at connector ⑦ of device rear panel

Step 3:Go to WIFI setup interface and set WIFI on and open the dynamic IP button.

Step 4:Click SSID sub-menu and the WIFI hot spot shows up. Select which hots pot to connect and enter the password.

Step 5:Click OK and quit the WIFI setup interface.

Step 6:Input WIFI Server IP and Port in “Network-Server Setup”  page.

Step 7:WIFI network status is “CONNECT SUCCESS” and server status is “Online”

  - 3G/4G control and its network setup

Cellular: Cellular is on, meaning that 3G/4G is on.

Network Standard: WCDMA by default.

APN &Access Number: Normally, user don’t need to enter user name and password for APN and Access number. The default setting is available. If it can’t communicate with the network under the default setting, please consult your local network carrier.

Username &Password: Reversed.

OK: Save the setting and quit

Cancel: cancel the setting and quit

- Enable 3G/4G

step 1: DVR can search 3G/4G signal locally.

step 2:Connect the 3G/4G antenna at connector ⑥ of DVR rear panel

step 3:Open the DVR front housing and insert the 3G/4G SIM card

step 4:Open Cellular setup interface and click Cellular.

step 5:Input the APN and Access Number correctly. Access Number can be skipped.

step 6:Click “OK” to exit CELL setup menu.

step 7: Input 3G/4G Server IP and Port in “Network-Server Setup”  page.

step 8: Cellular network status is “Success” and server status is “Online”

  - Network Status

The user can check information such as LAN IP address、MAC address、WIFI network status、WIFI IP address、WIFI signal strength、3G/4G network status、3G/4G signal strength, and Server status.

Additionally, users can verify whether network connection is successful or not.

LAN IP：Refers to the static IP set on Network-LAN page or the dynamic IP which is obtained automatically.

MAC：Refers to the static physical address set on Network-LAN page or dynamic physical address which is obtained automatically.

WIFI：WIFI on/off status obtained from Network-WIFI page

WiFi RSSI：Wifi signal strength icon

WiFi IP： Static IP obtained from Network-LAN pages or dynamic IP.

WIFI status：WIFI status will be shown as below:

CONNECT SUCCESS

GETIP ERROR

...

Cellular : The on/off status of cellular acquired from Network-cellular page

Wireless RSSI：3G/4G signal strength icon

Module type：Display the types of 3G/4G module, the parameters and the corresponding types shown are as follows

3G:3G module

4G:4G module

Wireless Status: Value and corresponding meanings

1:Module initialization

2:Module exception

3:No SIMcard

4:Cpin locked

5:Signal abnormal

6:Networking failure

SUCCESS: Networking success

...

Server Status: Online / Offline

  - Server

  - FTP

- System

  - Log in setup

Set user name and password for boot up,The initial password is 123.

  - License plate number setup

Set license plate number and IP number

  - System time setup

  - Scheduled Recording

Enable:Set scheduled recording ON/OFF.

Start:Set start time of scheduled recording.

End:Set end time of scheduled recording.

Week-day:Set scheduled recording by weekdays. Select the weekdays to set preset.

Scheduled Recording:

* Supports up to four appointed tasks, the recording duration is counted in   minutes.

* Recording time can overlap.

* The start time of scheduled recording must be set ahead of the end time.

  - ACC settings

Current vol.: Voltage of the working DVR

Shutdown vol.: Shutdown voltage function will take effect after DVR starts working for 10mins. DVR would shut down automatically if current voltage is lower than shutdown voltage, it would reboot only when the voltage is above the value.

ACC Delay: DVR will continue recording for a few seconds after ACC is disconnected. ACC delay time can be set to be 5s, 10s or 15s.

  - Alarm information setting

Alarm1~Alarm4： Customized alarm recording

Reverse：Reversing alarm recording, parking line cursor will display when reverse alarm is triggered.

Brake：Brake alarm recording, brake sign will display when brake alarm is triggered.

Left：Turning- left alarm recording, turn-left cursor will display when turn left alarm is triggered.

Right：Turning- right alarm recording, turn-right cursor will display when turn right alarm is triggered.

Priority : 可以设置Alarm1~Alarm4、Reverse、Brake、Left、Right的优先级，当几种类型的报警同时触发时，优先级高的报警会优先录像

Priority : Set priorities for Alarm1~Alarm4,Reverse,Brake, Left, and Right.

When different types of alarm are triggered at the same time, alarms with the highest priority will work first.

Trigger Level：There are 3 options of Trigger Level , the options “Low” and ” High” are used for turning on alarm function. “Low” is generally used for debugging while “High” will be selected to turn on alarm function for on road use. “Off” means turn off alarm trigger function.

Duration： Duration of alarm video recording

Alarm Out-Buzzer： Set the Buzzer ON, it would beep for 5 seconds when alarming.

Alarm Out-Output1：Set it ON, 12V level output would come from the alarm wire of Output 1

Alarm Out-Output2：Set it ON, 12V level output would come from the alarm wire of Output 1

Display：A full screen of one channel will display when the channel is triggered.

：按一下，选中报警的优先级的数值加1，数值越大，优先级越低

Click, the priority value of the selected alarm will be added by 1. The bigger the value, the lower the priority

：按一下，选中报警的优先级的数值减1，数值越小，优先级越高

Click, the priority value of the selected alarm will be reduced by 1. The smaller the value, the higher the priority

- 优先级高的报警会优先触发

- Alarms with higher priority will be triggered first

- 1是优先级最高，8是优先级最低

- 1 is the highest priority, and 8 is the lowest.

- 上图是优先级的默认排序

- The diagram above is the default sort of priority

- 上图的优先级排序是：

- The priority sort of the diagram above is:

Alarm 1>Alarm 2>Alarm 3>Alarm 4>Reverse>Brake>Left>Right

- 如果如果两个报警同时触发，优先级高的先录完，优先级低的在优先极高的录完之后就没有触发了，那优先级低的便不会录像了

- If two alarms A and B are triggered at the same time, and A’s priority is higher than B’s, A will record first. After A finishes the recording, if B is still being triggered,  B will then record. However, if B is no longer being triggered now, it will not record.

- 如果A的优先级比B的高，B先触发，在录像过程中，A又触发，此时B的录像会立即中断，A的录像会开始生成

- If alarm B is triggered and at the process of recording. However, alarm A, whose priority is higher than B, is triggered then, B will stop recording at once, and A will start to record

  - Update

- For single device

Step 1：Copy the folder to USB disk or SD card root directory and insert the USB disk or SD card into DVR.

Step 2：Power off the DVR and reboot it, then it will auto upgrade. Or in the menu Menu -> Setup -> System -> Update, click OK to confirm to upgrade. Both method can start the upgrade process.

Step 3：When “Update success!” shows on the DVR monitor, DVR will auto reboot.

Step 4：After reboot, please check if the version is the same as the one you copy into “upgrade” folder. Please go to Menu -> Setup -> System -> Info to check.

- For batch upgrade

Currently, as the upgrade package will be deleted after the upgrade process is done on device, we need to copy the package again from PC. And that is troublesome. So please carry out as follows:

Step 1：Rename the package “dv424_upgrade_2017xxxxxxxx” to “dv424_upgrade_never_rename”

Step 2：Copy the package to USB disk or SD card root directory, and insert it to DVR.

Step 3：Power off the DVR and reboot it, then it will auto upgrade. Or in the menu Menu -> System -> Update, click OK to confirm to upgrade. Both method can start the upgrade process.

Step 4：After When “Update success!” shows, unplug the USB disk or SD card with upgrade package, then DVR will auto reboot.

Note: Use “dv424_upgrade_never_rename” package to upgrade, when “update success!” shows on the screen,  we must unplug the USB disk or SD card, or else the DVR will go into infinite loop for upgrade and will not boot up.

SOLUTION: Unplug the USB disk or SD card with upgrade package. DVR will stop the upgrade process and boot up successfully.

- Remote upgrade

Step 1：DVR connects to server.

Step 2：Open the Windows client and log in.

Step 3：Find the exact DVR device’s license number in the device list of the client, right click and select “upgrade” and select the upgrade package “dv424_upgrade_2017xxxxxxxx”

Note: For remote upgrade the package can’t be named as “dv424_upgrade_never_rename”.

  - Configuration

Configuration Import: Import the configuration information from USB memory flash devices.

Configuration Export: Export Log to USB memory flash devices.

Factory Default: Click RESET to restore factory setting.

  - System Info

System Info：Software version number.

FAQ

1) The system can’t  start?

Usually this problem results from the incorrect power connection. Please follow the steps below to check the power connection:

1. Check the input power, whether the power wire is connected correctly, whether the ground wire is connected to the battery, and whether the fuse on the power wire is in good condition.

2. Check whether the voltage of the ACC signal wire is higher than 6 V.

3. Check whether he input voltage of the device is higher than the shutdown voltage set on the screen of DV424

2) The DV424 restarts uninterruptedly?

Please followthe steps below to check it:

1.Check whether the supply voltage of DV424 is insufficient.If it is lower than the start-up voltage of the device, the device would restart repeatedly.

2、Restart DV424 to see whether it can work properly this time

3) Unable to recognize disks？

1、Check whether the disk itself is in good condition and make sure it is installed and of fine contact

2、The disk has been formatted by DV424

3、Restart DV424 to see whether it can work properly this time

4) Unable to recognize cameras？

1、Make sure the camera is intact and the connection is correct.

2、Re-fit all wires (e.g,DB44 wire & extended wires) between cameras and the device

3、Restart DV424 to see whether it can work properly this time

5) GPS anomaly？

Check whether the GPS antenna is properly installed