OKLAHOMA SPS-10

MATERIALS SAMPLING AND TESTING PLAN

DRAFT

Contents

I.	GENERAL LAYOUT INFORMATION	1

I.1.	Core test sections	2

I.2.	Supplementary test sections	4

II.	MATERIALS SAMPLING AND FIELD TESTING	6

II.1.	Identification Codes	6

Note*: The layer number is assigned with one character instead of two in the Material Tracking System as it accepts only of eight characters. The specimen ID, therefore, was reduced to eight characters during sampling.	8

II.2.	Material Sampling	9

II.2.1.	Core Test Sections	9

II.2.2.	Supplementary test sections	15

II.2.3.	Samples acquired from plant	21

II.3.	Field Testing	22

II.3.1.	Core test sections	22

II.3.2.	Supplementary sections	24

II.4.	Samples to be shipped	26

II.5.	Note on change in Specimen ID in MST plan:	30

III.	LABORATORY MATERIAL TESTING	31

III.1.	Testing Plans	32

III.2.	Tracking Tables	37

# GENERAL LAYOUT INFORMATION

This section of the plan provides a description of the SPS-10 project in terms of the location of the test sections along the roadway. Table 1 lists each section and its respective description.

Table 1. Test sections identification and description

| Section Number | Section ID | Description |
| --- | --- | --- |
| 01 | 40AA01 | HMA Control Section |
| 02 | 40AA02 | Mix with WMA Foaming Process |
| 03 | 40AA03 | Mix with WMA Chemical Additive |
| 61 | 40AA61 | WMA with PG 64-22 with 10-25% RAP/RAS |
| 62 | 40AA62 | WMA with PG 58-28 with 10-25% RAP/RAS |
| 63 | 40AA63 | SMA/WMA with PG 70-28 without RAP/RAS |

## Core test sections

Table 2 tracks the core test sections from the beginning of the initial transition area at station 00+00 to the end of the last section at station 48+50.

Table 2 indicates transition areas between each section as well as the buffer and monitoring areas for each section.  Additionally, Figure 1 provides a visual representation of the core test sections layout.

Table 2. Core test sections layout

| Description | Core Test Sections |  |  |
| --- | --- | --- | --- |
|  | Begin STA | End STA | Section ID |
| Transition | 00+00 | 04+50 |  |
| Sampling Area | 04+50 | 05+50 | 40AA01 |
| Buffer Area | 05+50 | 06+00 |  |
| Monitoring Area | 06+00 | 11+00 |  |
| Buffer Area | 11+00 | 11+50 |  |
| Sampling Area | 11+50 | 12+50 |  |
| Transition | 12+50 | 23+50 |  |
| Sampling Area | 23+50 | 24+50 | 40AA02 |
| Buffer Area | 24+50 | 25+00 |  |
| Monitoring Area | 25+00 | 30+00 |  |
| Buffer Area | 30+00 | 30+50 |  |
| Sampling Area | 30+50 | 31+50 |  |
| Transition | 31+50 | 40+50 |  |
| Sampling Area | 40+50 | 41+50 | 40AA03 |
| Buffer Area | 41+50 | 42+00 |  |
| Monitoring Area | 42+00 | 47+00 |  |
| Buffer Area | 47+00 | 47+50 |  |
| Sampling Area | 47+50 | 48+50 |  |

Sampling Area 011

Buffer Area (50 ft)

40AA01

HMA Control

Test Section

Buffer Area (50 ft)

Sampling Area 021

Transition (length variable)

Sampling Area 031

Buffer Area (50 ft)

40AA02

WMA Foaming

Process Test Section

Buffer Area (50 ft)

Sampling Area 041

Transition (length variable)

Buffer Area (50 ft)

40AA03

WMA Chemical

Additive Test Section

Buffer Area (50 ft)

Sampling Area 061

Sampling Area 051

800ft

800ft

800ft

500ft

500ft

500ft

100ft

100ft

100ft

100ft

100ft

100ft

Traffic

12+50

23+50

04+50

31+50

40+50

48+50

11+50

16+50

29+50

34+50

47+50

52+50

Odd sampling areas used for t0, t2, t4, t6, t8, t10; even sampling areas used for t1, t3, t5, t7, t9

Figure 1. Oklahoma SPS-10 core test sections  layout

## Supplementary test sections

Table 3 tracks the test sections from the beginning of the initial transition area at station 48+50 to the end of the last section at station 100+50.  Table 3 indicates transition areas between each section as well as the buffer and monitoring areas for each section.  Additionally, Figure 2 provides a visual representation of the supplementary test sections layout.

Table 3. Supplementary test sections layout

| Description | Supplementary test sections |  |  |
| --- | --- | --- | --- |
|  | Begin STA | End STA | Section ID |
| Transition | 48+50 | 54+50 |  |
| Sampling Area | 54+50 | 55+50 | 40AA61 |
| Buffer Area | 55+50 | 56+00 |  |
| Monitoring Area | 56+00 | 61+00 |  |
| Buffer Area | 61+00 | 61+50 |  |
| Sampling Area | 61+50 | 62+50 |  |
| Transition | 62+50 | 70+50 |  |
| Sampling Area | 70+50 | 71+50 | 40AA62 |
| Buffer Area | 71+50 | 72+00 |  |
| Monitoring Area | 72+00 | 77+00 |  |
| Buffer Area | 77+00 | 77+50 |  |
| Sampling Area | 77+50 | 78+50 |  |
| Transition | 78+50 | 92+50 |  |
| Sampling Area | 92+50 | 93+50 | 40AA63 |
| Buffer Area | 93+50 | 94+00 |  |
| Monitoring Area | 94+00 | 99+00 |  |
| Buffer Area | 99+00 | 99+50 |  |
| Sampling Area | 99+50 | 100+50 |  |

Sampling Area 071

Buffer Area (50 ft)

40AA61

WMA

PG 70-28 with 10-25% RAP/RAS Test Section

Buffer Area (50 ft)

Sampling Area 081

Transition (length variable)

Sampling Area 091

Buffer Area (50 ft)

40AA62

WMA

PG 64-22 with 10-25% RAP/RAS Test Section

Buffer Area (50 ft)

Sampling Area 101

Transition (length variable)

Buffer Area (50 ft)

40AA63

WMA

PG 58-28 with 10-25% RAP/RAS Test Section

Buffer Area (50 ft)

Sampling Area 121

Sampling Area 111

800ft

800ft

800ft

500ft

500ft

500ft

100ft

100ft

100ft

100ft

100ft

100ft

Traffic

62+50

70+50

54+50

78+50

92+50

100+50

61+50

66+50

77+50

82+50

99+50

104+50

Odd sampling areas used for t0, t2, t4, t6, t8, t10; even sampling areas used for t1, t3, t5, t7, t9

Figure 2. Oklahoma SPS-10 project layout, supplementary sections

# MATERIALS SAMPLING AND FIELD TESTING

This section of the plan provides for the material sampling and testing activities that occur in the field as part of time 0 (t0 is at time of construction). Table 4 provides the project layer numbering schema.  It is very important that the project layer numbering be consistent to ensure that the resulting data is not misunderstood.  Table 5 provides the scope of material sampling for core sections and Table 9 provides the scope of material sampling for the supplemental sections.

Figures 3 through 6 show the material sampling locations for each section as well as the location of DCP testing.  Figures 7 through 12 show density measurement layouts. In situ density testing will be conducted on the finished bituminous surface layer. This testing shall be performed at the specified locations as shown in figures using AASHTO T310-11, backscatter method.  Each testing location shall have four readings with the density instrument rotated 90° between each reading.

Tables 6 through 8 list the samples acquired from each of the six test sections as well as the asphalt plant for the core sections and Tables 9 through 12 list that for the supplemental sections.  These tables define the sample number and specimen ID for every sample collected during the experiment. These tables will assist in the proper labeling of each sample as they are collected.

Table 13 lists the samples that are acquired from the plant while Table 14 and Table 15 list the scope of field testing in the test sections. Finally, Tables 16 through 18 list samples to be shipped to the central lab testing contractor laboratory, the LTPP Materials Reference Library (MRL), and the Turner Fairbanks Research Center, respectively.

## Identification Codes

There are three primary sample identification codes used in the materials sampling and testing plan: sample location, sample number, and specimen ID.   A description of each code format follows.

Sample Location: S-LL-t -XX

- S = Sample location type

  - A – bound, compacted bulk sample (12 inch diameter core)

  - F – bound bulk sample (uncompacted bulk mix obtained on site)

  - U – combined aggregate used in asphalt concrete mixes

  - C – 4 or 6 inch diameter core

  - T – nuclear density/moisture gage

  - H – samples obtained from the HMA plant

- LL = Sample location number

- t  = Sampling time interval

- XX  = Section number

Sample Number: S-M-##-t-XX

- S = Sample type

  - C – core sample

  - B – bulk sample

  - M – moisture sample

- M = Material type

  - A – asphalt concrete

  - C – asphalt cement

  - R – RAP

  - W – warm mix asphalt additive

  - T – treated, bound, or stabilized base/subbase

  - G – untreated, unbound granular base/subbase

  - S – subgrade soil or fill material

  - U – combined aggregate used in asphalt concrete mixes

- ## = Sample number

- t = Sampling time interval

- XX = Section number

Specimen ID: R-C-T-S-NN-L-##

- R = RSC Code (For Oklahoma SPS-10, R = “S,” representing southern region)

- C = Construction event (For Oklahoma SPS-10, C = 4)

- T = Test Section Number

  - 1 – 01

  - 2 – 02

  - 3 – 03

  - 4 – 61

  - 5 – 62

  - 6 – 63

- S = Sample Location Type

  - A – bound, compacted bulk sample (12 inch diameter core)

  - F – bound bulk sample (uncompacted bulk mix obtained on site)

  - U – combined aggregate used in asphalt concrete mixes

  - C – 4 or 6 inch diameter core

  - T – nuclear density/moisture gage

  - H – samples obtained from the HMA plant

- NN = Sample Location Number

- L = “L,” representing layer

- ## = Layer Number*

Table 4. Project layer numbering

| Layer No. | Description |
| --- | --- |
| 1 | Subgrade |
| 2 | Unbound Base |
| 3 | Original HMAC |
| 4 | Experimental AC |

Note*: The layer number is assigned with one character instead of two in the Material Tracking System as it accepts only of eight characters. The specimen ID, therefore, was reduced to eight characters during sampling.

| Layer No. | Project Layer Code | Thickness | Material Code | Description |
| --- | --- | --- | --- | --- |
| 1 | A |  | 101 | (SS) Subgrade – Fine Grade Soil |
| 2 | B |  | 307 | Base Layer (GB) Unbound granular base (soil aggregate mix) |
| 3 | C |  | 1 | AC below surface (binder) |
| 4 | D |  | 1 | AC below surface (binder) |
| 5 | E |  | 1 | AC below surface (binder) |
| 6 | F |  | 1 | AC below surface (binder) |
| 7 | G |  | 1 | Original Surface (top) |
| 8 | H |  | 1 | Experimental AC Overlay |

## Material Sampling

### Core Test Sections

Table 5. Scope of material sampling for core test sections

| Material and Sample Description |  | No. of Samples per section |  |  | Sample Location |
| --- | --- | --- | --- | --- | --- |
|  |  | 01 | 02 | 03 |  |
|  | PRE-CONSTRUCTION SAMPLING |  |  |  |  |
| A. | ORIGINAL HMAC |  |  |  |  |
|  | Bulk sample from the test pit (1’ x 1’) | 2 | - | 2 | A01001, A02001 A01003, A02003 |
|  | 6" Core | 3 | - | 3 | C19001-C21001, C19003-C21003 |
| B. | UNBOUND BASE |  |  |  |  |
|  | 150 lb bulk sample | 1 | - | 1 | A01001, A01003, |
| C. | SUBGRADE |  |  |  |  |
|  | 150 lb Bulk Sample and Moisture Content Jar | 1 | - | 1 | A01001, A01003 |
|  | DURING CONSTRUCTION SAMPLING |  |  |  |  |
| A. | EXPERIMENTAL AC |  |  |  |  |
|  | 5-gallon buckets uncompacted AC mix | 15 | 15 | 15 | F010XX-F050XX |
|  | 1-gallon can virgin AC Binder | 12 |  |  | H01001 |
|  | 5-gallon bucket combined aggregate | 20 |  |  | H02001 |
|  | 50 lb bulk sample RAP | 1 |  |  | H03001 |
|  | 5-gallon bucket RAP 50 lb bulk sample RAS 5-gallon bucket RAS | 10 1 10 |  |  | H03001 H04001 H04001 |
|  | 1-gallon chemical additive |  |  | 3 | H01003 |
|  | POST-CONSTRUCTION t0 SAMPLING |  |  |  |  |
| A. | EXPERIMENTAL AC |  |  |  |  |
|  | 6" Core | 15 | 15 | 15 | C01001-C15001, C01002-C15002, C01003-C15003 |

TABLE 6: Samples acquired from test section 40AA01

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| Pre-Construction Sampling |  |  |  |  |
| A01001 | 7 | BA01001 | S41A01L7 | Bulk sample from the test pit |
| A01001 | 6 | BA01001 | S41A01L6 | Bulk sample from the test pit |
| A01001 | 5 | BA01001 | S41A01L5 | Bulk sample from the test pit |
| A01001 | 2 | BG01001 | S41A01L2 | 150 lb Bulk Sample |
| A01001 | 1 | BS01001 | S41A01L1 | 150 lb Bulk Sample and Moisture Content Jar |
| A01001 | 1 | MS01001 | S41A01W1 | Moisture Content Jar |
| A02001 | 7 | BA02001 | S41A02L7 | Bulk sample from the test pit |
| A02001 | 6 | BA02001 | S41A02L6 | Bulk sample from the test pit |
| A02001 | 5 | BA02001 | S41A02L5 | Bulk sample from the test pit |
| C19001 | 7 | CA19001 | S41C19L7 | 6" Core |
| C19001 | 6 | CA19001 | S41C19L6 | 6" Core |
| C19001 | 5 | CA19001 | S41C19L5 | 6" Core |
| C20001 | 7 | CA20001 | S41C20L7 | 6" Core |
| C20001 | 6 | CA20001 | S41C20L6 | 6" Core |
| C20001 | 5 | CA20001 | S41C20L5 | 6" Core |
| C21001 | 7 | CA21001 | S41C21L7 | 6" Core |
| C21001 | 6 | CA21001 | S41C21L6 | 6" Core |
| C21001 | 5 | CA21001 | S41C21L5 | 6" Core |
| During Construction sampling |  |  |  |  |
| F01001 | 8 | BA03001 | S41F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02001 | 8 | BA04001 | S41F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03001 | 8 | BA05001 | S41F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04001 | 8 | BA06001 | S41F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05001 | 8 | BA07001 | S41F05L8 | 3 5-gallon buckets uncompacted AC mix |
| H01001 | 8 | BC01001 | S41H01L8 | 12 1-gallon can virgin AC Binder |
| H01001 | 8 | BC02001 | S41H02L8 | 12 1-gallon can virgin AC Binder |
| H01001 | 8 | BC03001 | S41H03L8 | 12 1-gallon can virgin AC Binder |
| H02001 | 8 | BU01001 | S41H04L8 | 10 5-gallon bucket combined aggregate |
| H03001 | 8 | BR01001 | S41H05L8 | 1 50 lb bulk sample RAP |
| H03001 | 8 | BR02001 | S41H06L8 | 10 5-gallon bucket RAP |
| H04001 | 8 | BR03001 | S41H07L8 | 1 50 lb bulk sample RAS |
| H04001 | 8 | BR04001 | S41H08L8 | 10 5-gallon bucket RAS |
| Post-construction – t0 sampling |  |  |  |  |
| C01001 | 8 | CA01001 | S41C01L8 | 6" Core |
| C02001 | 8 | CA02001 | S41C02L8 | 6" Core |
| C03001 | 8 | CA03001 | S41C03L8 | 6" Core |
| C04001 | 8 | CA04001 | S41C04L8 | 6" Core |
| C05001 | 8 | CA05001 | S41C05L8 | 6" Core |
| C06001 | 8 | CA06001 | S41C06L8 | 6" Core |
| C06001 | 6 | CA06001 | S41C06L6 | 6" Core |
| C06001 | 5 | CA06001 | S41C06L5 | 6" Core |
| C06001 | 4 | CA06001 | S41C06L4 | 6" Core |
| C06001 | 3 | CA06001 | S41C06L3 | 6" Core |
| C07001 | 8 | CA07001 | S41C07L8 | 6" Core |
| C08001 | 8 | CA08001 | S41C08L8 | 6" Core |
| C08001 | 6 | CA08001 | S41C08L6 | 6" Core |
| C08001 | 5 | CA08001 | S41C08L5 | 6" Core |
| C08001 | 4 | CA08001 | S41C08L4 | 6" Core |
| C08001 | 3 | CA08001 | S41C08L3 | 6" Core |
| C09001 | 8 | CA09001 | S41C09L8 | 6" Core |
| C09001 | 6 | CA09001 | S41C09L6 | 6" Core |
| C09001 | 5 | CA09001 | S41C09L5 | 6" Core |
| C09001 | 4 | CA09001 | S41C09L4 | 6" Core |
| C09001 | 3 | CA09001 | S41C09L3 | 6" Core |
| C10001 | 8 | CA10001 | S41C10L8 | 6" Core |
| C11001 | 8 | CA11001 | S41C11L8 | 6" Core |
| C11001 | 6 | CA11001 | S41C11L6 | 6" Core |
| C11001 | 5 | CA11001 | S41C11L5 | 6" Core |
| C11001 | 4 | CA11001 | S41C11L4 | 6" Core |
| C11001 | 3 | CA11001 | S41C11L3 | 6" Core |
| C12001 | 8 | CA12001 | S41C12L8 | 6" Core |
| C12001 | 6 | CA12001 | S41C12L6 | 6" Core |
| C12001 | 5 | CA12001 | S41C12L5 | 6" Core |
| C12001 | 4 | CA12001 | S41C12L4 | 6" Core |
| C12001 | 3 | CA12001 | S41C12L3 | 6" Core |
| C13001 | 8 | CA13001 | S41C13L8 | 6" Core |
| C13001 | 6 | CA13001 | S41C13L6 | 6" Core |
| C13001 | 5 | CA13001 | S41C13L5 | 6" Core |
| C13001 | 4 | CA13001 | S41C13L4 | 6" Core |
| C13001 | 3 | CA13001 | S41C13L3 | 6" Core |
| C14001 | 8 | CA14001 | S41C14L8 | 6" Core |
| C14001 | 6 | CA14001 | S41C14L6 | 6" Core |
| C14001 | 5 | CA14001 | S41C14L5 | 6" Core |
| C14001 | 4 | CA14001 | S41C14L4 | 6" Core |
| C14001 | 3 | CA14001 | S41C14L3 | 6" Core |
| C15001 | 8 | CA15001 | S41C15L8 | 6" Core |

Table 7. Samples acquired from test section 40AA02

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| During Construction sampling |  |  |  |  |
| F01002 | 8 | BA01002 | S42F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02002 | 8 | BA02002 | S42F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03002 | 8 | BA03002 | S42F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04002 | 8 | BA04002 | S42F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05002 | 8 | BA05002 | S42F05L8 | 3 5-gallon buckets uncompacted AC mix |
| Post-construction sampling |  |  |  |  |
| C01002 | 8 | CA01002 | S42C01L8 | 6" Core |
| C01002 | 6 | CA01002 | S42C01L6 | 6" Core |
| C01002 | 5 | CA01002 | S42C01L5 | 6" Core |
| C01002 | 4 | CA01002 | S42C01L4 | 6" Core |
| C01002 | 3 | CA01002 | S42C01L3 | 6" Core |
| C02002 | 8 | CA02002 | S42C02L8 | 6" Core |
| C03002 | 8 | CA03002 | S42C03L8 | 6" Core |
| C04002 | 8 | CA04002 | S42C04L8 | 6" Core |
| C05002 | 8 | CA05002 | S42C05L8 | 6" Core |
| C06002 | 8 | CA06002 | S42C06L8 | 6" Core |
| C07002 | 8 | CA07002 | S42C07L8 | 6" Core |
| C08002 | 8 | CA08002 | S42C08L8 | 6" Core |
| C08002 | 6 | CA08002 | S42C08L6 | 6" Core |
| C08002 | 5 | CA08002 | S42C08L5 | 6" Core |
| C08002 | 4 | CA08002 | S42C08L4 | 6" Core |
| C08002 | 3 | CA08002 | S42C08L3 | 6" Core |
| C09002 | 8 | CA09002 | S42C09L8 | 6" Core |
| C09002 | 6 | CA09002 | S42C09L6 | 6" Core |
| C09002 | 5 | CA09002 | S42C09L5 | 6" Core |
| C09002 | 4 | CA09002 | S42C09L4 | 6" Core |
| C09002 | 3 | CA09002 | S42C09L3 | 6" Core |
| C10002 | 8 | CA10002 | S42C10L8 | 6" Core |
| C10002 | 6 | CA10002 | S42C10L6 | 6" Core |
| C10002 | 5 | CA10002 | S42C10L5 | 6" Core |
| C10002 | 4 | CA10002 | S42C10L4 | 6" Core |
| C10002 | 3 | CA10002 | S42C10L3 | 6" Core |
| C11002 | 8 | CA11002 | S42C11L8 | 6" Core |
| C11002 | 6 | CA11002 | S42C11L6 | 6" Core |
| C11002 | 5 | CA11002 | S42C11L5 | 6" Core |
| C11002 | 4 | CA11002 | S42C11L4 | 6" Core |
| C11002 | 3 | CA11002 | S42C11L3 | 6" Core |
| C12002 | 8 | CA12002 | S42C12L8 | 6" Core |
| C12002 | 6 | CA12002 | S42C12L6 | 6" Core |
| C12002 | 5 | CA12002 | S42C12L5 | 6" Core |
| C12002 | 4 | CA12002 | S42C12L4 | 6" Core |
| C12002 | 3 | CA12002 | S42C12L3 | 6" Core |
| C13002 | 8 | CA13002 | S42C13L8 | 6" Core |
| C13002 | 6 | CA13002 | S42C13L6 | 6" Core |
| C13002 | 5 | CA13002 | S42C13L5 | 6" Core |
| C13002 | 4 | CA13002 | S42C13L4 | 6" Core |
| C13002 | 3 | CA13002 | S42C13L3 | 6" Core |
| C14002 | 8 | CA14002 | S42C14L8 | 6" Core |
| C14002 | 6 | CA14002 | S42C14L6 | 6" Core |
| C14002 | 5 | CA14002 | S42C14L5 | 6" Core |
| C14002 | 4 | CA14002 | S42C14L4 | 6" Core |
| C14002 | 3 | CA14002 | S42C14L3 | 6" Core |
| C15002 | 8 | CA15002 | S42C15L8 | 6" Core |
| C15002 | 6 | CA15002 | S42C15L6 | 6" Core |
| C15002 | 5 | CA15002 | S42C15L5 | 6" Core |
| C15002 | 4 | CA15002 | S42C15L4 | 6" Core |
| C15002 | 3 | CA15002 | S42C15L3 | 6" Core |

Table 8. Samples acquired from test section 40AA03

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| Pre-Construction Sampling |  |  |  |  |
| A01003 | 7 | BA01003 | S43A01L7 | Bulk sample from the test pit |
| A01003 | 6 | BA01003 | S43A01L6 | Bulk sample from the test pit |
| A01003 | 5 | BA01003 | S43A01L5 | Bulk sample from the test pit |
| A01003 | 2 | BG01003 | S43A01L2 | 150 lb Bulk Sample |
| A01003 | 1 | BS01003 | S43A01L1 | 1 50 lb Bulk Sample and Moisture Content Jar |
| A01003 | 1 | MS01003 | S43A01W1 | Moisture Content Jar |
| A02003 | 7 | BA02003 | S43A02L7 | Bulk sample from the test pit |
| A02003 | 6 | BA02003 | S43A02L6 | Bulk sample from the test pit |
| A02003 | 5 | BA02003 | S43A02L5 | Bulk sample from the test pit |
| C19003 | 7 | CA19003 | S43C19L7 | 6" Core |
| C19003 | 6 | CA19003 | S43C19L6 | 6" Core |
| C19003 | 5 | CA19003 | S43C19L5 | 6" Core |
| C19003 | 4 | CA19003 | S43C19L4 | 6" Core |
| C19003 | 3 | CA19003 | S43C19L3 | 6" Core |
| C20003 | 7 | CA20003 | S43C20L7 | 6" Core |
| C20003 | 6 | CA20003 | S43C20L6 | 6" Core |
| C20003 | 5 | CA20003 | S43C20L5 | 6" Core |
| C20003 | 4 | CA20003 | S43C20L4 | 6" Core |
| C20003 | 3 | CA20003 | S43C20L3 | 6" Core |
| C21003 | 7 | CA21003 | S43C21L7 | 6" Core |
| C21003 | 6 | CA21003 | S43C21L6 | 6" Core |
| C21003 | 5 | CA21003 | S43C21L5 | 6" Core |
| C21003 | 4 | CA21003 | S43C21L4 | 6" Core |
| C21003 | 3 | CA21003 | S43C21L3 | 6" Core |
| During Construction sampling |  |  |  |  |
| F01003 | 8 | BA03003 | S43F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02003 | 8 | BA04003 | S43F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03003 | 8 | BA05003 | S43F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04003 | 8 | BA06003 | S43F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05003 | 8 | BA07003 | S43F05L8 | 3 5-gallon buckets uncompacted AC mix |
| Post-construction sampling |  |  |  |  |
| C01003 | 8 | CA01003 | S43C01L8 | 6" Core |
| C01003 | 6 | CA01003 | S43C01L6 | 6" Core |
| C01003 | 5 | CA01003 | S43C01L5 | 6" Core |
| C01003 | 4 | CA01003 | S43C01L4 | 6" Core |
| C01003 | 3 | CA01003 | S43C01L3 | 6" Core |
| C02003 | 8 | CA02003 | S43C02L8 | 6" Core |
| C02003 | 6 | CA02003 | S43C02L6 | 6" Core |
| C02003 | 5 | CA02003 | S43C02L5 | 6" Core |
| C02003 | 4 | CA02003 | S43C02L4 | 6" Core |
| C02003 | 3 | CA02003 | S43C02L3 | 6" Core |
| C03003 | 8 | CA03003 | S43C03L8 | 6" Core |
| C03003 | 6 | CA03003 | S43C03L7 | 6" Core |
| C03003 | 5 | CA03003 | S43C03L6 | 6" Core |
| C03003 | 4 | CA03003 | S43C03L5 | 6" Core |
| C03003 | 3 | CA03003 | S43C03L4 | 6" Core |
| C04003 | 8 | CA04003 | S43C04L8 | 6" Core |
| C04003 | 6 | CA04003 | S43C04L6 | 6" Core |
| C04003 | 5 | CA04003 | S43C04L5 | 6" Core |
| C04003 | 4 | CA04003 | S43C04L4 | 6" Core |
| C04003 | 3 | CA04003 | S43C04L3 | 6" Core |
| C05003 | 8 | CA05003 | S43C05L8 | 6" Core |
| C05003 | 6 | CA05003 | S43C05L6 | 6" Core |
| C05003 | 5 | CA05003 | S43C05L5 | 6" Core |
| C05003 | 4 | CA05003 | S43C05L4 | 6" Core |
| C05003 | 3 | CA05003 | S43C05L3 | 6" Core |
| C06003 | 8 | CA06003 | S43C06L8 | 6" Core |
| C06003 | 6 | CA06003 | S43C06L6 | 6" Core |
| C06003 | 5 | CA06003 | S43C06L5 | 6" Core |
| C06003 | 4 | CA06003 | S43C06L4 | 6" Core |
| C06003 | 3 | CA06003 | S43C06L3 | 6" Core |
| C07003 | 8 | CA07003 | S43C07L8 | 6" Core |
| C07003 | 6 | CA07003 | S43C07L6 | 6" Core |
| C07003 | 5 | CA07003 | S43C07L5 | 6" Core |
| C07003 | 4 | CA07003 | S43C07L4 | 6" Core |
| C07003 | 3 | CA07003 | S43C07L3 | 6" Core |
| C08003 | 8 | CA08003 | S43C08L8 | 6" Core |
| C08003 | 6 | CA08003 | S43C08L6 | 6" Core |
| C08003 | 5 | CA08003 | S43C08L5 | 6" Core |
| C08003 | 4 | CA08003 | S43C08L4 | 6" Core |
| C08003 | 3 | CA08003 | S43C08L3 | 6" Core |
| C09003 | 8 | CA09003 | S43C09L8 | 6" Core |
| C09003 | 6 | CA09003 | S43C09L6 | 6" Core |
| C09003 | 5 | CA09003 | S43C09L5 | 6" Core |
| C09003 | 4 | CA09003 | S43C09L4 | 6" Core |
| C09003 | 3 | CA09003 | S43C09L3 | 6" Core |
| C10003 | 8 | CA10003 | S43C10L8 | 6" Core |
| C10003 | 6 | CA10003 | S43C10L6 | 6" Core |
| C10003 | 5 | CA10003 | S43C10L5 | 6" Core |
| C10003 | 4 | CA10003 | S43C10L4 | 6" Core |
| C10003 | 3 | CA10003 | S43C10L3 | 6" Core |
| C11003 | 8 | CA11003 | S43C11L8 | 6" Core |
| C11003 | 6 | CA11003 | S43C11L6 | 6" Core |
| C11003 | 5 | CA11003 | S43C11L5 | 6" Core |
| C11003 | 4 | CA11003 | S43C11L4 | 6" Core |
| C11003 | 3 | CA11003 | S43C11L3 | 6" Core |
| C12003 | 8 | CA12003 | S43C12L8 | 6" Core |
| C12003 | 6 | CA12003 | S43C12L6 | 6" Core |
| C12003 | 5 | CA12003 | S43C12L5 | 6" Core |
| C12003 | 4 | CA12003 | S43C12L4 | 6" Core |
| C12003 | 3 | CA12003 | S43C12L3 | 6" Core |
| C13003 | 8 | CA13003 | S43C13L8 | 6" Core |
| C13003 | 6 | CA13003 | S43C13L6 | 6" Core |
| C13003 | 5 | CA13003 | S43C13L5 | 6" Core |
| C13003 | 4 | CA13003 | S43C13L4 | 6" Core |
| C13003 | 3 | CA13003 | S43C13L3 | 6" Core |
| C14003 | 8 | CA14003 | S43C14L8 | 6" Core |
| C14003 | 6 | CA14003 | S43C14L6 | 6" Core |
| C14003 | 5 | CA14003 | S43C14L5 | 6" Core |
| C14003 | 4 | CA14003 | S43C14L4 | 6" Core |
| C14003 | 3 | CA14003 | S43C14L3 | 6" Core |
| C15003 | 8 | CA15003 | S43C15L8 | 6" Core |
| C15003 | 6 | CA15003 | S43C15L6 | 6" Core |
| C15003 | 5 | CA15003 | S43C15L5 | 6" Core |
| C15003 | 4 | CA15003 | S43C15L4 | 6" Core |
| C15003 | 3 | CA15003 | S43C15L3 | 6" Core |

A0200X

C2100X

C2000X

0-70

0-50

Section STA:

Section: 400A0X

Description: HMAC Control Section

Sampling Area: 01

Sampling Interval: t0

Shoulder

C1900X

18’

4’

2’

1’

A0100X

Section: 01

Description: HMA Control Section

Sampling Interval: t0

6’

6” Core Original HMAC

6” Core original HMAC; DCP

Sample Original HMAC;

Bulk Base and Subgrade

1’

2’

Figure 3. Sampling and testing plan for test section 01 and 03 at pre-construction Oklahoma SPS-10

0-70

0-50

Section STA:

Section: 400A0X

Description: HMAC Control Section

Sampling Area: 01

Sampling Interval: t0

Shoulder

C0200X

C0400X

C0600X

C0800X

C1000X

C1200X

C1400X

2’

1’

3’

2’

2’

2’

2’

2’

2’

2’

C0100X

C0300X

C0500X

C0700X

C0900X

C1100X

C1300X

C1500X

3’

2’

2’

2’

2’

2’

2’

Section: 01

Description: HMA Control Section

Sampling Interval: t0

6” Core Experimental AC

1’

2’

Figure 4. Sampling and testing plan for test section 01 to 03 at post-construction time 0 Oklahoma SPS-10

### Supplementary test sections

Table 9. Scope of material sampling for supplementary test sections

| Material and Sample Description | No. of Samples per section |  |  | Sample Location |
| --- | --- | --- | --- | --- |
|  | 61 | 62 | 63 |  |
| PRE-CONSTRUCTION SAMPLING |  |  |  |  |
| ORIGINAL HMAC |  |  |  |  |
| Bulk sample from the test pit (1’ x 1’) |  | 2 |  | A01062 |
| 6" Core |  | 3 |  | C19062-C21062 |
| UNBOUND BASE |  |  |  |  |
| 150 lb bulk sample |  | 1 |  | A01062 |
| SUBGRADE |  |  |  |  |
| 150 lb Bulk Sample and Moisture Content Jar |  | 1 |  | A01062 |
| DURING CONSTRUCTION SAMPLING |  |  |  |  |
| EXPERIMENTAL AC |  |  |  |  |
| 5-gallon buckets uncompacted AC mix | 15 | 15 | 15 | F010XX-F050XX |
| 5-gallon bucket combined aggregate |  |  | 20 | H01063 |
| 1-gallon can virgin AC Binder | 12 | 12 |  | H01061, H01062 |
| 1-gallon rejuvenator additive | 3 |  |  | H02061 |
| POST-CONSTRUCTION SAMPLING |  |  |  |  |
| EXPERIMENTAL AC |  |  |  |  |
| 6" Core | 15 | 15 | 15 | C01061-C15061, C01062-C15062, C01063-C15063 |

TABLE 10: Samples acquired from test section 40AA61

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| During Construction sampling |  |  |  |  |
| F01061 | 8 | BA01061 | S44F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02061 | 8 | BA02061 | S44F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03061 | 8 | BA03061 | S44F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04061 | 8 | BA04061 | S44F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05061 | 8 | BA05061 | S44F05L8 | 3 5-gallon buckets uncompacted AC mix |
| H01061 | 8 | BC01061 | S44H01L8 | 12 1-gallon can virgin AC Binder |
| H01061 | 8 | BC02061 | S44H02L8 | 12 1-gallon can virgin AC Binder |
| H01061 | 8 | BC03061 | S44H03L8 | 12 1-gallon can virgin AC Binder |
| H03061 | 8 | BW01061 | S44H04L8 | 3 1-gallon rejuvenator additive |
| Post-construction sampling |  |  |  |  |
| C01061 | 8 | CA01061 | S44C01L8 | 6" Core |
| C01061 | 6 | CA01061 | S44C01L6 | 6" Core |
| C01061 | 5 | CA01061 | S44C01L5 | 6" Core |
| C01061 | 4 | CA01061 | S44C01L4 | 6" Core |
| C01061 | 3 | CA01061 | S44C01L3 | 6" Core |
| C02061 | 8 | CA02061 | S44C02L8 | 6" Core |
| C02061 | 6 | CA02061 | S44C02L6 | 6" Core |
| C02061 | 5 | CA02061 | S44C02L5 | 6" Core |
| C02061 | 4 | CA02061 | S44C02L4 | 6" Core |
| C02061 | 3 | CA02061 | S44C02L3 | 6" Core |
| C03061 | 8 | CA03061 | S44C03L8 | 6" Core |
| C03061 | 6 | CA03061 | S44C03L6 | 6" Core |
| C03061 | 5 | CA03061 | S44C03L5 | 6" Core |
| C03061 | 4 | CA03061 | S44C03L4 | 6" Core |
| C03061 | 3 | CA03061 | S44C03L3 | 6" Core |
| C04061 | 8 | CA04061 | S44C04L8 | 6" Core |
| C04061 | 6 | CA04061 | S44C04L6 | 6" Core |
| C04061 | 5 | CA04061 | S44C04L5 | 6" Core |
| C04061 | 4 | CA04061 | S44C04L4 | 6" Core |
| C04061 | 3 | CA04061 | S44C04L3 | 6" Core |
| C05061 | 8 | CA05061 | S44C05L8 | 6" Core |
| C05061 | 6 | CA05061 | S44C05L6 | 6" Core |
| C05061 | 5 | CA05061 | S44C05L5 | 6" Core |
| C05061 | 4 | CA05061 | S44C05L4 | 6" Core |
| C05061 | 3 | CA05061 | S44C05L3 | 6" Core |
| C06061 | 8 | CA06061 | S44C06L8 | 6" Core |
| C07061 | 8 | CA07061 | S44C07L8 | 6" Core |
| C08061 | 8 | CA08061 | S44C08L8 | 6" Core |
| C09061 | 8 | CA09061 | S44C09L8 | 6" Core |
| C10061 | 8 | CA10061 | S44C10L8 | 6" Core |
| C11061 | 8 | CA11061 | S44C11L8 | 6" Core |
| C12061 | 8 | CA12061 | S44C12L8 | 6" Core |
| C13061 | 8 | CA13061 | S44C13L8 | 6" Core |
| C14061 | 8 | CA14061 | S44C14L8 | 6" Core |
| C15061 | 8 | CA15061 | S44C15L8 | 6" Core |

Table 11. Samples acquired from test section 40AA62

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| Pre-Construction Sampling |  |  |  |  |
| A01062 | 7 | BA01062 | S45A01L7 | Bulk sample from the test pit |
| A01062 | 6 | BA01062 | S45A01L6 | Bulk sample from the test pit |
| A01062 | 5 | BA01062 | S45A01L5 | Bulk sample from the test pit |
| A01062 | 2 | BG01062 | S45A01L2 | 150 lb Bulk Sample |
| A01062 | 1 | BS01062 | S45A01L1 | 150 lb Bulk Sample and Moisture Content Jar |
| A01062 | 1 | MS01062 | S45A01W1 | Moisture Content Jar |
| A02062 | 7 | BA02062 | S45A02L7 | Bulk sample from the test pit |
| A02062 | 6 | BA02062 | S45A02L6 | Bulk sample from the test pit |
| A02062 | 5 | BA02062 | S45A02L5 | Bulk sample from the test pit |
| C19062 | 7 | CA19062 | S45C19L7 | 6" Core |
| C19062 | 6 | CA19062 | S45C19L6 | 6" Core |
| C19062 | 5 | CA19062 | S45C19L5 | 6" Core |
| C19062 | 4 | CA19062 | S45C19L4 | 6" Core |
| C19062 | 3 | CA19062 | S45C19L3 | 6" Core |
| C20062 | 7 | CA20062 | S45C20L7 | 6" Core |
| C20062 | 6 | CA20062 | S45C20L6 | 6" Core |
| C20062 | 5 | CA20062 | S45C20L5 | 6" Core |
| C20062 | 4 | CA20062 | S45C20L4 | 6" Core |
| C20062 | 3 | CA20062 | S45C20L3 | 6" Core |
| C21062 | 7 | CA21062 | S45C21L7 | 6" Core |
| C21062 | 6 | CA21062 | S45C21L6 | 6" Core |
| C21062 | 5 | CA21062 | S45C21L5 | 6" Core |
| C21062 | 4 | CA21062 | S45C21L4 | 6" Core |
| C21062 | 3 | CA21062 | S45C21L3 | 6" Core |
| During Construction sampling |  |  |  |  |
| F01062 | 8 | BA03062 | S45F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02062 | 8 | BA04062 | S45F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03062 | 8 | BA05062 | S45F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04062 | 8 | BA06062 | S45F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05062 | 8 | BA07062 | S45F05L8 | 3 5-gallon buckets uncompacted AC mix |
| H01062 | 8 | BC01062 | S45H01L8 | 12 1-gallon can virgin AC Binder |
| H01062 | 8 | BC02062 | S45H02L8 | 12 1-gallon can virgin AC Binder |
| H01062 | 8 | BC03062 | S45H03L8 | 12 1-gallon can virgin AC Binder |
| Post-construction sampling |  |  |  |  |
| C01062 | 8 | CA01062 | S45C01L8 | 6" Core |
| C01062 | 6 | CA01062 | S45C01L6 | 6" Core |
| C01062 | 5 | CA01062 | S45C01L5 | 6" Core |
| C01062 | 4 | CA01062 | S45C01L4 | 6" Core |
| C01062 | 3 | CA01062 | S45C01L3 | 6" Core |
| C02062 | 8 | CA02062 | S45C02L8 | 6" Core |
| C02062 | 6 | CA02062 | S45C02L6 | 6" Core |
| C02062 | 5 | CA02062 | S45C02L5 | 6" Core |
| C02062 | 4 | CA02062 | S45C02L4 | 6" Core |
| C02062 | 3 | CA02062 | S45C02L3 | 6" Core |
| C03062 | 8 | CA03062 | S45C03L8 | 6" Core |
| C03062 | 6 | CA03062 | S45C03L6 | 6" Core |
| C03062 | 5 | CA03062 | S45C03L5 | 6" Core |
| C03062 | 4 | CA03062 | S45C03L4 | 6" Core |
| C03062 | 3 | CA03062 | S45C03L3 | 6" Core |
| C04062 | 8 | CA04062 | S45C04L8 | 6" Core |
| C04062 | 6 | CA04062 | S45C04L6 | 6" Core |
| C04062 | 5 | CA04062 | S45C04L5 | 6" Core |
| C04062 | 4 | CA04062 | S45C04L4 | 6" Core |
| C04062 | 3 | CA04062 | S45C04L3 | 6" Core |
| C05062 | 8 | CA05062 | S45C05L8 | 6" Core |
| C05062 | 6 | CA05062 | S45C05L6 | 6" Core |
| C05062 | 5 | CA05062 | S45C05L5 | 6" Core |
| C05062 | 4 | CA05062 | S45C05L4 | 6" Core |
| C05062 | 3 | CA05062 | S45C05L3 | 6" Core |
| C06062 | 8 | CA06062 | S45C06L8 | 6" Core |
| C06062 | 6 | CA06062 | S45C06L6 | 6" Core |
| C06062 | 5 | CA06062 | S45C06L5 | 6" Core |
| C06062 | 4 | CA06062 | S45C06L4 | 6" Core |
| C06062 | 3 | CA06062 | S45C06L3 | 6" Core |
| C07062 | 8 | CA07062 | S45C07L8 | 6" Core |
| C08062 | 8 | CA08062 | S45C08L8 | 6" Core |
| C08062 | 6 | CA08062 | S45C08L6 | 6" Core |
| C08062 | 5 | CA08062 | S45C08L5 | 6" Core |
| C08062 | 4 | CA08062 | S45C08L4 | 6" Core |
| C08062 | 3 | CA08062 | S45C08L3 | 6" Core |
| C09062 | 8 | CA09062 | S45C09L8 | 6" Core |
| C09062 | 6 | CA09062 | S45C09L6 | 6" Core |
| C09062 | 5 | CA09062 | S45C09L5 | 6" Core |
| C09062 | 4 | CA09062 | S45C09L4 | 6" Core |
| C09062 | 3 | CA09062 | S45C09L3 | 6" Core |
| C10062 | 8 | CA10062 | S45C10L8 | 6" Core |
| C10062 | 6 | CA10062 | S45C10L6 | 6" Core |
| C10062 | 5 | CA10062 | S45C10L5 | 6" Core |
| C10062 | 4 | CA10062 | S45C10L4 | 6" Core |
| C10062 | 3 | CA10062 | S45C10L3 | 6" Core |
| C11062 | 8 | CA11062 | S45C11L8 | 6" Core |
| C11062 | 6 | CA11062 | S45C11L6 | 6" Core |
| C11062 | 5 | CA11062 | S45C11L5 | 6" Core |
| C11062 | 4 | CA11062 | S45C11L4 | 6" Core |
| C11062 | 3 | CA11062 | S45C11L3 | 6" Core |
| C12062 | 8 | CA12062 | S45C12L8 | 6" Core |
| C12062 | 6 | CA12062 | S45C12L6 | 6" Core |
| C12062 | 5 | CA12062 | S45C12L5 | 6" Core |
| C12062 | 4 | CA12062 | S45C12L4 | 6" Core |
| C12062 | 3 | CA12062 | S45C12L3 | 6" Core |
| C13062 | 8 | CA13062 | S45C13L8 | 6" Core |
| C14062 | 8 | CA14062 | S45C14L8 | 6" Core |
| C14062 | 6 | CA14062 | S45C14L6 | 6" Core |
| C14062 | 5 | CA14062 | S45C14L5 | 6" Core |
| C14062 | 4 | CA14062 | S45C14L4 | 6" Core |
| C14062 | 3 | CA14062 | S45C14L3 | 6" Core |
| C15062 | 8 | CA15062 | S45C15L8 | 6" Core |
| C15062 | 6 | CA15062 | S45C15L6 | 6" Core |
| C15062 | 5 | CA15062 | S45C15L5 | 6" Core |
| C15062 | 4 | CA15062 | S45C15L4 | 6" Core |
| C15062 | 3 | CA15062 | S45C15L3 | 6" Core |

Table 12. Samples acquired from test section 40AA63

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| During Construction sampling |  |  |  |  |
| F01063 | 8 | BA01063 | S46F01L8 | 3 5-gallon buckets uncompacted AC mix |
| F02063 | 8 | BA02063 | S46F02L8 | 3 5-gallon buckets uncompacted AC mix |
| F03063 | 8 | BA03063 | S46F03L8 | 3 5-gallon buckets uncompacted AC mix |
| F04063 | 8 | BA04063 | S46F04L8 | 3 5-gallon buckets uncompacted AC mix |
| F05063 | 8 | BA05063 | S46F05L8 | 3 5-gallon buckets uncompacted AC mix |
| H01063 | 8 | BU01063 | S46H01L8 | 20 5-gallon bucket combined aggregate |
| Post-construction sampling |  |  |  |  |
| C01063 | 8 | CA01063 | S46C01L8 | 6" Core |
| C01063 | 6 | CA01063 | S46C01L6 | 6" Core |
| C01063 | 5 | CA01063 | S46C01L5 | 6" Core |
| C01063 | 4 | CA01063 | S46C01L4 | 6" Core |
| C01063 | 3 | CA01063 | S46C01L3 | 6" Core |
| C02063 | 8 | CA02063 | S46C02L8 | 6" Core |
| C02063 | 6 | CA02063 | S46C02L6 | 6" Core |
| C02063 | 5 | CA02063 | S46C02L5 | 6" Core |
| C02063 | 4 | CA02063 | S46C02L4 | 6" Core |
| C02063 | 3 | CA02063 | S46C02L3 | 6" Core |
| C03063 | 8 | CA03063 | S46C03L8 | 6" Core |
| C03063 | 6 | CA03063 | S46C03L6 | 6" Core |
| C03063 | 5 | CA03063 | S46C03L5 | 6" Core |
| C03063 | 4 | CA03063 | S46C03L4 | 6" Core |
| C03063 | 3 | CA03063 | S46C03L3 | 6" Core |
| C04063 | 8 | CA04063 | S46C04L8 | 6" Core |
| C04063 | 6 | CA04063 | S46C04L6 | 6" Core |
| C04063 | 5 | CA04063 | S46C04L5 | 6" Core |
| C04063 | 4 | CA04063 | S46C04L4 | 6" Core |
| C04063 | 3 | CA04063 | S46C04L3 | 6" Core |
| C05063 | 8 | CA05063 | S46C05L8 | 6" Core |
| C05063 | 6 | CA05063 | S46C05L6 | 6" Core |
| C05063 | 5 | CA05063 | S46C05L5 | 6" Core |
| C05063 | 4 | CA05063 | S46C05L4 | 6" Core |
| C05063 | 3 | CA05063 | S46C05L3 | 6" Core |
| C06063 | 8 | CA06063 | S46C06L8 | 6" Core |
| C06063 | 6 | CA06063 | S46C06L6 | 6" Core |
| C06063 | 5 | CA06063 | S46C06L5 | 6" Core |
| C06063 | 4 | CA06063 | S46C06L4 | 6" Core |
| C06063 | 3 | CA06063 | S46C06L3 | 6" Core |
| C07063 | 8 | CA07063 | S46C07L8 | 6" Core |
| C07063 | 6 | CA07063 | S46C07L6 | 6" Core |
| C07063 | 5 | CA07063 | S46C07L5 | 6" Core |
| C07063 | 4 | CA07063 | S46C07L4 | 6" Core |
| C07063 | 3 | CA07063 | S46C07L3 | 6" Core |
| C08063 | 8 | CA08063 | S46C08L8 | 6" Core |
| C08063 | 6 | CA08063 | S46C08L6 | 6" Core |
| C08063 | 5 | CA08063 | S46C08L5 | 6" Core |
| C08063 | 4 | CA08063 | S46C08L4 | 6" Core |
| C08063 | 3 | CA08063 | S46C08L3 | 6" Core |
| C09063 | 8 | CA09063 | S46C09L8 | 6" Core |
| C09063 | 6 | CA09063 | S46C09L6 | 6" Core |
| C09063 | 5 | CA09063 | S46C09L5 | 6" Core |
| C09063 | 4 | CA09063 | S46C09L4 | 6" Core |
| C09063 | 3 | CA09063 | S46C09L3 | 6" Core |
| C10063 | 8 | CA10063 | S46C10L8 | 6" Core |
| C10063 | 6 | CA10063 | S46C10L6 | 6" Core |
| C10063 | 5 | CA10063 | S46C10L5 | 6" Core |
| C10063 | 4 | CA10063 | S46C10L4 | 6" Core |
| C10063 | 3 | CA10063 | S46C10L3 | 6" Core |
| C11063 | 8 | CA11063 | S46C11L8 | 6" Core |
| C11063 | 6 | CA11063 | S46C11L6 | 6" Core |
| C11063 | 5 | CA11063 | S46C11L5 | 6" Core |
| C11063 | 4 | CA11063 | S46C11L4 | 6" Core |
| C11063 | 3 | CA11063 | S46C11L3 | 6" Core |
| C12063 | 8 | CA12063 | S46C12L8 | 6" Core |
| C12063 | 6 | CA12063 | S46C12L6 | 6" Core |
| C12063 | 5 | CA12063 | S46C12L5 | 6" Core |
| C12063 | 4 | CA12063 | S46C12L4 | 6" Core |
| C12063 | 3 | CA12063 | S46C12L3 | 6" Core |
| C13063 | 8 | CA13063 | S46C13L8 | 6" Core |
| C14063 | 8 | CA14063 | S46C14L8 | 6" Core |
| C14063 | 6 | CA14063 | S46C14L6 | 6" Core |
| C14063 | 5 | CA14063 | S46C14L5 | 6" Core |
| C14063 | 4 | CA14063 | S46C14L4 | 6" Core |
| C14063 | 3 | CA14063 | S46C14L3 | 6" Core |
| C15063 | 8 | CA15063 | S46C15L8 | 6" Core |
| C15063 | 6 | CA15063 | S46C15L6 | 6" Core |
| C15063 | 5 | CA15063 | S46C15L5 | 6" Core |
| C15063 | 4 | CA15063 | S46C15L4 | 6" Core |
| C15063 | 3 | CA15063 | S46C15L3 | 6" Core |
| C16063 | 4 | CA16063 | S46C16L04 | 6" Core (not found in PPDB sheet 2) |
| C17063 | 4 | CA17063 | S46C17L04 | 6" Core (not found in PPDB sheet 2) |
| C18063 | 4 | CA18063 | S46C18L04 | 6" Core (not found in PPDB sheet 2) |

A02062

C21062

C20062

0-70

0-50

Section STA:

Section: 400A0X

Description: HMAC Control Section

Sampling Area: 01

Sampling Interval: t0

Shoulder

C19062

18’

4’

2’

1’

A01062

Section: 01

Description: HMA Control Section

Sampling Interval: t0

6’

6” Core Original HMAC

6” Core original HMAC; DCP

Sample Original HMAC;

Bulk Base and Subgrade

1’

2’

Figure 5. Sampling and testing plan for test section 62 at pre-construction Oklahoma SPS-10

2’

0-70

0-50

Section STA:

Section: 400A0X

Description: HMAC Control Section

Sampling Area: 01

Sampling Interval: t0

Shoulder

C0206X

C0406X

C0606X

C0800X

C1006X

C1206X

C1406X

2’

1’

3’

2’

2’

2’

2’

2’

2’

2’

C0106X

C0306X

C0506X

C0706X

C0906X

C1106X

C1306X

C1506X

3’

2’

2’

2’

2’

2’

2’

Section: 01

Description: HMA Control Section

Sampling Interval: t0

6” Core Experimental AC

1’

Figure 6. Sampling and testing plan for test section 61 to 63 at post-construction time 0 Oklahoma SPS-10

### Samples acquired from plant

Table 13. Samples acquired from plant

| Sample Location | Layer Number | Sample No. | Specimen ID | Type of Sample |
| --- | --- | --- | --- | --- |
| H01001 | 8 | BC01001 | S41H01L8 | 1 1-gallon can virgin AC binder |
| H01001 | 8 | BC02001 | S41H02L8 | 1 1-gallon can virgin AC binder |
| H01001 | 8 | BC03001 | S41H03L8 | 10 1-gallon can virgin AC binder |
| H01061 | 8 | BC01061 | S44H01L8 | 1 1-gallon can virgin AC binder |
| H01061 | 8 | BC02061 | S44H02L8 | 1 1-gallon can virgin AC binder |
| H01061 | 8 | BC03061 | S44H03L8 | 10 1-gallon can virgin AC binder |
| H01062 | 8 | BC01062 | S45H01L8 | 1 1-gallon can virgin AC binder |
| H01062 | 8 | BC02062 | S45H02L8 | 1 1-gallon can virgin AC binder |
| H01062 | 8 | BC03062 | S45H03L8 | 10 1-gallon can virgin AC binder |
| H02001 | 8 | BU01001 | S41H04L8 | 20 5-gallon buckets combined aggregate |
| H01063 | 8 | BU01063 | S46H01L8 | 20 5-gallon buckets combined aggregate |
| H03001 | 8 | BR01001 | S41H05L8 | 50 lb Bulk Sample RAP |
| H03001 | 8 | BR02001 | S41H06L8 | 10 5-gallon buckets RAP |
| H04001 | 8 | BR03001 | S41H07L8 | 50 lb Bulk Sample RAS |
| H04001 | 8 | BR04001 | S41H08L8 | 10 5-gallon buckets RAS |
| H01003 | 8 | BW01003 | S43H01L8 | 3 gallons chemical additive in plastic bottles, with MSDS |
| H03061 | 8 | BW01061 | S44H04L8 | 3 gallons Rejuventor additive in plastic bottles, with MSDS |

## Field Testing

### Core test sections

Table 14. Scope of field testing in core test sections

| Material and Test Description |  | No. of Tests | Location Designation |
| --- | --- | --- | --- |
|  | PRE-CONSTRUCTION TESTING |  |  |
| A. | ORIGINAL HMAC |  |  |
|  | Elevation Survey | 3 | All Sections |
|  | Core Examination and Thickness (P01/AC01) | 6 | C19001-C21001, C19003-C21003 |
| B. | UNBOUND BASE |  |  |
|  | DCP | 2 | C19001, C19003 |
| C. | SUBGRADE |  |  |
|  | DCP | 2 | C19001, C19003 |
|  | POST-CONSTRUCTION TESTING |  |  |
| A. | EXPERIMENTAL AC |  |  |
|  | Elevation Survey | 3 | All Sections |
|  | In situ density measurement | 9 | T01001-T03001, T01002-T03002, T01003-T03003 |
|  | Core Examination and Thickness (P01/AC01) | 45 | C01001-C15001, C01002-C15002, C01003-C15003 |

Section 40AA01 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01001

T02001

T03001

Traffic

FIGURE 7:

Figure 7. Density measurement layout for test section 40AA01 Oklahoma SPS-10

Section 40AA02 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01002

T02002

T03002

Traffic

Figure 8. Density measurement layout for test section 40AA02 Oklahoma SPS-10

Section 40AA03 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01003

T02003

T03003

Traffic

Figure 9. Density measurement layout for test section 40AA03 Oklahoma SPS-10

### Supplementary sections

Table 15. Scope of field testing for the supplementary test sections

| Material and Test Description |  | No. of Tests | Location Designation |
| --- | --- | --- | --- |
|  | PRE-CONSTRUCTION TESTING |  |  |
| A. | ORIGINAL HMAC |  |  |
|  | Elevation Survey | 3 | All Sections |
|  | Core Examination and Thickness (P01/AC01) | 3 | C19062-C21062 |
| B. | UNBOUND BASE |  |  |
|  | DCP | 1 | C19062 |
| C. | SUBGRADE |  |  |
|  | DCP | 1 | C19062 |
|  | POST-CONSTRUCTION TESTING |  |  |
| A. | EXPERIMENTAL AC |  |  |
|  | Elevation Survey | 3 | All Sections |
|  | In situ density measurement | 9 | T01061-T03061, T01062-T03062, T01063-T03063 |
|  | Core Examination and Thickness (P01/AC01) | 45 | C01061-C15061, C01062-C15062, C01063-C15063 |

Section 40AA61 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01061

T02061

T03061

Traffic

Figure 10. Density measurement layout for test section 40AA61 Oklahoma SPS-10

Section 40AA62 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01062

T02062

T03062

Traffic

Figure 11. Density measurement layout for test section 40AA62 Oklahoma SPS-10

Section 40AA63 - Monitoring Area

6 ft

6 ft

167 ft

83 ft

167 ft

83 ft

T01063

T02063

T03063

Traffic

Figure 12. Density measurement layout for test section 40AA63 Oklahoma SPS-10

## Samples to be shipped

Table 16. Samples to be shipped to central lab testing contractor laboratory

| Specimen ID | Type of Sample |
| --- | --- |
| Experimental AC |  |
| S41C01L8 | 6" Core |
| S41C02L8 | 6" Core |
| S41C03L8 | 6" Core |
| S41C04L8 | 6" Core |
| S41C05L8 | 6" Core |
| S41C06L8 | 6" Core |
| S41C07L8 | 6" Core |
| S41C08L8 | 6" Core |
| S41C09L8 | 6" Core |
| S41C10L8 | 6" Core |
| S41C11L8 | 6" Core |
| S42C01L8 | 6" Core |
| S42C02L8 | 6" Core |
| S42C03L8 | 6" Core |
| S42C04L8 | 6" Core |
| S42C05L8 | 6" Core |
| S42C06L8 | 6" Core |
| S42C07L8 | 6" Core |
| S42C08L8 | 6" Core |
| S42C09L8 | 6" Core |
| S42C10L8 | 6" Core |
| S42C11L8 | 6" Core |
| S43C01L8 | 6" Core |
| S43C02L8 | 6" Core |
| S43C03L8 | 6" Core |
| S43C04L8 | 6" Core |
| S43C05L8 | 6" Core |
| S43C06L8 | 6" Core |
| S43C07L8 | 6" Core |
| S43C08Le8 | 6" Core |
| S43C09L8 | 6" Core |
| S43C10L8 | 6" Core |
| S43C11L8 | 6" Core |
| S44C01L8 | 6" Core |
| S44C02L8 | 6" Core |
| S44C03L8 | 6" Core |
| S44C04L8 | 6" Core |
| S44C05L8 | 6" Core |
| S44C06L8 | 6" Core |
| S44C07L8 | 6" Core |
| S44C08L8 | 6" Core |
| S44C09L8 | 6" Core |
| S44C10L8 | 6" Core |
| S44C11L8 | 6" Core |
| S45C01L8 | 6" Core |
| S45C02L8 | 6" Core |
| S45C03L8 | 6" Core |
| S45C04L8 | 6" Core |
| S45C05L8 | 6" Core |
| S45C06L8 | 6" Core |
| S45C07L8 | 6" Core |
| S45C08L8 | 6" Core |
| S45C09L8 | 6" Core |
| S45C10L8 | 6" Core |
| S45C11L8 | 6" Core |
| S46C01L8 | 6" Core |
| S46C02L8 | 6" Core |
| S46C03L8 | 6" Core |
| S46C04L8 | 6" Core |
| S46C05L8 | 6" Core |
| S46C06L8 | 6" Core |
| S46C07L8 | 6" Core |
| S46C08L8 | 6" Core |
| S46C09L8 | 6" Core |
| S46C10Le8 | 6" Core |
| S46C11L8 | 6" Core |
| Experimental AC |  |
| S41H01L8 | 1 1-gallon can of virgin AC binder |
| S44H01L8 | 1 1-gallon can of virgin AC binder |
| S45H01Le8 | 1 1-gallon can of virgin AC binder |
| S41H05L8 | 50 lb Bulk Sample RAP |
| S41H07L8 | 50 lb Bulk Sample RAS |
| Original HMAC |  |
| S41A01L5-7 | Bulk sample from the pit |
| S41A02L5-7 | Bulk sample from the pit |
| S43A01L5-7 | Bulk sample from the pit |
| S43A02L5-7 | Bulk sample from the pit |
| S45A01L5-7 | Bulk sample from the pit |
| S45A02L5-7 | Bulk sample from the pit |
| S41C19L5-7 | 6" Core |
| S41C20L5-7 | 6" Core |
| S41C21L5-7 | 6" Core |
| S43C19L5-7 | 6" Core |
| S43C20L5-7 | 6" Core |
| S43C21L5-7 | 6" Core |
| S45C19L5-7 | 6" Core |
| S45C20L5-7 | 6" Core |
| S45C21L5-7 | 6" Core |
| Unbound Base |  |
| S41A01L2 | 150 lb Bulk Sample |
| S43A01L2 | 150 lb Bulk Sample |
| S45A01L2 | 150 lb Bulk Sample |
| Subgrade |  |
| S41A01L1 | 150 lb Bulk Sample and Moisture Content Jar |
| S43A01L1 | 150 lb Bulk Sample and Moisture Content Jar |
| S45A01L1 | 150 lb Bulk Sample and Moisture Content Jar |

Table 17. Samples to be shipped to the LTPP MRL

| Specimen ID | Type of Sample |
| --- | --- |
| S41C12L3-8, no 7 | 6" Core |
| S41C13L3-8, no 7 | 6" Core |
| S41C14L3-8, no 7 | 6" Core |
| S41C15L8 | 6" Core |
| S42C12L3-8, no 7 | 6" Core |
| S42C13L3-8, no 7 | 6" Core |
| S42C14L3-8, no 7 | 6" Core |
| S42C15L3-8, no 7 | 6" Core |
| S43C12L3-8, no 7 | 6" Core |
| S43C13L3-8, no 7 | 6" Core |
| S43C14L3-8, no 7 | 6" Core |
| e, no 7 | 6" Core |
| S44C12L8 | 6" Core |
| S44C13L8 | 6" Core |
| S44C14L8 | 6" Core |
| S44C15L8 | 6" Core |
| S45C12L3-8, no 7 | 6" Core |
| S45C13L8 | 6" Core |
| S45C14L3-8, no 7 | 6" Core |
| S45C15L3-8, no 7 | 6" Core |
| S46C12L3-8, no 7 | 6" Core |
| S46C13L8 | 6" Core |
| S46C14L3-8, no 7 | 6" Core |
| S46C15L3-8, no 7 | 6" Core |
| S41H03L8 | 10 1-gallon cans of virgin AC binder |
| S44H03L8 | 10 1-gallon cans of virgin AC binder |
| S45H03L8 | 10 1-gallon cans of virgin AC binder |
| S41F01L8-S41F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S42F01L8-S42F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S43F01L8-S43F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S44F01L8-S44F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S45F01L8-S45F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S46F01L8-S46F05L8 | 15 5-gallon buckets uncompacted AC mix |
| S41H04L8 | 20 5-gallon buckets combined aggregate |
| S46H01L8 | 20 5-gallon buckets combined aggregate |
| S41H06L8 | 10 5-gallon buckets RAP |
| S41H08L8 | 10 5-gallon buckets RAS |
| S43H01L8 | 3 gallons chemical additive in plastic bottles, with MSDS |
| S44H04L8 | 3 gallons chemical additive 1 in plastic bottles, with MSDS |

Table 18. Samples to be shipped to the Turner Fairbanks Research Center

| Specimen ID | Type of Sample |
| --- | --- |
| Experimental AC Layer |  |
| S41H02L8 | 1 1-gallon can of virgin AC binder |
| S44H02L8 | 1 1-gallon can of virgin AC binder |
| S45H02L8 | 1 1-gallon can of virgin AC binder |

## Note on change in Specimen ID in MST plan:

All the samples that were collected initially assuming the experimental AC layer as designated as Layer Number 04 will be changed to Layer Number 08.  From here on, the top most experimental layer is taken as layer number 08. This causes change in all the above Specimen-ID from relating to samples from layer 04, with the last digit changing from 04 to 08. This was because during sampling of the material, the top AC layer was assumed as layer number 04, being the fourth layer, which later was found to be in fact the eighth layer for that site. It was also mainly because of lack of previous information about the historical maintenance and rehabilitation thicknesses. However, the issue was dealt by measuring the core thicknesses obtained from the sites during various sampling and coring operation related to LTPP.

# LABORATORY MATERIAL TESTING

It is the intent of this section of the sampling and testing plan to provide an outline for the laboratory testing that is planned for the Oklahoma SPS-10 project.  The previous section ended with lists of samples to be shipped to each of the two laboratories, the central lab testing contractor laboratory and the Turner Fairbanks Research center, as well as Material Reference Library.  In this section, the tests to be performed on each sample are listed.

Table 19 provides listings of laboratory tests to be performed for each material type and pavement layer as well as the associated laboratory testing protocol. It is imperative that the protocols listed be strictly followed during testing.

Tables 20 and 21 provide new sample numbers and specimen ID’s for samples that will either be combined or modified in the laboratory.  The provided sample numbers and specimen ID’s should be used in order to ensure proper tracking using the Materials Tracking System (MTS).

Tables 22 through 26 provide tracking tables for the testing to be conducted for each material type.  These tables itemize the testing to occur on each sample and provide indication of how to process a sample after a particular test is complete.

## Testing Plans

Table 19. SPS-10 laboratory testing plans

| Material Type and Properties |  | LTPP Designation | LTPP Protocol | No. of Tests per Layer | Test Conducted By: |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CLTC | TFHRC |
| A. | EXPERIMENTAL AC |  |  |  |  |  |
|  | Core Exam. and Thickness | AC01 | P01 | 66 | RSC |  |
|  | Bulk Specific Gravity | AC02 | P02 | 60 | X |  |
|  | Maximum Specific Gravity | AC03 | P03 | 6 | X |  |
|  | Asphalt Content (Extracted) | AC04 | P04 | 8 | X |  |
|  | Dynamic Modulus | AC08 | P74 | 18 | X |  |
|  | Hamburg Wheel Track | AC09 | P75 | 18 | X |  |
|  | Fenix Fracture Energy | AC10 | P79 | 24 | X |  |
|  | Specific Gravity at 60 ° F | AE03 | P23 | 8 | X |  |
|  | Dynamic Shear Rheometer (DSR) Test | AE07 | P27 | 23 | X |  |
|  | Bending Beam Rheometer | AE08 | P28 | 11 | X |  |
|  | Multiple Stress Creep Recovery (MSCR) | AE10 | P73 | 11 | X |  |
|  | Gradation of Aggregate | AG04 | P14 | 8 | X |  |
|  | PPA and Motor Oil Test | AE11 | P76 | 3 |  | X |
| B. | ORIGINAL HMAC |  |  |  |  |  |
|  | Core Exam. and Thickness | AC01 | P01 | 15 | RSC |  |
|  | Bulk Specific Gravity | AC02 | P02 | 9 | X |  |
|  | Maximum Specific Gravity | AC03 | P03 | 3 | X |  |
|  | Asphalt Content (Extracted) | AC04 | P04 | 3 | X |  |
|  | Dynamic Modulus | AC08 | P74 | 9 | X |  |
|  | Dynamic Shear Rheometer (DSR) Test | AE07 | P27 | 3 | X |  |
|  | Bending Beam Rheometer | AE08 | P28 | 3 | X |  |
|  | Multiple Stress Creep Recovery (MSCR) | AE10 | P73 | 3 | X |  |
|  | Gradation of Aggregate | AG04 | P14 | 3 | X |  |
| C. | FLEXIBLE BASE |  |  |  |  |  |
|  | Sieve Analysis | UG01* | P41 | 3 | X |  |
|  | Standard Proctor | UG05 | P44 | 3 | X |  |
|  | Atterberg Limits | UG04 | P43 | 3 | X |  |
|  | Resilient Modulus | UG07 | P46 | 3 | X |  |
|  | Classification and Description | UG08 | P47 | 3 | X |  |
| D. | SUBGRADE |  |  |  |  |  |
|  | Sieve Analysis | SS01 | P51 | 3 | X |  |
|  | Atterberg Limits | SS03 | P43 | 3 | X |  |
|  | Classification and Description | SS04 | P52 | 3 | X |  |
|  | Standard Proctor | SS05 | P55 | 3 | X |  |
|  | Resilient Modulus | SS07 | P46 | 3 | X |  |
|  | Natural Moisture Content | SS09 | P49 | 3 | X |  |

* SPS-10 Sampling and Testing guide suggests UG03 which does not exist in LTPP IMS guide.

Table 20. Original and combined specimen descriptions

1 Sample No.:  After combination, the third character is changed to “X,” and the fourth character is assigned numerically.  The other five characters follow the original format.

Specimen ID:  After combination, “L” is changed to “M.”

| Description Type | Status |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Original |  |  |  |  |  |  |  | Combined1 |
| Sample No. | BA01001 | BA02001 | <-not in T01B forms | - | - | - | - | - | BAX1001 |
| Specimen ID | S41A01L5-7 | S41A02L5-7 | - | - | - | - | - | - | S41A01M03 |
| PRE CONSTRUCTION SAMPLING (BEFORE EXPERIMENTAL LAYER WAS PLACED) |  |  |  |  |  |  |  |  |  |
| Sample No. | BA01003 | BA02003 | - | - | - | - | - | - | BAX1003 |
| Specimen ID | S43A01L7 | S43A02L7 | - | - | - | - | - | - | S43A01M7 |
| Sample No. | BA01003 | BA02003 | - | - | - | - | - | - | BAX1003 |
| Specimen ID | S43A01L6 | S43A02L6 | - | - | - | - | - | - | S43A01M6 |
| Sample No. | BA01003 | BA02003 | - | - | - | - | - | - | BAX1003 |
| Specimen ID | S43A01L5 | S43A02L5 | - | - | - | - | - | - | S43A01M5 |
| Sample No. | e | BA02062 | - | - | - | - | - | - | BAX1062 |
| Specimen ID | S45A01L5-7 | S45A02L5-7 | - | - | - | - | - | - | S45A01M5-7 |
| Sample No. | CA01001 | CA02001 | CA03001 | CA04001 | CA05001 | CA06001 | CA07001 | CA11001 | CAX1001 |
| Specimen ID | S41C01L8 | S41C02L8 | S41C03L8 | S41C04L8 | S41C05L8 | S41C06L8 | S41C07L8 | S41C11L8 | S41C01M8 |
| Sample No. | CA01002 | CA02002 | CA03002 | CA04002 | CA05002 | CA06002 | CA07002 | CA11002 | CAX1002 |
| Specimen ID | S42C01L8 | S42C02L8 | S42C03L8 | S42C04L8 | S42C05L8 | S42C06L8 | S42C07L8 | S42C11L8 | S42C01M8 |
| Sample No. | CA01003 | CA02003 | CA03003 | CA04003 | CA05003 | CA06003 | CA07003 | CA11003 | CAX1003 |
| Specimen ID | S43C01L8 | S43C02L8 | S43C03L8 | S43C04L8 | S43C05L8 | S43C06L8 | S43C07L8 | S43C11L8 | S43C01M8 |
| Sample No. | CA01061 | CA02061 | CA03061 | CA04061 | CA05061 | CA06061 | CA07061 | CA11061 | CAX1061 |
| Specimen ID | S44C01L8 | S44C02L8 | S44C03L8 | S44C04L8 | S44C05L8 | S44C06L8 | S44C07L8 | S44C11L8 | S44C01M8 |
| Sample No. | CA01062 | CA02062 | CA03062 | CA04062 | CA05062 | CA06062 | CA07062 | CA11062 | CAX1062 |
| Specimen ID | S45C01L8 | S45C02L8 | S45C03L8 | S45C04L8 | S45C05L8 | S45C06L8 | S45C07L8 | S45C11L8 | S45C01M8 |
| Sample No. | CA01063 | CA02063 | CA03063 | CA04063 | CA05063 | CA06063 | CA07063 | CA11063 | CAX1063 |
| Specimen ID | S46C01L8 | S46C02L8 | S46C03L8 | S46C04L8 | S46C05L8 | S46C06L8 | S46C07L8 | S46C11L8 | S46C01M8 |

Table 21. Original and modified specimen descriptions

| Process | Description Type | Status |  |
| --- | --- | --- | --- |
|  |  | Original | Modified1 |
| Extraction | Sample No. | BAX1001 | BAE1001 |
|  | Specimen ID | S41A01M03 | S41A01A03 |
|  | Sample No. | BAX1003 | BAE1003 |
|  | Specimen ID | S43A01M5-7 | S43A01A5-7 |
|  | Sample No. | BAX1062 | BAE1062 |
|  | Specimen ID | S45A01M5-7 | S45A01A5-7 |
|  | Sample No. | CAX1001 | CAE1001 |
|  | Specimen ID | S41C01Me | S41C01Ae |
|  | Sample No. | CAX1002 | CAE1002 |
|  | Specimen ID | S42C01M8 | S42C01A8 |
|  | Sample No. | CAX1003 | CAE1003 |
|  | Specimen ID | S43C01M8 | S43C01A8 |
|  | Sample No. | CAX1061 | CAE1061 |
|  | Specimen ID | S44C01M8 | S44C01A8 |
|  | Sample No. | CAX1062 | CAE1062 |
|  | Specimen ID | S45C01M8 | S45C01A8 |
|  | Sample No. | CAX1063 | CAE1063 |
|  | Specimen ID | S46C01M8 | S46C01A8 |
|  | Sample No. | BR01001 | BRE1001 |
|  | Specimen ID | S41H05L8 | S41H05M8 |
|  | Sample No. | BR03001 | BRE3001 |
|  | Specimen ID | S41H07L8 | S41H07M8 |
| PAV | Sample No. | CAE1001 | CAP1001 |
|  | Specimen ID | S41C01A8 | S41C01B8 |
|  | Sample No. | CAE1002 | CAP1002 |
|  | Specimen ID | S42C01A8 | S42C01B8 |
|  | Sample No. | CAE1003 | CAP1003 |
|  | Specimen ID | S43C01A8 | S43C01B8 |
|  | Sample No. | CAE1061 | CAP1061 |
|  | Specimen ID | S44C01A8 | S44C01B8 |
|  | Sample No. | CAE1062 | CAP1062 |
|  | Specimen ID | S45C01A8 | S45C01B8 |
|  | Sample No. | CAE1063 | CAP1063 |
|  | Specimen ID | S46C01A8 | S46C01B8 |
| RTFO | Sample No. | BC01001 | BCR1001 |
|  | Specimen ID | S41H01L8 | S41H01M8 |
|  | Sample No. | BC01061 | BCR1061 |
|  | Specimen ID | S44H01L8 | S44H01M8 |
|  | Sample No. | BC01062 | BCR1062 |
|  | Specimen ID | S45H01L8 | S45H01M8 |
| RTFO + PAV | Sample No. | BCR1001 | BCZ1001 |
|  | Specimen ID | S41H01M8 | S41H01A8 |
|  | Sample No. | BCR1061 | BCZ1061 |
|  | Specimen ID | S44H01M8 | S44H01A8 |
|  | Sample No. | BCR1062 | BCZ1062 |
|  | Specimen ID | S45H01M8 | S45H01A8 |

1. Sample No.:  After modification, the third character is changed to “E” for extracted, “P” for PAV, “R” for RTFO or “Z” for RTFO+PAV.  The fourth character is assigned numerically.

Specimen ID:  After modification, “L” is changed to “M.”  If the sample has already been combined or modified, the letter is advanced alphabetically beginning with “A.”

All the Specimen with layer number 3 as identified in the field will be changed to 3 additional layers. Layer 5, 6 and 7 having 3 Specimen IDs instead of just 1. For example, Hole S41A01 will have Specimen IDs as S41A01L5, S41A01L6, S41A01L7. The tests as assigned will be conducted 3 times instead of 1 time.

COMMENT FOR MTS:

There is no option for RTFO+PAV test at once, so there needs to be a specimen defined for between RTFO and PAV.  The specimen will have the modified sample number with third character changed to “Y” after RTFO and “Z” as shown in above table after PAV. The specimen ID will have “L” replaced by “Y” after RTFO and will be replaced by “A” after PAV.

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S41C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S41C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S41C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S41C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S41C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S41C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S41C11L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S41C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |
| S42C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S42C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S42C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S42C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S42C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S42C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S42C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S42C11L8 | P01/AC01 |  |  |  |  | No | C |
| S42C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |
| S43C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S43C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S43C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S43C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S43C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S43C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S43C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S43C11L8 | P01/AC01 |  |  |  |  | No | C |
| S43C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |

## Tracking Tables

Table 22. Tracking table of experimental asphalt concrete in the central lab testing contractor laboratory

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S44C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S44C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S44C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S44C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S44C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S44C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S44C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S44C11L8 | P01/AC01 |  |  |  |  | No | C |
| S44C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |
| S45C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S45C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S45C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S45C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S45C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S45C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S45C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S45C11L8 | P01/AC01 |  |  |  |  | No | C |
| S45C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |
| S46C01L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S46C02L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S46C03L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | No | C |
| S46C04L8 | P01/AC01 | P02/AC02 | P79/AC10 |  |  | Yes | C |
| S46C05L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S46C06L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | C |
| S46C07L8 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | C |
| S46C11L8 | P01/AC01 |  |  |  |  | No | C |
| S46C01M8 | P03/AC03 | P04/AC04 |  |  |  | No | E |

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S41C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S41C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S42C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S42C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S42C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S43C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S43C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S43C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S44C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S44C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S44C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S45C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S45C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S45C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S46C08L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S46C09L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | No | D |
| S46C10L8 | P01/AC01 | P02/AC02 | P75/AC09 |  |  | Yes | S |
| S41C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S42C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S43C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S44C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S45C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S46C01A8 | P23/AE03 | P27/AE07 | P73/AE10 | P14/AG04 |  | No | S |
| S41C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S42C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S43C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S44C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S45C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S46C01B8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S41H01L8 | P23/AE03 | P27/AE07 |  |  |  | No | D |
| S41H01M8 | P27/AE07 | P73/AE10 |  |  |  | No | S |
| S41H01A8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S44H01L8 | P23/AE03 | P27/AE07 |  |  |  | No | D |
| S44H01M8 | P27/AE07 | P73/AE10 |  |  |  | No | S |
| S44H01A8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S45H01L8 | P23/AE03 | P27/AE07 |  |  |  | No | D |
| S45H01M8 | P27/AE07 | P73/AE10 |  |  |  | No | S |
| S45H01A8 | P27/AE07 | P28/AE08 |  |  |  | No | S |
| S41H05L8 | P04/AC04 |  |  |  |  | Yes | S |
| S41H07L8 | P04/AC04 |  |  |  |  | Yes | S |
| S41H05M8 | P23/AE03 | P27/AE07 | P28/AE08 | P73/AE10 | P14/AG04 | Yes | S |
| S41H07M8 | P23/AE03 | P27/AE07 | P28/AE08 | P73/AE10 | P14/AG04 | Yes | S |

Table 23. Tracking table of original hot mix asphalt concrete in the central lab testing contractor laboratory

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41C19L5-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | D |
| S41C20L5-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S41C21L5-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S43C19L3-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | D |
| S43C20L3-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S43C21L3-7 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S45C19L3 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | No | D |
| S45C20L3 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S45C21L3 | P01/AC01 | P02/AC02 | P74/AC08 |  |  | Yes | S |
| S41A01L3 | P01/AC01 |  |  |  |  | No | C |
| S41A02L3 | P01/AC01 |  |  |  |  | No | C |
| S43A01L3 | P01/AC01 |  |  |  |  | No | C |
| S43A02L3 | P01/AC01 |  |  |  |  | No | C |
| S45A01L3 | P01/AC01 |  |  |  |  | No | C |
| S45A02L3 | P01/AC01 |  |  |  |  | No | C |
| S41A01M3 | P03/AC03 | P04/AC04 |  |  |  | Yes | E+S |
| S43A01M5-7 | P03/AC03 | P04/AC04 |  |  |  | Yes | E+S |
| S45A01M5-7 | P03/AC03 | P04/AC04 |  |  |  | Yes | E+S |
| S41A01A3 | P27/AE07 | P28/AE08 | P73/AE10 | P14/AG04 |  | Yes | S |
| S43A01A3 | P27/AE07 | P28/AE08 | P73/AE10 | P14/AG04 |  | Yes | S |
| S45A01A3 | P27/AE07 | P28/AE08 | P73/AE10 | P14/AG04 |  | Yes | S |

Table 24. Tracking table of unbound base in the central lab testing contractor

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41A01L2 | P41/UG01 | P47/UG08 | P43/UG04 | P46/UG07 | P44/UG05 | Yes | S |
| S43A01L2 | P41/UG01 | P47/UG08 | P43/UG04 | P46/UG07 | P44/UG05 | Yes | S |
| S45A01L2 | P41/UG01 | P47/UG08 | P43/UG04 | P46/UG07 | P44/UG05 | Yes | S |

The moisture content sample may be disposed after testing.  However, any remaining bulk sample should be stored.

Note: UG05 and UG07 was not found in the MTS portal so was not able to assign the test on the online portal.

Table 25. Tracking table of subgrade in the central lab testing contractor laboratory

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41A01L1 | P51/SS01 | P43/SS03 | P52/SS04 | P46/SS07 | P55/SS05 | Yes | S |
| S41A01W1 | P49/SS09 |  |  |  |  | Yes | S |
| S43A01L1 | P51/SS01 | P43/SS03 | P52/SS04 | P46/SS07 | P55/SS05 | Yes | S |
| S43A01W1 | P49/SS09 |  |  |  |  | Yes | S |
| S45A01L1 | P51/SS01 | P43/SS03 | P52/SS04 | P46/SS07 | P55/SS05 | Yes | S |
| S45A01W1 | P49/SS09 |  |  |  |  | Yes | S |

The moisture content sample may be disposed after testing.  However, any remaining bulk sample should be stored.

Table 26. Tracking Table of Experimental Asphalt Concrete Layer in the Turner Fairbanks Highway Research Center

| Specimen ID | Steps Involved in Laboratory Handling and Testing Sequence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required Laboratory Tests Per Layer |  |  |  |  | Extra Sample | Next Process1 |
|  | First | Second | Third | Fourth | Fifth |  |  |
| S41H02L8 | P76/AE11 |  |  |  |  | Yes | S |
| S44H02L8 | P76/AE11 |  |  |  |  | Yes | S |
| S45H02L8 | P76/AE11 |  |  |  |  | Yes | S |

1 Next Process
	D 	Remaining sample should be disposed
	S 	Remaining sample should be stored
	C	Remaining sample will be combined with another sample
	E 	Perform further testing on extracted sample
	P 	Perform further testing with PAV
	R 	Perform further testing with RFTO
	R+P  	Perform further testing with RTFO + PAV